Protocol for Computer Supported Telecommunications Applications (CSTA) Phase II

.

Protocol for Computer Supported Telecommunications Applications (CSTA) Phase II

.

Brief History

This Standard defines Phase II of the Protocol for Computer Supported Telecommunications Applications (CSTA) for OSI Layer 7 communication between a computing network and a telecommunications network. This Standard and its companion Standard ECMA-217 Services for Computer Supported Telecommunications Applications (CSTA) Phase II reflect agreements of ECMA member companies on Phase II of the standards for CSTA. Additional phases are anticipated. This Standard is based on the practical experience of ECMA member companies and represents a pragmatic and widely-based consensus.

This Standard takes direction from Technical Report ECMA TR/52 Computer Supported Telecommunications Applications.

Phase II of CSTA extends the previous Phase I standard in major theme directions as well as numerous details. Major areas of advancement include:

- the addition of explicit application context negotiation mechanisms;

- the addition of I/O services;

- the addition of Special Resource Functions and, particularly, Voice Unit services;

- new and/or enhanced services and event reports for commonly used call control and monitoring applications; new services include Single Step Transfer, Single Step Conference, Call Park and Send DTMF Signals.

- The Phase II CSTA standards are not fully backwards compatible with the Phase I standards. Although backwards compatibility is an important consideration and has been maintained whenever possible, the addition of new parameters in certain services and events, as well as the deletion of unused Phase I services and the addition of entirely new Phase II services and events, did not allow complete backwards compatibility.

- This Standard is dedicated to the memory of Terry Wuerfel.

- This ECMA Standard has been adopted by the ECMA General Assembly of December 1994.

- .

- Usage Note regarding removing Year 2000 (Y2K) risks 
in the CSTA Phase I and Phase II Protocol Standards

- 1 January 1999

- In both ITU-T Recs. X.208 (ASN.1 1988) and X.680 (ASN.1 1994), the definition of UTCTime, which is used in the CSTA Phase I and Phase II Protocol Standards (ECMA-180 and ECMA-218 respectively), is not Y2K-safe. The Year field (the YY field) is represented by a two-digit string, with no accompanying text mandating a Y2K-safe interpretation.

- For the CSTA Phase I and Phase II Protocol Standards, the following interpretation of the Year field is mandated :

- If the YY component is in the range [00-49], the century is determined to be "20" (i.e. the year is 2000 to 2049);

- If the YY component is in the range [50-99], the century is determined to be "19" (i.e. the year is 1950 to 1999).

- The risk has been removed in the CSTA Phase III Protocol Standard (ECMA-285) by replacing the references to "UTCTime" with references to "GeneralizedTime", which is Y2K-safe.

- .

- List of corrected errata for ECMA-218

- 11 December 1996

- Summary

- Following is a summary of the errors detected and corrected in Standard ECMA-218, Protocol for Computer Supported Telecommunications Applications (CSTA) Phase II.  These results were obtained by using the OSS and SNACC ASN.1 compilers.

## All definitions

- Compilers complained about “icd-ecma(0012)” statements, therefore “00” has been removed from the definitions

- Corrected:

- CSTA-application-context-information

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) application-context-information( 200) }

- Original:

- CSTA-application-context-information

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta2( 218) application-context-information( 200) }

## Multiple definitions

- Many of the definitions referred to “ECMA-180” instead of “ECMA-218”

- Corrected:

- CSTA-route-end-request

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) route-end-request( 85) }

- CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- CSTA-change-monitor-filter

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) change-monitor-filter( 102) }

- CSTAObject FROM CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) switching-function-objects( 122) }

- CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218)switching-function-objects( 122) }

- Original:

- CSTA-route-end-request

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta( 180) route-end-request( 85) }

- CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta( 180) version1(1) extension-types( 129) };

- CSTA-change-monitor-filter

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta( 180) version1 ( 1) change-monitor-filter( 102) }

- CSTAObject FROM CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta( 180) switching-function-objects( 122) }

- CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta( 180) switching-function-objects( 122) }

## Multiple Sections

- “BITSTRING” should be “BIT STRING”

- Corrected:

- CSTAVersion ::= BIT STRING

- SwitchingFunctionServices ::= BIT STRING

- EventReportServices ::= BIT STRING

- ComputingFunctionServices ::= BIT STRING

- BidirectionalServices ::= BIT STRING

- StatusReportingServices ::= BIT STRING

- InputOutputServices ::= BIT STRING

- VoiceUnitServices ::= BIT STRING

- DeviceClass ::= BIT STRING

- Original:

- CSTAVersion ::= BITSTRING

- SwitchingFunctionServices ::= BITSTRING

- EventReportServices ::= BITSTRING

- ComputingFunctionServices ::= BITSTRING

- BidirectionalServices ::= BITSTRING

- StatusReportingServices ::= BITSTRING

- InputOutputServices ::= BITSTRING

- VoiceUnitServices ::= BITSTRING

- DeviceClass ::= BITSTRING

## Section 8.2.1, Encoding of application context information

- Comma missing after reRouteRequest

- Corrected:

- ComputingFunctionServices ::= BIT STRING

- {routeRequest			(0),

- reRouteRequest			(1),

- routeSelectRequest		(2),

- routeUsedRequest		(3),

- routeEndRequest			(4) }

- Original:

- ComputingFunctionServices ::= BIT STRING

- {routeRequest			(0),

- reRouteRequest			(1)

- routeSelectRequest		(2),

- routeUsedRequest		(3),

- routeEndRequest			(4) }

## Section 9.3, Associate data

- AssociateDataArgument/Correlator Data misspelled

- Extra brace at end of SEQUENCE statement for AssociateDataArgument

- Corrected:

- correlatorData	[2] IMPLICIT CorrelatorData	OPTIONAL,

- extensions	CSTACommonArguments		OPTIONAL}

- Original:

- correlatorData	[2] IMPLICIT CorellatorData	OPTIONAL,

- extensions             CSTACommonArguments	OPTIONAL} }

## Section 9.8, Consultation call

- Missing comma after AuthCode

- Corrected:

- AccountInfo, AuthCode, CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

- Original:

- AccountInfo, AuthCode CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

## Section 9.17, Send DTMF tones

- Removed specific characters from IA5String type (or need to define a new type of just these
characters)

- Corrected:

- {connectionToSendTones  ConnectionID,

- charactersToSend	IA5String,

- toneDuration	[0] IMPLICIT INTEGER	OPTIONAL,

- Original:

- {connectionToSendTones  ConnectionID,

- charactersToSend	IA5String		{0,1,2,3,4,5,6,7,8,9,*,#,A,B,C,D},

- toneDuration	[0] IMPLICIT INTEGER	OPTIONAL,

## Section 9.20, Single step transfer

- SingleStepTransResult should be SEQUENCE

- Corrected:

- SingleStepTransResult ::=

- SEQUENCE

- {transferedCall         ConnectionID                    OPTIONAL,

- Original:

- SingleStepTransResult ::=

- {transferedCall         ConnectionID                    OPTIONAL,

## Section 10.1.10, Originated

- OriginatingDevice (SubjectDeviceID) missing

- Corrected:

- CalledDeviceID, CallingDeviceID, SubjectDeviceID FROM CSTA-device-identifiers

- Original:

- CalledDeviceID, CallingDeviceID FROM CSTA-device-identifiers

## Section 10.3.1, Agent busy

- Missing agentGroup from AgentBusyEvent sequence

- Corrected:

- Add to IMPORTS:

- AgentID, AgentGroup FROM CSTA-device-feature-types

- AgentBusyEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID		[10] IMPLICIT 	AgentID		OPTIONAL,

- agentGroup	AgentGroup 			OPTIONAL,

- cause		EventCause			OPTIONAL}

- Original:

- AgentBusyEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID		AgentID			OPTIONAL,

- cause		EventCause		OPTIONAL}

## Sections 10.3.2 and 10.3.3, Agent logged on and Agent logged off

- Conflict between tag for agentID and DeviceID, dialingNumber

- Corrected:

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID		[10] IMPLICIT	AgentID		OPTIONAL,

- agentGroup	AgentGroup			OPTIONAL,

- Original:

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID		[0] IMPLICIT	AgentID		OPTIONAL,

- agentGroup	AgentGroup			OPTIONAL,

## Section 10.3.6, Agent working after call

- Missing agentGroup from WorkingAfterCallEvent sequence

- Corrected:

- Add to IMPORTS:

- AgentID, AgentGroup FROM CSTA-device-feature-types

- WorkingAfterCallEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID		[10] IMPLICIT 	AgentID		OPTIONAL,

- agentGroup	AgentGroup 			OPTIONAL,

- cause		EventCause			OPTIONAL}

- Original:

- WorkingAfterCallEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID		AgentID			OPTIONAL,

- cause		EventCause		OPTIONAL}

## Section 11.1, Route request

- DeviceProfile missing from IMPORTS

- Corrected:

- CorrelatorData, DeviceClass, DeviceProfile, SelectValue, PriorityValue, RoutingCrossRefID

- Original:

- CorrelatorData, DeviceClass, SelectValue, PriorityValue, RoutingCrossRefID

## Section 12.1, Escape

- “CSTA-escape-service” should be “escape-service” in header

- Corrected:

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) escape-service( 91) }

- Original:

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta2( 218) CSTA-escape-service( 91) }

## Section 13.1, Monitor start

- CSTACommonArguments misspelled

- Corrected:

- extensions             CSTACommonArguments     OPTIONAL}

- Original:

- extensions             CSTACommomArguments     OPTIONAL}

## Section 14.1, Start data path

- “IA5STRING” should be “IA5String”, in argument and result

- Corrected:

- terminationCharacter           IA5String                               OPTIONAL,

- Original:

- terminationCharacter           IA5STRING                          OPTIONAL,

## Section 14.2, Stop data path

- “RRORS” should be “ERRORS” in stopDataPath

- Corrected:

- ERRORS  {universalFailure}

- Original:

- RRORS   {universalFailure}

## Section 15.1, Concatenate Message

- Tag conflicted between CHOICE elements, because MessageIDList should be marked as IMPLICIT

- Corrected:

- ConcatenateMessageArgument ::=

- CHOICE

- {messagesToConcatenate	[0]IMPLICIT	MessageIDList,

- SEQUENCE

- {messagesToConcatenate	MessageIDList,

- extensions		CSTACommonArguments} }

- Original:

- ConcatenateMessageArgument ::=

- CHOICE

- {messagesToConcatenate		MessageIDList,

- SEQUENCE

- {messagesToConcatenate	MessageIDList,

- extensions		CSTACommonArguments} }

## Section 15.5, Record Message

- Missing IMPLICIT tags for SEQUENCE elements

- Extraneous brace at end of RecordMessageArgument

- Corrected:

- RecordMessageArgument ::=

- SEQUENCE

- {callToBeRecorded	ConnectionID,

- samplingRate		REAL					OPTIONAL,

- encodingAlg		[0]IMPLICIT ENUMERATED

- {aDPCM6K		(0),

- aDPCM8K		(1),

- muLawPCM6K		(2),

- aLawPCM6K		(3)}	OPTIONAL,

- maxDuration		[1]IMPLICIT INTEGER			OPTIONAL,

- termination		[2]IMPLICIT TerminatingConditions	OPTIONAL,

- extensions		CSTACommonArguments			OPTIONAL}

- Original:

- RecordMessageArgument ::=

- SEQUENCE

- {callToBeRecorded	ConnectionID,

- samplingRate		REAL					OPTIONAL,

- encodingAlg		ENUMERATED

- {aDPCM6K		(0),

- aDPCM8K		(1),

- muLawPCM6K		(2),

- aLawPCM6K		(3)}	OPTIONAL,

- maxDuration		INTEGER				OPTIONAL,

- termination		TerminatingConditions			OPTIONAL,

- extensions		CSTACommonArguments			OPTIONAL}}

## Section 15.9, Set voice attribute

- OPERATION, ERROR imports missing

- Corrected:

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- Original:

- IMPORTS

- -- Data Types --

- universalFailure import missing

- Corrected:

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CSTACommonArguments import mising

- Corrected:

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- Original:

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta2( 218) device-feature-types( 127) };

- SetVoiceAttributeArgument misspelled, comma after brace missing

- Corrected:

- SetVoiceAttributeArgument ::=

- SEQUENCE

- {connection		ConnectionID,

- attributeToSet		CHOICE

- {speed		[0] IMPLICIT Speed,

- speakerVolume 	[1] IMPLICIT INTEGER (0 .. 100),

- recordingLevel	[2] IMPLICIT INTEGER (0 .. 100)	},

- message		MessageID			OPTIONAL,

- extensions		CSTACommonArguments		OPTIONAL}

- Original:

- SetVoiveAttributeArgument ::=

- SEQUENCE

- {connection		ConnectionID,

- attributeToSet		CHOICE

- {speed	 	[0] IMPLICIT Speed,

- speakerVolume 	[1] IMPLICIT INTEGER (0 .. 100),

- recordingLevel	[2] IMPLICIT INTEGER (0 .. 100)	}

- message		MessageID			OPTIONAL,

- extensions		CSTACommonArguments		OPTIONAL}

- setVoiceAttributeResult misspelled

- Corrected:

- SetVoiceAttributeResult ::=

- CHOICE

- Original:

- SetVoiveAttributeResult ::=

- CHOICE

## Section 15.10, Stop

- Opening brace missing in StopArgument

- Corrected:

- StopArgument ::=

- SEQUENCE

- {connection     ConnectionID,

- Original:

- StopArgument ::=

- {SEQUENCE

- connection     ConnectionID,

## Section 15.11, Suspend

- Opening brace missing in SuspendArgument

- Corrected:

- SuspendArgument ::=

- SEQUENCE

- {connection     ConnectionID,

- Original:

- SuspendArgument ::=

- {SEQUENCE

- connection     ConnectionID,

## Section 15.12, Synthesize message

- Opening brace missing in SynthesizeMessageArgument

- Corrected:

- SynthesizeMessageArgument ::=

- SEQUENCE

- {textToBeSynthesized            OCTET STRING,

- Original:

- SynthesizeMessageArgument ::=

- {SEQUENCE

- textToBeSynthesized            OCTET STRING,

## Section 18, CSTA data types

- Comment line not properly aligned

- Corrected:

- --The major parameters have been assigned distinct application tags to facilitate parsing. The
data is defined in logical

- --groups in ascending order of application tag. Application tags used are:

- Original:

- --The major parameters have been assigned distinct application tags to facilitate parsing. The
data is defined in logical --gro

- ps in ascending order of application tag. Application tags used are:

## Section 18.4, Connection states

- ConnectionIDList should be removed from EXPORTS list

- Corrected:

- EXPORTS

- ConnectionList, LocalConnectionState;

- Original:

- EXPORTS

- ConnectionList, ConnectionIDList, LocalConnectionState;

## Section 18.5, Status reporting

- Extraneous semicolon at end of CSTAObject import statement

- Corrected:

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) switching-function-objects( 122) }

- Original:

- { iso( 1) identified-organization( 3) icd-ecma( 0012)

- standard( 0) csta2( 218) switching-function-objects( 122) };

## Section 18.6, Device and feature types and other parameters

- IOCrossRefIDList misspelled in EXPORTS statement

- Corrected:

- IOCrossRefIDList, IOData, Speed, DataPathDirec

- Original:

- IOCrassRefIDList, IOData, Speed, DataPathDirec

- SetupValues misspelled in EXPORTS statement

- Corrected:

- SetupValues

- Original:

- SetUpValues,

- CalledDeviceID misspelled in IMPORTS statement

- Corrected:

- DeviceID, NumberDigits, CalledDeviceID FROM CSTA-device-identifiers

- Original:

- DeviceID, NumberDigits, CallDeviceID FROM CSTA-device-identifiers

- DeviceInfo definition had extra space in “::=” symbol

- Corrected:

- DeviceInfo      ::= SEQUENCE

- {       deviceID        DeviceID        OPTIONAL,

- Original:

- DeviceInfo      :: = SEQUENCE

- {       deviceID        DeviceID        OPTIONAL,

- DivertInfo, groupRequestingDevice choice should not be marked IMPLICIT

- Corrected:

- DivertInfo ::= CHOICE	-- used by Divert Call service

- {	deflect			[0] IMPLICIT SEQUENCE

- {callToBeDiverted	ConnectionID,

- newDestination		CalledDeviceID	},

- pickup			[1] IMPLICIT SEQUENCE

- {callToBePickedUp	ConnectionID,

- requestingDevice	DeviceID	},

- groupRequestingDevice	[2] DeviceID	}

- Original:

- DivertInfo ::= CHOICE	-- used by Divert Call service

- {	deflect			[0] IMPLICIT SEQUENCE

- {callToBeDiverted	ConnectionID,

- newDestination		CalledDeviceID	},

- pickup			[1] IMPLICIT SEQUENCE

- {callToBePickedUp	ConnectionID,

- requestingDevice	DeviceID	},

- groupRequestingDevice	[2] IMPLICIT 		DeviceID	}

- SetupValues needs space in OCTETSTRING

- Corrected:

- SetupValues ::= OCTET STRING

- Original:

- SetupValues ::= OCTETSTRING

- LoggedOffInfo missing “password” in sequence

- Corrected:

- LoggedOffInfo	::= SEQUENCE

- {	agentID	 	[10] IMPLICIT	AgentID		OPTIONAL,

- password	[11] IMPLICIT	AgentPassword	OPTIONAL,

- group		AgentGroup			OPTIONAL	}

- Original:

- LoggedOffInfo	::= SEQUENCE

- {	agentID	 	[10] IMPLICIT	AgentID		OPTIONAL,

- group		AgentGroup			OPTIONAL	}

- Comma missing in IOCrossRefID definition, and OCTETSTRING needs space

- Corrected:

- IOCrossRefID ::= CHOICE

- {      switchProvided		[0] OCTET STRING,

- computerProvided	[1] OCTET STRING }

- Original:

- IOCrossRefID ::= CHOICE

- {      switchProvided		[0] OCTETSTRING

- computerProvided	[1] OCTETSTRING }

- IOData needs space in OCTETSTRING

- Corrected:

- IOData ::= OCTET STRING

- Original:

- IOData ::= OCTETSTRING

- MessageID definition had extra space, and needs space in OCTETSTRING

- Corrected:

- MessageID ::= OCTET STRING

- Original:

- Message ID::= OCTETSTRING

- “SetUpValues” (with capital “u”) should be removed, as “Setupvalues” is already defined

- Original:

- SetUpValues ::= OCTET STRING  -- Contains Q.931 Setup message

- Period description missing name for choices - selected “relativePosition” and “position” 
for names; also, choice designations (“[0]” and “[1]”) missing

- Corrected:

- Period ::= CHOICE

- {	relativePosition [0] ENUMERATED

- {	beginningOfMessage		(0),

- endOfMessage			(1)	},

- position	[1] INTEGER	}

- Original:

- Period ::= CHOICE

- {	ENUMERATED

- {       beginingOfMessage           (0),

- endOfMessage                    (1)     },

- INTEGER }

- AttributeInfo/fileName needs space in OCTETSTRING

- Corrected:

- fileName 	OCTET STRING

- Original:

- fileName	OCTETSTRING

- ControlData/languague needs space in OCTETSTRING

- Corrected:

- language		OCTET STRING

- Original:

- language		OCTETSTRING

- .

- List of corrected errata for ECMA-218

- 6 January 1999

- Summary

- Following is a summary of the errors detected and corrected in Standard ECMA-218, Protocol for Computer Supported Telecommunications Applications (CSTA) Phase II.

- Section 8.2.1, Encoding of application Context information, definition of CSTAFunctionality

- The first field is misspelt (was sitchingFunctionServices)

- The definition of CSTAFunctionality can't be accurately decoded because:

- a)	Each of the fields (eventReportServices, computingFunctionServices etc) can be ommitted during encoding as indicated by the DEFAULT {}.

- b)	Each of the underlying definitions of these fields is a BIT STRING and hence has the same tag (0x03).

- Because it is not possible to uniquely identify each field during decoding, it is not possible to determine if a given field has been omitted. Addition of tags removes any ambiguity.

## Corrected:

- CSTAFunctionality ::= SEQUENCE

- {switchingFunctionServices	[0] IMPLICIT SwitchingFunctionServices	DEFAULT {},

- eventReportServices		[1] IMPLICIT EventReportServices	DEFAULT {},

- computingFunctionServices	[2] IMPLICIT ComputingFunctionServices	DEFAULT {},

- bidirectionalServices		[3] IMPLICIT BidirectionalServices	DEFAULT {},

- statusReportingServices		[4] IMPLICIT StatusReportingServices	DEFAULT {},

- inputOutputServices		[5] IMPLICIT InputOutputServices		DEFAULT {},

- voiceUnitServices		[6] IMPLICIT VoiceUnitServices		DEFAULT {} }

## Original:

- CSTAFunctionality ::= SEQUENCE

- {sitchingFunctionServices		SwitchingFunctionServices	DEFAULT {},

- eventReportServices		EventReportServices		DEFAULT {},

- computingFunctionServices	ComputingFunctionServices	DEFAULT {},

- bidirectionalServices		BidirectionalServices		DEFAULT {},

- statusReportingServices		StatusReportingServices		DEFAULT {},

- inputOutputServices		InputOutputServices		DEFAULT {},

- voiceUnitServices		VoiceUnitServices		DEFAULT {} }

- Section 10.1.11, Queued event, field misspelt

- The field name for the number of calls in queue is misspelt.

## Corrected:

- numberQueued		[0] IMPLICIT NoOfCallsInQueue	OPTIONAL,

## Original:

- numberedQueued		[0] IMPLICIT NoOfCallsInQueue	OPTIONAL,

- Section 13.3, Monitor Stop service, IMPORTS statement

- The import of CSTAPrivateData is not needed.

- Corrected:

- CSTACommonArguments FROM CSTA-extension-types

## Original:

- CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

- Section 16, Switching Function Errors, definition of UniversalFailure

- The decoding of a UniversalFailure choice is ambigious:
a)	CSTAPrivateData is defined as NULL

- CSTAPrivateData ::= NULL	-- The actual encoding of the private

- -- event is added here, replacing NULL with

- -- another valid ASN.1 type.

- b)	The nonStandardErrors CHOICE of UniversalFailure is defined as CSTAPrivateData

- UniversalFailure ::= CHOICE

- {operationalErrors			[1] IMPLICIT Operations,

- stateErrors				[2] IMPLICIT StateIncompatibility,

- systemResourceErrors			[3] IMPLICIT SystemResourceAvailability,

- subscribedResourceAvail...		[4] IMPLICIT SubscribedResourceAvail…,

- performanceErrors			[5] IMPLICIT PerformanceManagement,

- securityErrors				[6] IMPLICIT SecurityError,

- unspecifiedErrors			[7] IMPLICIT NULL,

- nonStandardErrors			CSTAPrivateData}

- The tag chosen for CSTAPrivateData by a particular implementation may clash with one of the existing UniversalFailure CHOICE tags making the decode process ambigious.

- When decoding a UniversalFailure CHOICE an application has no way of knowing the range of tag values that may be encountered, without prior arrangement with the switch.

- Therefore the nonStandardErrors field must have an implicit tag (as all other uses of CSTAPrivateData in ECMA-218 do).

## Corrected:

- nonStandardErrors		[8] IMPLICIT  CSTAPrivateData}

## Original:

- nonStandardErrors		CSTAPrivateData}

- Note: This problem does not exist in Phase I as CSTAPrivateData had a defined tag ([Application 29]) rather than NULL.

- Section 16, Switching function errors, error value name

- The name of StateIncompatibility value 14 is misspelt.

## Corrected:

- messageSuspended		(14)	}

## Original:

- massageSuspended		(14)	}

- Section 18.5, Status Reporting, CallFilter definition

- The name of CallFilter value 13 is misspelt.

## Corrected:

- transferred			(13)	}

## Original:

- transferrd			(13)	}

- Section 18.6, Device and feature types and other parameters, DomainValue description

- The word "destination" in the comment is misspelt.

## Corrected:

DomainValue ::= BOOLEAN	-- TRUE means CSTA subdomain destination

## Original:

DomainValue ::= BOOLEAN	-- TRUE means CSTA subdomain destinationp

.

Table of contents

Page

SECTION I - GENERAL

1 Scope

2 Conformance

2.1 Static requirements

2.2 Dynamic requirements

2.3 PICS requirement

3 References

4 Definitions

SECTION II - PROTOCOL STRUCTURE FOR CSTA

5 CSTA service definition model

5.1 CSTA application layer structure

5.2 Remote operations

5.3 The CSTA service response

5.4 Cross referencing of event reports

5.5 Handling of private data

6 Interconnection service boundary

7 Security

SECTION III - CSTA PROTOCOL

8 Association management

8.1 Implicit association

8.2 Dynamic association management using ACSE

8.2.1 Encoding of application context information

9 Switching function services

9.1 Alternate call

9.2 Answer call

9.3 Associate data

9.4 Call completion

9.5 Clear call

9.6 Clear connection

9.7 Conference call

9.8 Consultation call

9.9 Divert call

9.10 Hold call

9.11 Make call

9.12 Make predictive call

9.13 Park call

9.14 Query device

9.15 Reconnect call

9.16 Retrieve call

9.17 Send DTMF tones

9.18 Set feature

9.19 Single step conference

9.20 Single step transfer

9.21 Transfer call

10 Event report service

10.1 Call events

10.1.1 Call cleared

10.1.2 Conferenced

10.1.3 Connection cleared

10.1.4 Delivered

10.1.5 Diverted

10.1.6 Established

10.1.7 Failed

10.1.8 Held

10.1.9 Network reached

10.1.10 Originated

10.1.11 Queued

10.1.12 Retrieved

10.1.13 Service initiated

10.1.14 Transferred

10.2 Feature events

10.2.1 Auto answer

10.2.2 Call information

10.2.3 Do not disturb

10.2.4 Forwarding

10.2.5 Message waiting

10.2.6 Microphone mute

10.2.7 Speaker mute

10.2.8 Speaker volume

10.3 Agent state events

10.3.1 Agent busy

10.3.2 Agent logged on

10.3.3 Agent logged off

10.3.4 Agent not ready

10.3.5 Agent ready

10.3.6 Agent working after call

10.4 Maintenance events

10.4.1 Back in service

10.4.2 Out of service

10.5 Private events

10.6. Voice unit events

10.6.1 Play

10.6.2 Record

10.6.3 Review

10.6.4 Stop

10.6.5 Suspend play

10.6.6 Suspend record

10.6.7 Voice attributes change

11 Computing function services

11.1 Route request

11.2 Reroute request

11.3 Route select request

11.4 Route used request

11.5 Route end request

12 Bidirectional services

12.1 Escape

12.2 System status

13 Status reporting services

13.1 Monitor start

13.2 Change monitor filter

13.3 Monitor stop

13.4 Snapshot device

13.5 Snapshot call

14 Input/output services

14.1 Start data path

14.2 Stop data path

14.3 Send data

14.4 Send multicast data

14.5 Send broadcast data

14.6 Suspend data path

14.7 Data path suspended

14.8 Resume data path

14.9 Data path resumed

14.10 Fast data

15 Voice unit services

15.1 Concatenate message

15.2 Delete message

15.3 Play message

15.4 Query voice attribute

15.5 Record message

15.6 Reposition

15.7 Resume

15.8 Review

15.9 Set voice attribute

15.10 Stop

15.11 Suspend

15.12 Synthesize message

16 Switching function errors

17 Switching event cause values

18 CSTA data types

18.1 Switching function objects

18.2 Device identifiers

18.3 Call and connection identifiers

18.4 Connection states

18.5 Status reporting

18.6 Device and feature types and other parameters

18.7 Security service

18.8 Common extensions

19 CSTA parameter size constraints

Annex A - Protocol Implementation Conformance Statement (PICS) Proforma	139

# SECTION I - GENERAL

# 1	Scope

This Standard specifies application protocol data units (APDUs) for the services described in Standard ECMA-217, Services for Computer Supported Telecommunications Applications (CSTA) Phase II. The field of application of this Standard is the interconnection of switches and computers in a private telecommunications environment.

Section II (clause 5 to clause 7 inclusive) describes the concepts underlying the Remote Operations model, notation and service.

Section III (clause 8 to clause 19 inclusive) contains CSTA-specific protocol details and forms the main part of this Standard.

The protocol in this Standard operates in the context of an application association.

# 2	Conformance

A manufacturer may select any part (one or more operations) of the CSTA protocol, as specified in this Standard, for implemention on a system.

A system is in conformance with this Standard if one or more of the CSTA operations are implemented according to the requirements of this Standard.

A Protocol Implementation Conformance Statement (PICS) shall be used to specify the operations which are provided by a particular implementation. A PICS shall also specify the parameter options which are used.

## 2.1	Static requirements

To conform to this Standard, a system shall support the transfer syntax (derived from the encoding rules specified in CCITT Rec. X.209) named {joint-iso-ccitt( 2) asn1(1) basic-encoding(1)};  for the purpose of generating and interpreting CSTA protocol information as defined by the abstract syntax "CSTA-ASN.1-Object-Descriptor" for the operations supported.

## 2.2	Dynamic requirements

To conform to this Standard, a system shall:

- follow the procedures as specified in this Standard, and Standard ECMA-217, relevant to each CSTA operation that the system claims to implement; and

- satisfy the definitions, as specified in Standard ECMA-217, relevant to each CSTA service that the system claims to implement.

## 2.3	PICS requirement

To conform to this Standard, the following shall be stated by the implementer when defining a PICS corresponding to an application or implementation:

- which CSTA operations, as defined in Standard ECMA-217, are supported by the system for the particular implementation; and

- which optional parameters are supported by the PDUs belonging to the supported operations; and

- the types and ranges of values for all the parameters supported.

A PICS proforma is given in annex A of this Standard.

# 3	References

ECMA-138	Security in Open Systems - Data Elements and Service Definitions (1989)

ECMA-217	Services for Computer Supported Telecommunications Applications (CSTA) Phase II (1994)

ECMA TR/46	Security in Open Systems - A Security Framework (1988)

ECMA TR/52	Computer Supported Telecommunications Applications (1990)

ECMA TR/68	Scenarios for Computer Supported Telecommunications Applications (CSTA) Phase II (1994)

CENELEC ENV 41007	Definitions of Terms in Private Telecommunications Networks

CCITT Rec. E.164	Numbering Plan for ISDN

ISO 8649:1988	Information processing systems - Open Systems Interconnection - Service definition for the Association Control Service Element (this corresponds to CCITT Rec. X.217)

ISO 8650:1988	Information processing systems - Open Systems Interconnection - Protocol specification for the Association Control Service Element (this corresponds to CCITT Rec. X.227)

ISO/IEC 8824:1990	Information technology - Open Systems Interconnection - Specification of Abstract Syntax Notation One (ASN.1) (this corresponds to CCITT Rec. X.208)

ISO/IEC 8825:1990	Information technology - Open Systems Interconnection - Specification of basic encoding rules for Abstract Syntax Notation One (ASN.1) (this corresponds to CCITT Rec. X.209)

ISO/IEC 9072-1:1989	Information processing systems - Text communication - Remote operations - Part 1: Model, notation and service definition (this corresponds to CCITT Rec. X.219)

ISO/IEC 9072-2:1989	Information processing systems - Text communication - Remote operations - Part 2: Protocol specification (this corresponds to CCITT Rec. X.229)

ISO/IEC 9545:1994	Information technology - Open Systems Interconnection - Application Layer structure

All the CCITT Recommendations that are referenced are the 1988 (Blue Book) version.

# 4	Definitions

CSTA-specific terminology is defined in Standard ECMA-217. For the purposes of this Standard, the following additional definitions, defined in other standards, shall apply:

- Remote Operations	(as per CCITT Rec. X.219)

- Application Association	(as per CCITT Rec. X.217)

- Application Context	(as per CCITT Rec. X.217)

- Private Telecommunications Network	(as per CENELEC ENV 41007)

# SECTION II - PROTOCOL STRUCTURE FOR CSTA

# 5	CSTA service definition model

## 5.1	CSTA application layer structure

- The CSTA Application Layer structure conforms to the model described in ISO/IEC 9545.

## 5.2	Remote operations

- The services of CSTA are modelled as Remote Operations as described in CCITT Rec. X.219. Typically, one entity requests that a particular operation be performed;  the other entity attempts to perform the operation and responds to the requestor.  Consequently the operation of the protocol is an elementary request/reply interaction, supported within the OSI Application Layer, and carried out within the context of an application association.

- For some of the CSTA services, the entity to which the request is sent generates a reply which can indicate success or failure. For these services, CSTA shall adopt the operations Class 2, defined in CCITT Rec. X.219 as:

- Asynchronous, reporting success or failure (result or error).

- For some of the CSTA services, the entity to which the request is sent generates a reply which can only indicate failure. For these services, CSTA shall adopt the operations Class 3, defined in CCITT Rec. X.219 as:

- Asynchronous, reporting failure (error) only, if any.

- For some of the CSTA services, particularly the ongoing reporting of events, no reply is generated. For these services, CSTA shall adopt the operations Class 5, defined in CCITT Rec. X.219 as:

- Asynchronous, outcome not reported.

The protocol description for the particular service defines the relevant class of the operation used for that service.

CSTA shall correlate the single response, denoting success or failure, with the originating request by using the mechanisms within the ROSE protocol.

## 5.3	The CSTA service response

CSTA employs a generic response mechanism which is, in principle, decoupled from the specifics of the switching activity. The following points describe the operation of the CSTA service response:

- Specific services may have an unconfirmed mode where responses to correct requests are not returned.

- The server shall check the correctness of the request (e.g. syntactical checks) before issuing the response. Incorrect requests shall result in an error response, even in the unconfirmed mode.

- If a response is sent before the action requested by the service is completed (i.e. the response is a service request acknowledgement), event reporting may be used to keep track of the subsequent server activity.

- The precise moment at which the response is generated in relation to the switching activity is implementation- and service- dependent.

- Some implementations may generate the response after checking the correctness of the request and at the point they initiate the request.

- Other implementations may delay the response until the service has completed (or is guaranteed to complete). In this case, a failure of the switching request will  be reflected in the response.

NOTE 1

Irrespective of implementation details,  when an operation succeeds, the same event reports are generated if the necessary monitoring has been established.  As an example, a Held-Event (if selected) is always reported in addition to the response to a successful request (even in those implementations that delay the response until the Hold operation was complete).  In a given context, and with appropriate monitoring in place, an operation generates the same set of event reports, whether it was invoked manually or with CSTA service requests from the computer.

## 5.4	Cross referencing of event reports

A computer application process may need to cross reference a CSTAEventReport to one of the following:

- a CSTA Object ID (Call ID or Device ID),

- an earlier Monitor request; or

- one of many Monitor requests (pertaining to the same CSTA Object).

For the above scenarios the necessary cross referencing function may be fulfilled by use of the parameter "MonitorCrossRefID". The content of MonitorCrossRefID depends upon the context and it may be one of the following: Call ID, StaticDevice ID or another independently switch managed static identifier. The independent identifier may have a unique correlation to either: one device, one call, or one monitor request.

The switching system limit on the number of monitor requests on one CSTA Object (Call or Device) is an implementation option. This Standard does not stipulate how many monitor requests on one object are to be supported by the switch. If using Static Device or Call identifiers the limit can only be one.

## 5.5	Handling of private data

If an entity receives the parameter CSTAPrivateData, and it can not recognize the information contained, the parameter shall be discarded, and the rest of the message shall be processed.

# 6	Interconnection service boundary

The protocol in this Standard is an OSI Application Layer protocol and uses the Remote Operations protocol defined in CCITT Rec. X.229. The Remote Operations protocol assumes certain services are provided by the underlying layers, and these services are also assumed by the protocol for CSTA.

# 7	Security

This protocol also provides a mechanism for secure transmission of CSTA PDUs as defined in this Standard. The parameters that constitute this secure PDU are imported from Standard ECMA-138.

# SECTION III - CSTA PROTOCOL

# 8	Association management

The protocol in this Standard operates in the context of an application association. This application association can be either:

- an implicit association achieved via off-line agreement; or

- a dynamically negotiated association realsied through the use of ACSE.

## 8.1	Implicit association

- An a-priori agreement exists between switching and computing functions: the application context is implicit, dynamic negotiation is not possible.

## 8.2	Dynamic association management using ACSE

- A common application context Name is used, and CSTA protocol version information is carried within the User Information field of the A-ASSOCIATE request and response PDUs.

- An application context is established (using ACSE) as follows:

- the system generating the A-ASSOCIATE request includes a list of all protocol versions that it is prepared to offer in the User Information field;

- on receipt of the A-ASSOCIATE request, the receiving system selects the protocol version to be used by identifying the highest version that is common to both systems;

- the protocol version selected is conveyed to the requestor in the User Information field of the A-ASSOCIATE response.

- In addition to negotiating the protocol version, it is necessary for the requesting and responding systems to specify the CSTA services that they support. As with the protocol version information, this is also achieved by carrying additional information in the User Information field of the A-ASSOCIATE request and response PDUs. The application association requestor shall:

- list the services required from the serving application;

- list the services it can supply.

- The responder shall include similar information for the responding application. At this point the association requestor will either accept or reject the association.

### 8.2.1	Encoding of application context information

- CSTA-application-context-information

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) application-context-information( 200) }

- DEFINITIONS ::=

- BEGIN

- ACSEUserInformationForCSTA ::= SEQUENCE

- {cSTAVersion					CSTAVersion,

- cSTAFunctionsRequiredByApplication		CSTAFunctionality,

- cSTAFunctionsThatCanBeSupplied		CSTAFunctionality	}

- CSTAVersion ::= BIT STRING

- {versionOne		(0),

- versionTwo		(1)

- -- others to be added in future

- }

- --	When requesting an application association, the requestor will set the corresponding bits for each

- --	version that it supports. When responding, the responder shall specify only one bit corresponding

- --	to the version selected (the highest common to both systems).

- CSTAFunctionality ::= SEQUENCE

- {switchingFunctionServices	[0] IMPLICIT SwitchingFunctionServices		DEFAULT {},

- eventReportServices		[1] IMPLICIT EventReportServices		DEFAULT {},

- computingFunctionServices	[2] IMPLICIT ComputingFunctionServices	DEFAULT {},

- bidirectionalServices		[3] IMPLICIT BidirectionalServices		DEFAULT {},

- statusReportingServices	[4] IMPLICIT StatusReportingServices		DEFAULT {},

- inputOutputServices		[5] IMPLICIT InputOutputServices		DEFAULT {},

- voiceUnitServices		[6] IMPLICIT VoiceUnitServices			DEFAULT {} }

- SwitchingFunctionServices ::= BIT STRING

- {alternateCall			(0),

- answerCall			(1),

- associateData			(2),

- callCompletion			(3),

- clearCall			(4),

- clearConnection		(5),

- conferenceCall			(6),

- consultationCall		(7),

- divertCall			(8),

- holdCall			(9),

- makeCall			(10),

- makePredictiveCall		(11),

- parkCall			(12),

- queryDevice			(13),

- reconnectCall			(14),

- retrieveCall			(15),

- sendDTMFTones		(16),

- setFeature			(17),

- singleStepConference		(18),

- singleStepTransfer		(19),

- transferCall			(20)	}

- EventReportServices ::= BIT STRING

- {

- -- Call events

- callCleared			(0),

- conferenced			(1),

- connectionCleared		(2),

- delivered			(3),

- diverted			(4),

- established			(5),

- failed				(6),

- held				(7),

- networkReached		(8),

- originated			(9),

- queued			(10),

- retrieved			(11),

- serviceInitiated			(12),

- transferred			(13),

- -- Feature events

- autoAnswer			(14),

- callInformation			(15),

- doNotDisturb			(16),

- forwarding			(17),

- messageWaiting		(18),

- microphoneMute		(19),

- speakerMute			(20),

- speakerVolume		(21),

- -- Agent state events

- agentBusy			(22),

- loggedOn			(23),

- loggedOff			(24),

- notReady			(25),

- ready				(26),

- workingAfterCall		(27),

- -- Maintenance events

- backInService			(28),

- outOfServiceEvent		(29),

- -- Private events

- privateEvent			(30),

- -- Voice unit events

- playEvent			(31),

- recordEvent			(32),

- reviewEvent			(33),

- stopEvent			(34),

- suspendPlayEvent		(35),

- suspendRecordEvent		(36),

- voiceAttributesChangeEvent	(37)	}

- ComputingFunctionServices ::= BIT STRING

- {routeRequest			(0),

- reRouteRequest		(1),

- routeSelectRequest		(2),

- routeUsedRequest		(3),

- routeEndRequest		(4)	}

- BidirectionalServices ::= BIT STRING

- {escapeService			(0),

- systemStatus			(1)	}

- StatusReportingServices ::= BIT STRING

- {monitorStart			(0),

- changeMonitorFilter		(1),

- monitorStop			(2),

- snapshotDevice		(3),

- snapshotCall			(4)	}

- InputOutputServices ::= BIT STRING

- {startDataPathService		(0),

- stopDataPathService		(1),

- sendDataService		(2),

- sendMulticastDataService	(3),

- sendBroadcastDataService	(4),

- suspendDataPathService	(5),

- dataPathSuspendedService	(6),

- resumeDataPath		(7),

- dataPathResumedService	(8),

- fastData			(9)	}

- VoiceUnitServices ::= BIT STRING

- {concatenateMessage		(0),

- deleteMessage			(1),

- playMessage			(2),

- queryVoiceAttribute		(3),

- recordMessage			(4),

- reposition			(5),

- resume			(6),

- review				(7),

- setVoiceAttribute		(8),

- stop				(9),

- suspend			(10),

- synthesizeMessage		(11)	}

- END  -- of CSTA-application-context-information

# 9	Switching function services

- This clause defines the protocol for the switching function services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in Standard ECMA-217.

- The encoding of the protocol is defined in CCITT Rec. X.209.

- NOTE 2

- The range of services supported on a particular association is specified in the application context at association time.

## 9.1	Alternate call

- CSTA-alternate-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) alternate-call( 1) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionDetails FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- alternateCall	OPERATION

- ARGUMENT	AlternateCallArgument

- RESULT	AlternateCallResult

- ERRORS	{universalFailure}

- ::= 1

- AlternateCallArgument ::=

- CHOICE

- {callsInvolved		ConnectionDetails,

- SEQUENCE

- {callsInvolved	ConnectionDetails,

- extensions	CSTACommonArguments} }

- AlternateCallResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-alternate-call

## 9.2	Answer call

- CSTA-answer-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) answer-call( 2) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- answerCall		OPERATION

- ARGUMENT	AnswerCallArgument

- RESULT	AnswerCallResult

- ERRORS	{universalFailure}

- ::= 2

- AnswerCallArgument ::=

- CHOICE

- {callToBeAnswered		ConnectionID,

- SEQUENCE

- {callToBeAnswered	ConnectionID,

- extensions		CSTACommonArguments} }

- AnswerCallResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-answer-call

## 9.3	Associate data

- CSTA-associate-data

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) associate-data( 17) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- AccountInfo, AuthCode, CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- associateData	OPERATION

- ARGUMENT	AssociateDataArgument

- RESULT	AssociateDataResult

- ERRORS	{universalFailure}

- ::= 17

- AssociateDataArgument ::=

- SEQUENCE

- {existingCall		ConnectionID,

- accountCode		[0] IMPLICIT AccountInfo	OPTIONAL,

- authCode		[1] IMPLICIT AuthCode		OPTIONAL,

- correlatorData		[2] IMPLICIT CorrelatorData	OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- AssociateDataResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-associate-data

## 9.4	Call completion

- CSTA-call-completion

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-completion( 3) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- FeatureInfo FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- callCompletion	OPERATION

- ARGUMENT	CallCompletionArgument

- RESULT	CallCompletionResult

- ERRORS	{universalFailure}

- ::= 3

- CallCompletionArgument ::=

- CHOICE

- {featureInfo		FeatureInfo,

- SEQUENCE

- {featureInfo	FeatureInfo,

- extensions	CSTACommonArguments} }

- CallCompletionResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-call-completion

## 9.5	Clear call

- CSTA-clear-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) clear-call( 4) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- clearCall		OPERATION

- ARGUMENT	ClearCallArgument

- RESULT	ClearCallResult

- ERRORS	{universalFailure}

- ::= 4

- ClearCallArgument ::=

- CHOICE

- {callToBeCleared		ConnectionID,

- SEQUENCE

- {callToBeCleared	ConnectionID,

- extensions		CSTACommonArguments} }

- ClearCallResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-clear-call

## 9.6	Clear connection

- CSTA-clear-connection

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) clear-connection( 5) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- clearConnection	OPERATION

- ARGUMENT	ClearConnectionArgument

- RESULT	ClearConnectionResult

- ERRORS	{universalFailure}

- ::= 5

- ClearConnectionArgument ::=

- CHOICE

- {connectionToBeCleared		ConnectionID,

- SEQUENCE

- {connectionToBeCleared	ConnectionID,

- extensions			CSTACommonArguments} }

- ClearConnectionResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-clear-connection

## 9.7	Conference call

- CSTA-conference-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) conference-call( 6) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- ConnectionList FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- ConnectionDetails FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- conferenceCall	OPERATION

- ARGUMENT	ConferenceCallArgument

- RESULT	ConferenceCallResult

- ERRORS	{universalFailure}

- ::= 6

- ConferenceCallArgument ::=

- CHOICE

- {callsInvolved		ConnectionDetails,

- SEQUENCE

- {callsInvolved	ConnectionDetails,

- extensions	CSTACommonArguments} }

- ConferenceCallResult ::=

- SEQUENCE

- {conferenceCall		ConnectionID,

- connections		ConnectionList			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-conference-call

## 9.8	Consultation call

- CSTA-consultation-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) consultation-call( 7) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- AccountInfo, AuthCode, CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- consultationCall	OPERATION

- ARGUMENT	ConsultationCallArgument

- RESULT	ConsultationCallResult

- ERRORS	{universalFailure}

- ::= 7

- ConsultationCallArgument ::=

- SEQUENCE

- {existingCall		ConnectionID,

- consultedDevice	CalledDeviceID,

- consultedDeviceProfile	DeviceProfile			OPTIONAL,

- accountCode		[0] IMPLICIT AccountInfo	OPTIONAL,

- authCode		[1] IMPLICIT AuthCode		OPTIONAL,

- correlatorData		[2] IMPLICIT CorrelatorData	OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- ConsultationCallResult ::=

- CHOICE

- {initiatedCall		ConnectionID,

- SEQUENCE

- {initiatedCall	ConnectionID,

- extensions	CSTACommonArguments} }

- END  -- of CSTA-consultation-call

## 9.9	Divert call

- CSTA-divert-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) divert-call( 8) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CorrelatorData, DivertInfo, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- divertCall		OPERATION

- ARGUMENT	DivertCallArgument

- RESULT	DivertCallResult

- ERRORS	{universalFailure}

- ::= 8

- DivertCallArgument ::=

- SEQUENCE

- {divertInfo	DivertInfo,

- deviceProfile	DeviceProfile			OPTIONAL,

- correlatorData	CorrelatorData			OPTIONAL,

- extensions 	CSTACommonArguments	OPTIONAL}

- DivertCallResult ::=

- CHOICE

- {extensions 	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-divert-call

## 9.10	Hold call

- CSTA-hold-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) hold-call( 9) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- ReserveConnection FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- holdCall	OPERATION

- ARGUMENT	HoldCallArgument

- RESULT	HoldCallResult

- ERRORS	{universalFailure}

- ::= 9

- HoldCallArgument ::=

- SEQUENCE

- {callToBeHeld		ConnectionID,

- connectionReservation	ReserveConnection		OPTIONAL,

- extensions 		CSTACommonArguments	OPTIONAL}

- HoldCallResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-hold-call

## 9.11	Make call

- CSTA-make-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) make-call( 10) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID, CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- AccountInfo, AuthCode, CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- makeCall		OPERATION

- ARGUMENT	MakeCallArgument

- RESULT	MakeCallResult

- ERRORS	{universalFailure}

- ::= 10

- MakeCallArgument ::=

- SEQUENCE

- {callingDevice		DeviceID,

- calledDirectoryNumber	CalledDeviceID,

- deviceProfile		DeviceProfile				OPTIONAL,

- accountCode		[0] IMPLICIT AccountInfo		OPTIONAL,

- authCode		[1] IMPLICIT AuthCode			OPTIONAL,

- correlatorData		[2] IMPLICIT CorrelatorData		OPTIONAL,

- extensions		CSTACommonArguments		OPTIONAL}

- MakeCallResult ::=

- CHOICE

- {initiatedCall		ConnectionID,

- SEQUENCE

- {initiatedCall	ConnectionID,

- extensions	CSTACommonArguments} }

- END  -- of CSTA-make-call

## 9.12	Make predictive call

- CSTA-make-predictive-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) make-predictive-call( 11) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID, CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- AccountInfo, AuthCode, AllocationState, CorrelatorData, DeviceProfile

- FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- makePredictiveCall	OPERATION

- ARGUMENT	MakePredictiveCallArgument

- RESULT	MakePredictiveCallResult

- ERRORS	{universalFailure}

- ::= 11

- MakePredictiveCallArgument ::=

- SEQUENCE

- {callingDevice		DeviceID,

- calledDirectoryNumber	CalledDeviceID,

- allocation		AllocationState				OPTIONAL,

- deviceProfile		DeviceProfile				OPTIONAL,

- accountCode		[0] IMPLICIT AccountInfo		OPTIONAL,

- authCode		[1] IMPLICIT AuthCode			OPTIONAL,

- correlatorData		[2] IMPLICIT CorrelatorData		OPTIONAL,

- extensions		CSTACommonArguments		OPTIONAL}

- MakePredictiveCallResult ::=

- CHOICE

- {initiatedCall		ConnectionID,

- SEQUENCE

- {initiatedCall	ConnectionID,

- extensions	CSTACommonArguments} }

- END  -- of CSTA-make-predictive-call

## 9.13	Park call

- CSTA-park-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) park-call( 18) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- parkCall		OPERATION

- ARGUMENT	ParkCallArgument

- RESULT	ParkCallResult

- ERRORS	{universalFailure}

- ::= 18

- ParkCallArgument ::=

- SEQUENCE

- {callToPark		ConnectionID,

- parkTo			SubjectDeviceID,

- parkToDeviceProfile	DeviceProfile			OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- ParkCallResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-park-call

## 9.14	Query device

- CSTA-query-device

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) query-device( 12) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- QueryDeviceFeature, QueryDeviceInformation FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- queryDevice	OPERATION

- ARGUMENT	QueryDeviceArgument

- RESULT	QueryDeviceResult

- ERRORS	{universalFailure}

- ::= 12

- QueryDeviceArgument ::=

- SEQUENCE

- {device		DeviceID,

- feature		QueryDeviceFeature,

- extensions	CSTACommonArguments	OPTIONAL}

- QueryDeviceResult ::=

- CHOICE

- {deviceInformation 		QueryDeviceInformation,

- SEQUENCE

- {deviceInformation	QueryDeviceInformation,

- extensions		CSTACommonArguments} }

- END  -- of CSTA-query-device

## 9.15	Reconnect call

- CSTA-reconnect-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) reconnect-call( 13) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionDetails FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- reconnectCall	OPERATION

- ARGUMENT	ReconnectCallArgument

- RESULT	ReconnectCallResult

- ERRORS	{universalFailure}

- ::= 13

- ReconnectCallArgument ::=

- CHOICE

- {reconnectInfo		ConnectionDetails,

- SEQUENCE

- {reconnectInfo	ConnectionDetails,

- extensions 	CSTACommonArguments} }

- ReconnectCallResult ::=

- CHOICE

- {extensions 	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-reconnect-call

## 9.16	Retrieve call

- CSTA-retrieve-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) retrieve-call( 14) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- retrieveCall		OPERATION

- ARGUMENT	RetrieveCallArgument

- RESULT	RetrieveCallResult

- ERRORS	{universalFailure}

- ::= 14

- RetrieveCallArgument ::=

- CHOICE

- {callToBeRetrieved		ConnectionID,

- SEQUENCE

- {callToBeRetrieved	ConnectionID,

- extensions		CSTACommonArguments} }

- RetrieveCallResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-retrieve-call

## 9.17	Send DTMF tones

- CSTA-send-DTMF-tones

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) send-dtmf-tones( 19) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- sendDTMFTones	OPERATION

- ARGUMENT	SendDTMFTonesArgument

- RESULT	SendDTMFTonesResult

- ERRORS	{universalFailure}

- ::= 19

- SendDTMFTonesArgument ::=

- SEQUENCE

- {connectionToSendTones	ConnectionID,

- charactersToSend		IA5String,

- toneDuration			[0] IMPLICIT INTEGER		OPTIONAL,

- pauseDuration			[1] IMPLICIT INTEGER		OPTIONAL,

- extensions 			CSTACommonArguments	OPTIONAL}

- SendDTMFTonesResult ::=

- CHOICE

- {extensions 	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-send-DTMF-tones

## 9.18	Set feature

- CSTA-set-feature

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) set-feature( 15) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- DeviceProfile, SetDeviceFeature FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- setFeature		OPERATION

- ARGUMENT	SetFeatureArgument

- RESULT	SetFeatureResult

- ERRORS	{universalFailure}

- ::= 15

- SetFeatureArgument ::=

- SEQUENCE

- {device		DeviceID,

- feature		SetDeviceFeature,

- deviceProfile	DeviceProfile			OPTIONAL,

- extensions	CSTACommonArguments	OPTIONAL}

- SetFeatureResult ::=

- CHOICE

- {extensions 	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-set-feature

## 9.19	Single step conference

- CSTA-single-step-conference

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) single-step-conference( 20) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- ConnectionList FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- AccountInfo, AuthCode, CorrelatorData, DeviceProfile, ParticipationType

- FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- singleStepConf	OPERATION

- ARGUMENT	SingleStepConfArgument

- RESULT	SingleStepConfResult

- ERRORS	{universalFailure}

- ::= 20

- SingleStepConfArgument ::=

- SEQUENCE

- {activeCall		ConnectionID,

- deviceToJoin		DeviceID,

- participationType	ParticipationType		OPTIONAL,

- joiningDeviceProfile	DeviceProfile			OPTIONAL,

- accountCode		[0] IMPLICIT AccountInfo	OPTIONAL,

- authCode		[1] IMPLICIT AuthCode		OPTIONAL,

- correlatorData		[2] IMPLICIT CorrelatorData	OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- SingleStepConfResult ::=

- CHOICE

- {conferencedCall		ConnectionID,

- SEQUENCE

- {conferencedCall	ConnectionID,

- extensions		CSTACommonArguments} }

- END  -- of CSTA-single-step-conference

## 9.20	Single step transfer

- CSTA-single-step-transfer

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) single-step-conference( 50) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- ConnectionList FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- AccountInfo, AuthCode, CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- singleStepTrans	OPERATION

- ARGUMENT	SingleStepTransArgument

- RESULT	SingleStepTransResult

- ERRORS	{universalFailure}

- ::= 50

- SingleStepTransArgument ::=

- SEQUENCE

- {activeCall		ConnectionID,

- deviceToTransferTo	DeviceID,

- transferToDeviceProfile	DeviceProfile			OPTIONAL,

- accountCode		[0] IMPLICIT AccountInfo	OPTIONAL,

- authCode		[1] IMPLICIT AuthCode		OPTIONAL,

- correlatorData		[2] IMPLICIT CorrelatorData	OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- SingleStepTransResult ::=

- SEQUENCE

- {transferedCall		ConnectionID			OPTIONAL,

- connections		ConnectionList			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-single-step-transfer

## 9.21	Transfer call

- CSTA-transfer-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) transfer-call( 16) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID  FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- ConnectionList FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- ConnectionDetails FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- transferCall		OPERATION

- ARGUMENT	TransferCallArgument

- RESULT	TransferCallResult

- ERRORS	{universalFailure}

- ::= 16

- TransferCallArgument ::=

- CHOICE

- {transferInfo	ConnectionDetails,

- SEQUENCE

- {transferInfo	ConnectionDetails,

- extensions 	CSTACommonArguments} }

- TransferCallResult ::=

- SEQUENCE

- {transferredCall	ConnectionID			OPTIONAL,

- connections	ConnectionList			OPTIONAL,

- extensions 	CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-transfer-call

# 10	Event report service

- This clause defines the protocol for the event services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in Standard ECMA-217.

- NOTE 3

- The range of services supported on a particular association is specified in the application context at association time.

- The structure of an event is as described by the remote operation ‘cSTAEventReport’. This is a ROSE class 5 operation which has an argument containing some common information (that is provided for all events), an indication of the event type, and other information associated with that event.

- CSTA-event-report-definitions

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-report-definitions( 21) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- CallClearedEvent FROM CSTA-call-cleared-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-cleared-event( 22) }

- ConferencedEvent FROM CSTA-conferenced-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) conferenced-event( 23) }

- ConnectionClearedEvent FROM CSTA-connection-cleared-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-cleared-event( 24) }

- DeliveredEvent FROM CSTA-delivered-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) delivered-event( 25) }

- DivertedEvent FROM CSTA-diverted-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) diverted-event( 26) }

- EstablishedEvent FROM CSTA-established-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) established-event( 27) }

- FailedEvent FROM CSTA-failed-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) failed-event( 28) }

- HeldEvent FROM CSTA-held-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) held-event( 29) }

- NetworkReachedEvent FROM CSTA-network-reached-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) network-reached-event( 30) }

- OriginatedEvent FROM CSTA-originated-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) originated-event( 31) }

- QueuedEvent FROM CSTA-queued-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) queued-event( 32) }

- RetrievedEvent FROM CSTA-retrieved-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) retrieved-event( 33) }

- ServiceInitiatedEvent FROM CSTA-service-initiated-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) service-initiated-event( 34) }

- TransferedEvent FROM CSTA-transferred-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) transferred-event( 35) }

- AutoAnswerEvent FROM CSTA-auto-answer-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) auto-answer-event( 40) }

- CallInformationEvent FROM CSTA-call-information-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-information-event( 41) }

- DoNotDisturbEvent FROM CSTA-do-not-disturb-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) do-not-disturb-event( 42) }

- ForwardingEvent FROM CSTA-forwarding-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) forwarding-event( 43) }

- MessageWaitingEvent FROM CSTA-message-waiting-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) message-waiting-event( 44) }

- MicrophoneMuteEvent FROM CSTA-microphone-mute-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) microphone-mute-event( 45) }

- SpeakerMuteEvent FROM CSTA-speaker-mute-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) speaker-mute-event( 46) }

- SpeakerVolumeEvent FROM CSTA-speaker-volume-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) speaker-volume-event( 47) }

- AgentBusyEvent FROM CSTA-agent-busy-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) agent-busy-event( 56) }

- LoggedOnEvent FROM CSTA-logged-on-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) logged-on-event( 51) }

- LoggedOffEvent FROM CSTA-logged-off-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) logged-off-event( 52) }

- NotReadyEvent FROM CSTA-not-ready-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) not-ready-event( 53) }

- ReadyEvent FROM CSTA-ready-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) ready-event( 54) }

- WorkingAfterCallEvent FROM CSTA-working-after-call-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) working-after-call-event( 55) }

- BackInServiceEvent FROM CSTA-back-in-service-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) back-in-service-event( 61) }

- OutOfServiceEvent FROM CSTA-out-of-service-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) out-of-service-event( 62) }

- PrivateEvent FROM CSTA-private-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) private-event( 71) }

- PlayEvent FROM CSTA-play-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) play( 75) }

- RecordEvent FROM CSTA-record-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) record( 76) }

- ReviewEvent FROM CSTA-review-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) review( 77) }

- StopEvent FROM CSTA-stop-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) stop( 78) }

- SuspendPlayEvent FROM CSTA-suspend-play-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) suspend-play( 79) }

- SuspendRecordEvent FROM CSTA-suspend-record-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) suspend-record( 80) }

- VoiceAttributesChangeEvent FROM CSTA-voice-attributes-change-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) voice-attributes-change-event( 74) }

- MonitorCrossRefID FROM CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- cSTAEventReport	OPERATION

- ARGUMENT	CSTAEventReportArgument

- ::= 21

- CSTAEventReportArgument ::=

- SEQUENCE

- {crossRefIdentifier	MonitorCrossRefID,

- eventSpecificInfo	EventSpecificInfo,

- extensions		CSTACommonArguments	OPTIONAL}

- EventSpecificInfo ::=

- -- The event type (Call Cleared, Conferenced etc.) is encoded within this parameter

- CHOICE

- {callEvent			[0] CallEvent,

- featureEvent			[1] FeatureEvent,

- agentStateEvent		[2] AgentStateEvent,

- maintenanceEvent		[3] MaintenanceEvent,

- privateEvent			[4] PrivateEvent,

- voiceUnitEvent			[5] VoiceUnitEvent		}

- CallEvent ::=

- CHOICE

- {callClearedEvent		[0] CallClearedEvent,

- conferencedEvent		[1] ConferencedEvent,

- connectionClearedEvent	[2] ConnectionClearedEvent,

- deliveredEvent			[3] DeliveredEvent,

- divertedEvent			[4] DivertedEvent,

- establishedEvent		[5] EstablishedEvent,

- failedEvent			[6] FailedEvent,

- heldEvent			[7] HeldEvent,

- networkReachedEvent		[8] NetworkReachedEvent,

- originatedEvent			[9] OriginatedEvent,

- queuedEvent			[10] QueuedEvent,

- retrievedEvent			[11] RetrievedEvent,

- serviceInitiatedEvent		[12] ServiceInitiatedEvent,

- transferedEvent		[13] TransferedEvent		}

- FeatureEvent ::=

- CHOICE

- {autoAnswerEvent		[0] AutoAnswerEvent,

- callInformationEvent		[1] CallInformationEvent,

- doNotDisturbEvent		[2] DoNotDisturbEvent,

- forwardingEvent		[3] ForwardingEvent,

- messageWaitingEvent		[4] MessageWaitingEvent,

- microphoneMuteEvent		[5] MicrophoneMuteEvent,

- speakerMuteEvent		[6] SpeakerMuteEvent,

- speakerVolumeEvent		[7] SpeakerVolumeEvent		}

- AgentStateEvent ::=

- CHOICE

- {agentBusyEvent		[0] AgentBusyEvent,

- loggedOnEvent			[1] LoggedOnEvent,

- loggedOffEvent			[2] LoggedOffEvent,

- notReadyEvent			[3] NotReadyEvent,

- readyEvent			[4] ReadyEvent,

- workingAfterCallEvent		[5] WorkingAfterCallEvent	}

- MaintenanceEvent ::=

- CHOICE

- {backInServiceEvent		[0] BackInServiceEvent,

- outOfServiceEvent		[1] OutOfServiceEvent		}

- VoiceUnitEvent ::=

- CHOICE

- {playEvent			[0] PlayEvent,

- recordEvent			[1] RecordEvent,

- reviewEvent			[2] ReviewEvent,

- stopEvent			[3] StopEvent,

- suspendPlayEvent		[4] SuspendPlayEvent,

- suspendRecordEvent		[5] SuspendRecordEvent,

- voiceAttributesChangeEvent	[6] VoiceAttributesChangeEvent	}

- END  -- of CSTA-event-report-definitions

## 10.1	Call events

- Each event contains a ConnectionID which identifies the object of interest. Other parameters may also be used to identify the relevant devices.

- Events may result from a call interacting with switch features that had been previously set or invoked. These features and their settings may be reflected in an Event as a value of the EventCause parameter.

### 10.1.1	Call cleared

- CSTA-call-cleared-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-cleared-event( 22) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState  FROM  CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- CallClearedEvent ::=

- SEQUENCE

- {clearedCall		ConnectionID,

- localConnectionInfo	LocalConnectionState		OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- cause			EventCause			OPTIONAL}

- END  -- of CSTA-call-cleared-event

### 10.1.2	Conferenced

- CSTA-conferenced-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) conferenced-event( 23) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState, ConnectionList FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- ConferencedEvent ::=

- SEQUENCE

- {primaryOldCall		ConnectionID,

- secondaryOldCall	ConnectionID			OPTIONAL,

- confController		SubjectDeviceID,

- addedParty		SubjectDeviceID,

- conferenceConnectionsConnectionList			OPTIONAL,

- localConnectionInfo	LocalConnectionState		OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- cause			EventCause			OPTIONAL}

- END  -- of CSTA-conferenced-event

### 10.1.3	Connection cleared

- CSTA-connection-cleared-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-cleared-event( 24) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- ConnectionClearedEvent ::=

- SEQUENCE

- {droppedConnection	ConnectionID,

- releasingDevice	SubjectDeviceID,

- localConnectionInfo	LocalConnectionState		OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- cause			EventCause			OPTIONAL}

- END  -- of CSTA-connection-cleared-event

### 10.1.4	Delivered

- CSTA-delivered-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) delivered-event( 25) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID, CallingDeviceID, CalledDeviceID, RedirectionDeviceID

- FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- DeliveredEvent ::=

- SEQUENCE

- {connection		ConnectionID,

- alertingDevice		SubjectDeviceID,

- callingDevice		CallingDeviceID,

- calledDevice		CalledDeviceID,

- lastRedirectionDevice	RedirectionDeviceID,

- originatingConnection	ConnectionID			OPTIONAL,

- localConnectionInfo	LocalConnectionState		OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- cause			EventCause			OPTIONAL}

- END  -- of CSTA-delivered-event

### 10.1.5	Diverted

- CSTA-diverted-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) diverted-event( 26) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- DivertedEvent ::= SEQUENCE

- {connection		ConnectionID		OPTIONAL,

- divertingDevice		SubjectDeviceID,

- newDestination		SubjectDeviceID,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of diverted-event

### 10.1.6	Established

- CSTA-established-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) established-event( 27) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- SubjectDeviceID, CalledDeviceID, CallingDeviceID, RedirectionDeviceID

- FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) };

- EstablishedEvent ::=

- SEQUENCE

- {establishedConnection	ConnectionID,

- answeringDevice	SubjectDeviceID,

- callingDevice		CallingDeviceID,

- calledDevice		CalledDeviceID,

- lastRedirectionDevice	RedirectionDeviceID,

- originatingConnection	ConnectionID		OPTIONAL,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-established-event

### 10.1.7	Failed

- CSTA-failed-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) failed-event( 28) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID, CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- FailedEvent ::=

- SEQUENCE

- {failedConnection	ConnectionID,

- failingDevice		SubjectDeviceID,

- calledDevice		CalledDeviceID,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-failed-event

### 10.1.8	Held

- CSTA-held-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) held-event( 29) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- HeldEvent ::=

- SEQUENCE

- {heldConnection	ConnectionID,

- holdingDevice		SubjectDeviceID,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-held-event

### 10.1.9	Network reached

- CSTA-network-reached-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) network-reached-event( 30) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID, CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- NetworkReachedEvent ::=

- SEQUENCE

- {outboundConnection	ConnectionID,

- trunkUsed		SubjectDeviceID,

- calledDevice		CalledDeviceID,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-network-reached-event

### 10.1.10	Originated

- CSTA-originated-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) originated-event( 31) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- CalledDeviceID, CallingDeviceID, SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- OriginatedEvent ::=

- SEQUENCE

- {originatedConnection	ConnectionID,

- callingDevice		CallingDeviceID,

- calledDevice		CalledDeviceID,

- originatingDevice	SubjectDeviceID	OPTIONAL,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-originated-event

### 10.1.11	Queued

- CSTA-queued-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) queued-event( 32) }

- DEFINITIONS    ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- CalledDeviceID, CallingDeviceID, RedirectionDeviceID, SubjectDeviceID

- FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData, NoOfCallsInQueue, NoOfCallsInFront FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- QueuedEvent ::=

- SEQUENCE

- {queuedConnection	ConnectionID,

- queue			SubjectDeviceID,

- callingDevice		CallingDeviceID,

- calledDevice		CalledDeviceID,

- lastRedirectionDevice	RedirectionDeviceID,

- numberQueued	[0] IMPLICIT	NoOfCallsInQueue	OPTIONAL,

- callsInFront		[1] IMPLICIT	NoOfCallsInFront	OPTIONAL,

- localConnectionInfo	LocalConnectionState			OPTIONAL,

- correlatorData		CorrelatorData				OPTIONAL,

- cause			EventCause				OPTIONAL}

- END  -- of CSTA-queued-event

### 10.1.12	Retrieved

- CSTA-retrieved-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) retrieved-event( 33) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- RetrievedEvent ::=

- SEQUENCE

- {retrievedConnection	ConnectionID,

- retrievingDevice	SubjectDeviceID,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-retrieved-event

### 10.1.13	Service initiated

- CSTA-service-initiated-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) service-initiated-event( 34) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) };

- ServiceInitiatedEvent ::=

- SEQUENCE

- {initiatedConnection	ConnectionID,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-service-initiated-event

### 10.1.14	Transferred

- CSTA-transferred-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) transferred-event( 35) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState, ConnectionList FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- TransferedEvent ::=

- SEQUENCE

- {primaryOldCall		ConnectionID,

- secondaryOldCall	ConnectionID		OPTIONAL,

- transferringDevice	SubjectDeviceID,

- transferredDevice	SubjectDeviceID,

- transferredConnections	ConnectionList		OPTIONAL,

- localConnectionInfo	LocalConnectionState	OPTIONAL,

- correlatorData		CorrelatorData		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-transferred-event

## 10.2	Feature events

### 10.2.1	Auto answer

- CSTA-auto-answer-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) auto-answer-event( 40) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- AutoAnswerEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- autoAnswerOn		BOOLEAN}

- END  -- of CSTA-auto-answer-event

### 10.2.2	Call information

- CSTA-call-information-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-information-event( 41) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- AccountInfo, AuthCode, CorrelatorData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- CallInformationEvent ::=

- SEQUENCE

- {connection		ConnectionID,

- device			SubjectDeviceID,

- accountInfo		[0] IMPLICIT	AccountInfo	OPTIONAL,

- authorisationCode	[1] IMPLICIT	AuthCode	OPTIONAL,

- correlatorData		[2] IMPLICIT	CorrelatorData	OPTIONAL}

- END  -- of CSTA-call-information-event

### 10.2.3	Do not disturb

- CSTA-do-not-disturb-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) do-not-disturb-event( 42) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- DoNotDisturbEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- doNotDisturbOn	BOOLEAN}

- END  -- of CSTA-do-not-disturb-event

### 10.2.4	Forwarding

- CSTA-forwarding-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) forwarding-event( 43) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ForwardParameter FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- ForwardingEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- forwardingInformation	ForwardParameter,

- forwardedTo		SubjectDeviceID		OPTIONAL}

- END  -- of CSTA-forwarding-event

### 10.2.5	Message waiting

- CSTA-message-waiting-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) message-waiting-event( 44) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- MessageWaitingEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- invokingDevice		SubjectDeviceID,

- messageWaitingOn	BOOLEAN}

- END  -- of CSTA-message-waiting-event

### 10.2.6	Microphone mute

- CSTA-microphone-mute-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) microphone-mute-event( 45) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- MicrophoneMuteEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- microphoneMuteOn	BOOLEAN}

- END  -- of CSTA-microphone-mute-event

### 10.2.7	Speaker mute

- CSTA-speaker-mute-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) speaker-mute-event( 46) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- SpeakerMuteEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- speakerMuteOn	BOOLEAN}

- END  -- of CSTA-speaker-mute-event

### 10.2.8	Speaker volume

- CSTA-speaker-volume-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) speaker-volume-event( 47) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- SpeakerVolumeEvent ::=

- SEQUENCE

- {device			SubjectDeviceID,

- speakerVolume	INTEGER (0 .. 100)}

- END  -- of CSTA-speaker-volume-event

## 10.3	Agent state events

### 10.3.1	Agent busy

- CSTA-agent-busy-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) agent-busy-event( 56) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- AgentID, AgentGroup FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- AgentBusyEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID	[10] IMPLICIT 	AgentID	OPTIONAL,

- agentGroup	AgentGroup 			OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-agent-busy-event

### 10.3.2	Agent logged on

- CSTA-logged-on-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) logged-on-event( 51) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- AgentID, AgentGroup, AgentPassword FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- LoggedOnEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID	[10] IMPLICIT	AgentID	OPTIONAL,

- agentGroup	AgentGroup			OPTIONAL,

- password	AgentPassword			OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-logged-on-event

### 10.3.3	Agent logged off

- CSTA-logged-off-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) logged-off-event( 52) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- AgentID, AgentGroup, AgentPassword FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- LoggedOffEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID	[10] IMPLICIT	AgentID	OPTIONAL,

- agentGroup	AgentGroup			OPTIONAL,

- password	AgentPassword			OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-logged-off-event

### 10.3.4	Agent not ready

- CSTA-not-ready-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) not-ready-event( 53) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123)}

- AgentID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- NotReadyEvent ::=

- SEQUENCE

- {agentDevice		SubjectDeviceID,

- agentID		AgentID		OPTIONAL,

- cause			EventCause		OPTIONAL}

- END  -- of CSTA-not-ready-event

### 10.3.5	Agent ready

- CSTA-ready-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) ready-event( 54) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- AgentID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- ReadyEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID	AgentID		OPTIONAL,

- cause		EventCause		OPTIONAL}

- END  -- of CSTA-ready-event

### 10.3.6	Agent working after call

- CSTA-working-after-call-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) working-after-call-event( 55) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- AgentID, AgentGroup FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- WorkingAfterCallEvent ::=

- SEQUENCE

- {agentDevice	SubjectDeviceID,

- agentID	[10] IMPLICIT 	AgentID	OPTIONAL,

- agentGroup	AgentGroup 			OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-working-after-call-event

## 10.4	Maintenance events

### 10.4.1	Back in service

- CSTA-back-in-service-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) back-in-service-event( 61) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- BackInServiceEvent ::=

- SEQUENCE

- {device		DeviceID,

- cause		EventCause	OPTIONAL}

- END  -- of CSTA-back-in-service-event

### 10.4.2	Out of service

- CSTA-out-of-service-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) out-of-service-event( 62) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- OutOfServiceEvent ::=

- SEQUENCE

- {device		DeviceID,

- cause		EventCause	OPTIONAL}

- END  -- of CSTA-out-of-service-event

## 10.5	Private events

- CSTA-private-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) private-event( 71) }

- DEFINITIONS ::=

- BEGIN

- PrivateEvent ::= NULL	-- 	The actual encoding of the private event is added here,

- --	replacing NULL with another valid ASN.1 type.

- END  -- of CSTA-private-event

## 10.6.	Voice unit events

### 10.6.1	Play

- CSTA-play-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) play( 75) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, Speed FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- PlayEvent ::=

- SEQUENCE

- {connection	ConnectionID,

- message	MessageID,

- length		[0] IMPLICIT	INTEGER	OPTIONAL,

- currentPosition [1] IMPLICIT	INTEGER	OPTIONAL,

- speed		[2] IMPLICIT	Speed		OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-play-event

### 10.6.2	Record

- CSTA-record-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) record( 76) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- RecordEvent ::=

- SEQUENCE

- {connection	ConnectionID,

- message	MessageID,

- length		[0] IMPLICIT	INTEGER	OPTIONAL,

- currentPosition [1] IMPLICIT	INTEGER	OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-record-event

### 10.6.3	Review

- CSTA-review-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) review( 77) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- ReviewEvent ::=

- SEQUENCE

- {connection	ConnectionID,

- message	MessageID,

- length		[0] IMPLICIT	INTEGER	OPTIONAL,

- currentPosition [1] IMPLICIT	INTEGER	OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-review-event

### 10.6.4	Stop

- CSTA-stop-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) stop( 78) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- StopEvent ::=

- SEQUENCE

- {connection	ConnectionID,

- message	MessageID,

- length		[0] IMPLICIT	INTEGER	OPTIONAL,

- currentPosition [1] IMPLICIT	INTEGER	OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-stop-event

### 10.6.5	Suspend play

- CSTA-suspend-play-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) suspend-play( 79) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- SuspendPlayEvent ::=

- SEQUENCE

- {connection	ConnectionID,

- message	MessageID,

- length		[0] IMPLICIT	INTEGER	OPTIONAL,

- currentPosition [1] IMPLICIT	INTEGER	OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-suspend-play-event

### 10.6.6	Suspend record

- CSTA-suspend-record-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) suspend-record( 80) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- SuspendRecordEvent ::=

- SEQUENCE

- {connection	ConnectionID,

- message	MessageID,

- length		[0] IMPLICIT	INTEGER	OPTIONAL,

- currentPosition [1] IMPLICIT	INTEGER	OPTIONAL,

- cause		EventCause			OPTIONAL}

- END  -- of CSTA-suspend-record-event

### 10.6.7	Voice attributes change

- CSTA-voice-attributes-change-event

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) voice-attributes-change-event( 74) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- -- Data Types --

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, Speed FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- VoiceAttributesChangeEvent ::=

- SEQUENCE

- {connection		ConnectionID,

- message		MessageID,

- speakerVolume	[0] IMPLICIT INTEGER (0 .. 100)	OPTIONAL,

- recordingLevel		[1] IMPLICIT INTEGER (0 .. 100)	OPTIONAL,

- speed			[2] IMPLICIT Speed			OPTIONAL,

- currentPosition 	[3] IMPLICIT INTEGER			OPTIONAL,

- cause			EventCause				OPTIONAL}

- END  -- of CSTA-voice-attributes-change-event

# 11	Computing function services

- This clause defines the protocol for the Computing function services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in Standard ECMA-217.

- NOTE 4

- The range of services supported on a particular association is specified in the application context at association time.

## 11.1	Route request

- CSTA-route-request

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) route-request( 81) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CalledDeviceID, CallingDeviceID, SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- CorrelatorData, DeviceClass, DeviceProfile, SelectValue, PriorityValue, RoutingCrossRefID

- FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- routeRequest OPERATION

- ARGUMENT	RouteRequestArgument

- ERRORS	{universalFailure}

- ::= 31

- RouteRequestArgument ::=

- SEQUENCE

- {crossRefIdentifier	RoutingCrossRefID,

- currentRoute		CalledDeviceID,

- callingDevice		CallingDeviceID			OPTIONAL,

- routingDevice		SubjectDeviceID		OPTIONAL,

- routedCall		ConnectionID			OPTIONAL,

- routeSelAlgorithm	SelectValue			OPTIONAL,

- priority			PriorityValue			OPTIONAL,

- deviceProfile		DeviceProfile			OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-route-request

## 11.2	Reroute request

- CSTA-re-route-request

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) re-route-request( 82) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- RoutingCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) }

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) };

- reRouteRequest OPERATION

- ARGUMENT	ReRouteRequestArgument

- ERRORS	{universalFailure}

- ::= 32

- ReRouteRequestArgument ::=

- SEQUENCE

- {crossRefIdentifier	RoutingCrossRefID,

- extensions 		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-re-route-request

## 11.3	Route select request

- CSTA-Route-select-request

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) route-select-request( 83) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- RouteUsedFlag, RetryValue, RoutingCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) }

- CorrelatorData, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) };

- routeSelectRequest	OPERATION

- ARGUMENT	RouteSelectRequestArgument

- ERRORS	{universalFailure}

- ::= 33

- RouteSelectRequestArgument ::=

- SEQUENCE

- {crossRefIdentifier	RoutingCrossRefID,

- routeSelected		CalledDeviceID,

- remainRetry		RetryValue			OPTIONAL,

- deviceProfile		DeviceProfile			OPTIONAL,

- routeUsedReq		RouteUsedFlag			OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- extensions 		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-route-select-request

## 11.4	Route used request

- CSTA-route-used-request

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) route-used-request( 84) }

- DEFINITIONS    ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- CallingDeviceID, CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- CorrelatorData, DomainValue, RoutingCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) }

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) };

- routeUsedRequest	OPERATION

- ARGUMENT	RouteUsedRequestArgument

- ERRORS	{universalFailure}

- ::= 34

- RouteUsedRequestArgument ::=

- SEQUENCE

- {crossRefIdentifier	RoutingCrossRefID,

- routeUsed		CalledDeviceID,

- callingDevice		CallingDeviceID			OPTIONAL,

- domain			DomainValue			OPTIONAL,

- correlatorData		CorrelatorData			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-route-used-request

## 11.5	Route end request

- CSTA-route-end-request

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) route-end-request( 85) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- RoutingCrossRefID  FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) }

- universalFailure, UniversalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) };

- routeEndRequest	OPERATION

- ARGUMENT	RouteEndRequestArgument

- ERRORS	{universalFailure}

- ::= 35

- RouteEndRequestArgument ::=

- SEQUENCE

- {crossRefIdentifier	RoutingCrossRefID,

- errorValue		UniversalFailure			OPTIONAL,

- extensions 		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-route-end-request

# 12	Bidirectional services

- This clause defines the protocol for the Bidirectional services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in Standard ECMA-217.

- NOTE 5

- The range of services supported on a particular association is specified in the application context at association time.

## 12.1	Escape

- CSTA-escape-service

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) escape-service( 91) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CSTASecurityData FROM CSTA-security

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) security( 128) }

- CSTACommonArguments, CSTAPrivateData FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- escapeService	OPERATION

- ARGUMENT	EscapeServiceArgument

- RESULT	EscapeServiceResult

- ERRORS	{universalFailure}

- ::=  51

- EscapeServiceArgument ::=

- SEQUENCE

- {security	[0] IMPLICIT	CSTASecurityData			OPTIONAL,

- privateData	[1] IMPLICIT	SEQUENCE OF CSTAPrivateData}

- EscapeServiceResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-escape-service

## 12.2	System status

- CSTA-system-status

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) system-status( 92) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- SystemStatus FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- systemStatus	OPERATION

- ARGUMENT	SystemStatusArgument

- RESULT	SystemStatusResult

- ERRORS	{universalFailure}

- ::= 52

- SystemStatusArgument ::=

- CHOICE

- {systemStatus		SystemStatus,

- SEQUENCE

- {systemStatus	SystemStatus,

- extensions	CSTACommonArguments} }

- SystemStatusResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-system-status

# 13	Status reporting services

- This clause defines the protocol for the Status reporting services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in Standard ECMA-217.

- NOTE 6

- The range of services supported on a particular association is specified in the application context at association time.

## 13.1	Monitor start

- CSTA-monitor-start

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) monitor-start( 101) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- MonitorObject, MonitorFilter, MonitorType, MonitorCrossRefID FROM CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- monitorStart	OPERATION

- ARGUMENT	MonitorStartArgument

- RESULT	MonitorStartResult

- ERRORS	{universalFailure}

- ::= 71

- MonitorStartArgument ::=

- SEQUENCE

- {monitorObject		MonitorObject,

- monitorFilter		MonitorFilter			OPTIONAL,

- monitorType		MonitorType			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- MonitorStartResult ::=

- SEQUENCE

- {crossRefIdentifier	MonitorCrossRefID,

- monitorFilter		MonitorFilter			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-monitor-start

## 13.2	Change monitor filter

- CSTA-change-monitor-filter

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) change-monitor-filter( 102) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- MonitorFilter, MonitorCrossRefID FROM CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- changeMonitorFilter	OPERATION

- ARGUMENT	ChangeMonitorFilterArgument

- RESULT	ChangeMonitorFilterResult

- ERRORS	{universalFailure}

- ::= 72

- ChangeMonitorFilterArgument ::=

- SEQUENCE

- {monitorCrossRefID	MonitorCrossRefID,

- filterlist			MonitorFilter,

- extensions		CSTACommonArguments	OPTIONAL}

- ChangeMonitorFilterResult ::=

- CHOICE

- {filterList		[0] IMPLICIT	MonitorFilter,

- SEQUENCE

- {filterList	MonitorFilter,

- extensions	CSTACommonArguments} }

- END  -- of CSTA-change-monitor-filter

## 13.3	Monitor stop

- CSTA-monitor-stop

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) monitor-stop( 103) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- MonitorCrossRefID FROM CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- monitorStop	OPERATION

- ARGUMENT	MonitorStopArgument

- RESULT	MonitorStopResult

- ERRORS 	{universalFailure}

- ::= 73

- MonitorStopArgument ::=

- CHOICE

- {crossRefIdentifier	MonitorCrossRefID,

- SEQUENCE

- {crossRefIdentifier	MonitorCrossRefID,

- extensions		CSTACommonArguments} }

- MonitorStopResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-monitor-stop

## 13.4	Snapshot device

- CSTA-snapshot-device

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) snapshot-device( 104) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DeviceID FROM Device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- SnapshotDeviceData FROM CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- snapshotDevice	OPERATION

- ARGUMENT	SnapshotDeviceArgument

- RESULT	SnapshotDeviceResult

- ERRORS	{universalFailure}

- ::= 74

- SnapshotDeviceArgument ::=

- CHOICE

- {snapshotObject		DeviceID,

- SEQUENCE

- {snapshotObject	DeviceID,

- extensions		CSTACommonArguments} }

- SnapshotDeviceResult ::=

- CHOICE

- {snapshotData			SnapshotDeviceData,

- SEQUENCE

- {snapshotData		SnapshotDeviceData,

- extensions		CSTACommonArguments} }

- END  -- of CSTA-snapshot-device

## 13.5	Snapshot call

- CSTA-snapshot-call

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) snapshot-call( 105) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- SnapshotCallData FROM CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- snapshotCall	OPERATION

- ARGUMENT	SnapshotCallArgument

- RESULT	SnapshotCallResult

- ERRORS	{universalFailure}

- ::= 75

- SnapshotCallArgument ::=

- CHOICE

- {snapshotObject		ConnectionID,

- SEQUENCE

- {snapshotObject	ConnectionID,

- extensions 		CSTACommonArguments} }

- SnapshotCallResult ::=

- CHOICE

- {snapshotData			SnapshotCallData,

- SEQUENCE

- {snapshotData		SnapshotCallData,

- extensions 		CSTACommonArguments} }

- END  -- of CSTA-snapshot-call

# 14	Input/output services

- This clause defines the protocol for the input/output (I/O) services of CSTA, using ASN.1. Text descriptions of the CSTA services are provided in Standard ECMA-217.

## 14.1	Start data path

- CSTA-start-data-path

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) start-data-path( 110) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CSTAObject FROM CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) switching-function-objects( 122) }

- DataPathDirection, DataPathType, IOCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- startDataPath	OPERATION

- ARGUMENT	StartDataPathArgument

- RESULT	StartDataPathResult

- ERRORS	{universalFailure}

- ::= 110

- StartDataPathArgument ::=

- SEQUENCE

- {device				CSTAObject,

- dataPathDirection		[0] IMPLICIT DataPathDirection		OPTIONAL,

- dataPathType			[1] IMPLICIT DataPathType		OPTIONAL,

- numberOfCharactersToCollect	[2] IMPLICIT INTEGER			OPTIONAL,

- terminationCharacter		IA5String				OPTIONAL,

- timeout				[3] IMPLICIT INTEGER			OPTIONAL,

- extensions			CSTACommonArguments		OPTIONAL}

- StartDataPathResult ::=

- SEQUENCE

- {ioCrossRefID			IOCrossRefID,

- numberOfCharactersToCollect	[0] IMPLICIT INTEGER			OPTIONAL,

- terminationCharacter		IA5String				OPTIONAL,

- timeout				[1] IMPLICIT INTEGER			OPTIONAL,

- extensions			CSTACommonArguments		OPTIONAL}

- END  -- of CSTA-start-data-path

## 14.2	Stop data path

- CSTA-stop-data-path

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) stop-data-path( 111) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- IOCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- stopDataPath	OPERATION

- ARGUMENT	StopDataPathArgument

- RESULT	StopDataPathResult

- ERRORS	{universalFailure}

- ::= 111

- StopDataPathArgument ::=

- CHOICE

- {ioCrossRefID		IOCrossRefID,

- SEQUENCE

- {ioCrossRefID	IOCrossRefID,

- extensions 	CSTACommonArguments} }

- StopDataPathResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-stop-data-path

## 14.3	Send data

- CSTA-send-data

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) send-data( 112) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- IOCrossRefID, IOData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- sendData		OPERATION

- ARGUMENT	SendDataArgument

- RESULT	SendDataResult

- ERRORS	{universalFailure}

- ::= 112

- SendDataArgument ::=

- SEQUENCE

- {ioCrossRefID		IOCrossRefID,

- data			IOData,

- cause			EventCause			OPTIONAL,

- extensions 		CSTACommonArguments	OPTIONAL}

- SendDataResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-send-data

## 14.4	Send multicast data

- CSTA-send-multicast-data

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) send-multicast-data( 113) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- IOCrossRefIDList, IOData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- sendMulticastData	OPERATION

- ARGUMENT	SendMulticastDataArgument

- RESULT	SendMulticastDataResult

- ERRORS	{universalFailure}

- ::= 113

- SendMulticastDataArgument ::=

- SEQUENCE

- {ioCrossRefIDList	IOCrossRefIDList,

- data			IOData,

- extensions		CSTACommonArguments}

- SendMulticastDataResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-send-multicast-data

## 14.5	Send broadcast data

- CSTA-send-broadcast-data

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) send-broadcast-data( 114) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DataPathType, IOData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- sendBroadcastData	OPERATION

- ARGUMENT	SendBroadcastDataArgument

- RESULT	SendBroadcastDataResult

- ERRORS	{universalFailure}

- ::= 114

- SendBroadcastDataArgument ::=

- SEQUENCE

- {data			IOData,

- dataPathType		DataPathType			OPTIONAL,

- extensions 		CSTACommonArguments	OPTIONAL}

- SendBroadcastDataResult ::=

- CHOICE

- {extensions 	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-send-broadcast-data

## 14.6	Suspend data path

- CSTA-suspend-data-path

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) suspend-data-path( 115) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- IOCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- suspendDataPath	OPERATION

- ARGUMENT	SuspendDataPathArgument

- RESULT	SuspendDataPathResult

- ERRORS	{universalFailure}

- ::= 115

- SuspendDataPathArgument ::=

- CHOICE

- {ioCrossRefID			IOCrossRefID,

- SEQUENCE

- {ioCrossRefID		IOCrossRefID,

- extensions		CSTACommonArguments} }

- SuspendDataPathResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL }

- END  -- of CSTA-suspend-data-path

## 14.7	Data path suspended

- CSTA-data-path-suspended

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) data-path-suspended( 116) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- IOCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- dataPathSuspended	OPERATION

- ARGUMENT		DataPathSuspendedArgument

- RESULT		DataPathSuspendedResult

- ERRORS		{universalFailure}

- ::= 116

- DataPathSuspendedArgument ::=

- CHOICE

- {ioCrossRefID			IOCrossRefID,

- SEQUENCE

- {ioCrossRefID		IOCrossRefID,

- extensions		CSTACommonArguments} }

- DataPathSuspendedResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL }

- END  -- of CSTA-data-path-suspended

## 14.8	Resume data path

- CSTA-resume-data-path

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) resume-data-path( 117) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- IOCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- resumeDataPath	OPERATION

- ARGUMENT	ResumeDataPathArgument

- RESULT	ResumeDataPathResult

- ERRORS	{universalFailure}

- ::= 117

- ResumeDataPathArgument ::=

- CHOICE

- {ioCrossRefID			IOCrossRefID,

- SEQUENCE

- {ioCrossRefID		IOCrossRefID,

- extensions		CSTACommonArguments} }

- ResumeDataPathResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL }

- END  -- of CSTA-resume-data-path

## 14.9	Data path resumed

- CSTA-data-path-resumed

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) data-path-resumed( 118) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- IOCrossRefID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- dataPathResumed	OPERATION

- ARGUMENT	DataPathResumedArgument

- RESULT	DataPathResumedResult

- ERRORS	{universalFailure}

- ::= 118

- DataPathResumedArgument ::=

- CHOICE

- {ioCrossRefID			IOCrossRefID,

- SEQUENCE

- {ioCrossRefID		IOCrossRefID,

- extensions		CSTACommonArguments} }

- DataPathResumedResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL }

- END  -- of CSTA-data-path-resumed

## 14.10	Fast data

- CSTA-fast-data

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) fast-data( 119) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- CSTAObject FROM CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) switching-function-objects( 122) }

- DataPathDirection, DataPathType, IOCrossRefID, IOData FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- fastData		OPERATION

- ARGUMENT	FastDataArgument

- RESULT	FastDataResult

- ERRORS	{universalFailure}

- ::= 119

- FastDataArgument ::=

- SEQUENCE

- {device			CSTAObject,

- data			IOData,

- dataPathDirection	[0] IMPLICIT DataPathDirection		OPTIONAL,

- dataPathType		[1] IMPLICIT DataPathType		OPTIONAL,

- extensions		CSTACommonArguments		OPTIONAL}

- FastDataResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-fast-data

# 15	Voice unit services

## 15.1	Concatenate message

- CSTA-concatenate-message

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) concatenate-message( 500) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- MessageID, MessageIDList FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- concatenateMessage	OPERATION

- ARGUMENT		ConcatenateMessageArgument

- RESULT		ConcatenateMessageResult

- ERRORS		{universalFailure}

- ::= 500

- ConcatenateMessageArgument ::=

- CHOICE

- {messagesToConcatenate	[0]IMPLICIT	MessageIDList,

- SEQUENCE

- {messagesToConcatenate	MessageIDList,

- extensions			CSTACommonArguments} }

- ConcatenateMessageResult ::=

- CHOICE

- {concatenatedMessage			MessageID,

- SEQUENCE

- {concatenatedMessage		MessageID,

- extensions			CSTACommonArguments} }

- END  -- of CSTA-concatenate-message

## 15.2	Delete message

- CSTA-delete-message

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) delete-message( 501) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- deleteMessage	OPERATION

- ARGUMENT	DeleteMessageArgument

- RESULT	DeleteMessageResult

- ERRORS	{universalFailure}

- ::= 501

- DeleteMessageArgument ::=

- CHOICE

- {messageToBeDeleted		MessageID,

- SEQUENCE

- {messageToBeDeleted	MessageID,

- extensions		CSTACommonArguments} }

- DeleteMessageResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-delete-message

## 15.3	Play message

- CSTA-play-message

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) play-message( 502) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, TerminatingConditions FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- playMessage	OPERATION

- ARGUMENT	PlayMessageArgument

- RESULT	PlayMessageResult

- ERRORS	{universalFailure}

- ::= 502

- PlayMessageArgument ::=

- SEQUENCE

- {messageToBePlayed	MessageID,

- overConnection	ConnectionID,

- duration		INTEGER			OPTIONAL,

- termination		TerminatingConditions		OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- PlayMessageResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-play-message

## 15.4	Query voice attribute

- CSTA-query-voice-attribute

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) query-voice-attribute( 503) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- AttributeInfo, MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- queryVoiceAttribute	OPERATION

- ARGUMENT	QueryVoiceAttributeArgument

- RESULT	QueryVoiceAttributeResult

- ERRORS	{universalFailure}

- ::= 503

- QueryVoiceAttributeArgument ::=

- SEQUENCE

- {messageToQuery	MessageID,

- attributeToQuery	ENUMERATED

- {encodingAlgorithm	(0),

- samplingRate		(1),

- duration		(2),

- fileName		(3),

- currentPosition		(4),

- currentSpeed		(5),

- currentVolume		(6),

- currentLevel		(7),

- currentState		(8)	},

- connection		ConnectionID					OPTIONAL,

- extensions		CSTACommonArguments			OPTIONAL}

- QueryVoiceAttributeResult ::=

- SEQUENCE

- {attribute		AttributeInfo,

- extensions		CSTACommonArguments	OPTIONAL}

- END  -- of CSTA-query-voice-attribute

## 15.5	Record message

- CSTA-record-message

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) record-message( 511) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, TerminatingConditions FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- recordMessage	OPERATION

- ARGUMENT	RecordMessageArgument

- RESULT	RecordMessageResult

- ERRORS	{universalFailure}

- ::= 511

- RecordMessageArgument ::=

- SEQUENCE

- {callToBeRecorded	ConnectionID,

- samplingRate		REAL						OPTIONAL,

- encodingAlg		[0]IMPLICIT ENUMERATED

- {aDPCM6K		(0),

- aDPCM8K		(1),

- muLawPCM6K		(2),

- aLawPCM6K		(3)	}	OPTIONAL,

- maxDuration		[1]IMPLICIT INTEGER				OPTIONAL,

- termination		[2]IMPLICIT TerminatingConditions		OPTIONAL,

- extensions		CSTACommonArguments			OPTIONAL}

- RecordMessageResult ::=

- CHOICE

- {identityOfMessageRecorded		MessageID,

- SEQUENCE

- {identityOfMessageRecorded	MessageID,

- extensions			CSTACommonArguments} }

- END  -- of CSTA-record-message

## 15.6	Reposition

- CSTA-reposition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) reposition( 504) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, Period FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- reposition		OPERATION

- ARGUMENT	RepositionArgument

- RESULT	RepositionResult

- ERRORS	{universalFailure}

- ::= 504

- RepositionArgument ::=

- SEQUENCE

- {connection		ConnectionID,

- periodOfReposition	Period,

- mesageToReposition	MessageID			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- RepositionResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-reposition

## 15.7	Resume

- CSTA-resume

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) resume( 505) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- resume		OPERATION

- ARGUMENT	ResumeArgument

- RESULT	ResumeResult

- ERRORS	{universalFailure}

- ::= 505

- ResumeArgument ::=

- SEQUENCE

- {connection		ConnectionID,

- mesageToResume	MessageID			OPTIONAL,

- duration		INTEGER			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- ResumeResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-resume

## 15.8	Review

- CSTA-review

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) review( 506) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, Period FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- review		OPERATION

- ARGUMENT	ReviewArgument

- RESULT	ReviewResult

- ERRORS	{universalFailure}

- ::= 506

- ReviewArgument ::=

- SEQUENCE

- {connection		ConnectionID,

- periodToReview	Period,

- mesageToReview	MessageID			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- ReviewResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-review

## 15.9	Set voice attribute

- CSTA-set-voice-attribute

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) set-voice-attribute( 507) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- EventCause FROM CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID, Speed FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- setVoiceAttribute	OPERATION

- ARGUMENT	SetVoiveAttributeArgument

- RESULT	SetVoiveAttributeResult

- ERRORS	{universalFailure}

- ::= 507

- SetVoiceAttributeArgument ::=

- SEQUENCE

- {connection		ConnectionID,

- attributeToSet		CHOICE

- {speed		 [0] IMPLICIT Speed,

- speakerVolume [1] IMPLICIT INTEGER (0 .. 100),

- recordingLevel	 [2] IMPLICIT INTEGER (0 .. 100)	},

- message		MessageID			OPTIONAL,

- extensions		CSTACommonArguments	OPTIONAL}

- SetVoiceAttributeResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-set-voice-attribute

## 15.10	Stop

- CSTA-stop

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) stop( 508) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- stop		OPERATION

- ARGUMENT	StopArgument

- RESULT	StopResult

- ERRORS	{universalFailure}

- ::= 508

- StopArgument ::=

- SEQUENCE

- {connection	ConnectionID,

- mesage	MessageID			OPTIONAL,

- extensions	CSTACommonArguments	OPTIONAL}

- StopResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-stop

## 15.11	Suspend

- CSTA-suspend

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) suspend( 509) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- suspend		OPERATION

- ARGUMENT	SuspendArgument

- RESULT	SuspendResult

- ERRORS	{universalFailure}

- ::= 509

- SuspendArgument ::=

- SEQUENCE

- {connection	ConnectionID,

- mesage	MessageID			OPTIONAL,

- extensions	CSTACommonArguments	OPTIONAL}

- SuspendResult ::=

- CHOICE

- {extensions	CSTACommonArguments,

- noData		NULL}

- END  -- of CSTA-suspend

## 15.12	Synthesize message

- CSTA-synthesize-message

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) synthesize-message( 510) }

- DEFINITIONS ::=

- BEGIN

- IMPORTS

- OPERATION, ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- -- Data Types --

- universalFailure FROM CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- ControlData, MessageID FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- CSTACommonArguments FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- synthesizeMessage	OPERATION

- ARGUMENT	SynthesizeMessageArgument

- RESULT	SynthesizeMessageResult

- ERRORS	{universalFailure}

- ::= 510

- SynthesizeMessageArgument ::=

- SEQUENCE

- {textToBeSynthesized		OCTET STRING,

- control				ControlData			OPTIONAL,

- extensions			CSTACommonArguments	OPTIONAL}

- SynthesizeMessageResult ::=

- CHOICE

- {synthesizedMessage		MessageID,

- SEQUENCE

- {synthesizedMessage	MessageID,

- extensions		CSTACommonArguments} }

- END  -- of CSTA-synthesize-message

# 16	Switching function errors

- CSTA-error-definition

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) error-definition( 120) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- UniversalFailure, universalFailure;

- IMPORTS

- ERROR FROM Remote-Operations-Notation

- { joint-iso-ccitt( 2) remote-operations( 4) notation( 0) }

- CSTAPrivateData FROM CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) };

- universalFailure	ERROR

- PARAMETER	UniversalFailure

- ::= 1

- UniversalFailure ::= CHOICE

- {operationalErrors			[1] IMPLICIT	Operations,

- stateErrors				[2] IMPLICIT	StateIncompatibility,

- systemResourceErrors			[3] IMPLICIT	SystemResourceAvailability,

- subscribedResourceAvailabilityErrors	[4] IMPLICIT	SubscribedResourceAvailability,

- performanceErrors			[5] IMPLICIT	PerformanceManagement,

- securityErrors				[6] IMPLICIT	SecurityError,

- unspecifiedErrors			[7] IMPLICIT	NULL,

- nonStandardErrors			[8] IMPLICIT	CSTAPrivateData}

- Operations ::= ENUMERATED

- {	generic					(1),

- requestIncompatibleWithObject		(2),

- valueOutOfRange			(3),

- objectNotKnown			(4),

- invalidCallingDevice			(5),

- invalidCalledDevice			(6),

- invalidForwardingDestination		(7),

- privilegeViolationOnSpecifiedDevice	(8),

- privilegeViolationOnCalledDevice	(9),

- privilegeViolationOnCallingDevice	(10),

- invalidCSTACallIdentifier		(11),

- invalidCSTADeviceIdentifier		(12),

- invalidCSTAConnectionIdentifier		(13),

- invalidDestination			(14),

- invalidFeature				(15),

- invalidAllocationState			(16),

- invalidCrossRefID			(17),

- invalidObjectType			(18),

- securityViolation			(19),

- invalidCSTAApplicationCorrelator	(20),

- invalidAccountCode			(21),

- invalidAuthorisationCode		(22),

- requestIncompatibleWithCallingDevice	(23),

- requestIncompatibleWithCalledDevice	(24),

- invalidMessageIdentifier			(25),

- messageIdentifierRequired		(26),

- mediaIncompatible			(27)	}

- StateIncompatibility ::= ENUMERATED

- {	generic					(1),

- invalidObjectState			(2),

- invalidConnectionID			(3),

- noActiveCall				(4),

- noHeldCall				(5),

- noCallToClear				(6),

- noConnectionToClear			(7),

- noCallToAnswer			(8),

- noCallToComplete			(9),

- notAbleToPlay				(10),

- notAbleToResume			(11),

- endOfMessage				(12),

- beginingOfMessage			(13),

- messageSuspended			(14)	}

- SystemResourceAvailability ::= ENUMERATED

- {	generic					(1),

- serviceBusy				(2),

- resourceBusy				(3),

- resourceOutOfService			(4),

- networkBusy				(5),

- networkOutOfService			(6),

- overallMonitorLimitExceeded		(7),

- conferenceMemberLimitExceeded	(8)	}

- SubscribedResourceAvailability ::= ENUMERATED

- {	generic					(1),

- objectMonitorLimitExceeded		(2),

- trunkLimitExceeded			(3),

- outstandingRequestLimitExceeded	(4)	}

- PerformanceManagement ::= ENUMERATED

- {	generic					(1),

- performanceLimitExceeded		(2)	}

- SecurityError ::= ENUMERATED

- {	unspecified				(0),

- sequenceNumberViolated		(1),

- timeStampViolated			(2),

- pACViolated				(3),

- sealViolated				(4)	}

- END  -- of CSTA-error-definition

# 17	Switching event cause values

- CSTA-event-causes

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) event-causes( 121) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- EventCause;

- EventCause ::= ENUMERATED

- -- a general list of cause codes

- --					Present in	Added in

- -- 					Version 1	Version2

- {	activeMonitor			(1),

- alternate			(2),

- blocked						(35),

- busy				(3),

- callBack			(4),

- callCancelled			(5),

- callForwardImmediate		(6),

- callForwardBusy		(7),

- callForwardNoAnswer		(8),

- callForward			(9),

- callNotAnswered		(10),

- callPickup			(11),

- campOn			(12),

- characterCountReached			(36),

- consultation					(37),

- destNotObtainable		(13),

- distributed					(38),

- doNotDisturb			(14),

- dTMFToneDetected				(39),

- durationExceeded				(40),

- endOfDataDetected				(41),

- enteringDistribution				(42),

- forcedPause					(43),

- incompatibleDestination		(15),

- invalidAccountCode		(16),

- keyOperation			(17),

- lockout				(18),

- maintenance			(19),

- makeCall					(44),

- messageSizeExceeded				(45),

- networkCongestion		(20),

- networkNotObtainable		(21),

- networkSignal					(46),

- newCall				(22),

- nextMessage					(47),

- noAvailableAgents		(23),

- normalClearing					(48),

- noSpeechDetected				(49),

- numberChanged				(50),

- override			(24),

- park				(25),

- overflow			(26),

- recall				(27),

- redirected			(28),

- reorderTone			(29),

- resourcesNotAvailable		(30),

- silentMonitor			(31),

- singleStepConference				(51),

- singleStepTransfer				(52),

- speechDetected				(53),

- switchTerminated				(54),

- terminationCharacterReceived			(55),

- timeout						(56),

- transfer				(32),

- trunksBusy			(33),

- voiceUnitInitiator		(34) }

- END  -- of event-cause-definitions

# 18	CSTA data types

- The major parameters have been assigned distinct application tags to facilitate parsing. The data is defined in logical groups in ascending order of application tag. Application tags used are:

- APPLICATION  1 - 5 :	Device identifiers

- APPLICATION  11 - 14:	Connection identifiers and local connection states

- APPLICATION  21 - 24 :	Status reporting

- APPLICATION  30 :	CSTACommonArguments

## 18.1	Switching function objects

- CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) switching-function-objects( 122) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- CSTAObject;

- IMPORTS

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM  CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) };

- CSTAObject ::= CHOICE

- {device		DeviceID,

- call		ConnectionID}

- END  -- of CSTA-switching-function-objects

## 18.2	Device identifiers

- CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- DeviceID, NumberDigits, ExtendedDeviceID, CallingDeviceID, CalledDeviceID, SubjectDeviceID, RedirectionDeviceID;

- DeviceID ::= CHOICE

- {dialingNumber		[0] IMPLICIT	NumberDigits,

- deviceNumber		[1] IMPLICIT	DeviceNumber}

- -- NumberDigits is a string of digits that represents a number (address) that

- -- the switch can route a call on. It can be dialled by a user  (i.e from a telephone

- -- keypad) to make a call.  DeviceNumber is not a routing address and cannot be dialed

- -- from a keypad. It is allocated by the switch to reference a device.

- NumberDigits	::= IA5String

- DeviceNumber	::= INTEGER

- ExtendedDeviceID ::= CHOICE

- {deviceIdentifier		DeviceID,

- implicitPublic		[2] IMPLICIT	NumberDigits,

- explicitPublic		[3]		PublicTON,

- implicitPrivate		[4] IMPLICIT	NumberDigits,

- explicitPrivate		[5]		PrivateTON,

- other			[6] IMPLICIT	OtherPlan}

- CallingDeviceID	::= [APPLICATION 1] CHOICE

- {deviceIdentifier		ExtendedDeviceID,

- notKnown		[7] IMPLICIT	NULL,

- notRequired		[8] IMPLICIT	NULL}

- CalledDeviceID ::= [APPLICATION 2] CHOICE

- {deviceIdentifier		ExtendedDeviceID,

- notKnown		[7] IMPLICIT	NULL,

- notRequired		[8] IMPLICIT	NULL}

- SubjectDeviceID ::= [APPLICATION 3] CHOICE

- {deviceIdentifier		ExtendedDeviceID,

- notKnown		[7] IMPLICIT	NULL,

- notRequired		[8] IMPLICIT	NULL}

- RedirectionDeviceID ::= [APPLICATION 4] CHOICE

- {numberdialed		ExtendedDeviceID,

- notKnown		[7] IMPLICIT	NULL,

- notRequired		[8] IMPLICIT	NULL}

- -- SubjectDeviceID is used in some event reports to specify which device the report

- -- refers to. If the SubjectDeviceID has had a monitor invoked then this data is not

- -- required and so the implicit NULL encoding for notRequired is returned.

- -- RedirectionDeviceID is used in Events as the lastRedirectionDevice.

- -- CalledDeviceID is used in Events to specify the number dialled.

- PublicTON ::= CHOICE

- {	unknown		[0] IMPLICIT	IA5String,

- international		[1] IMPLICIT	IA5String,

- national			[2] IMPLICIT	IA5String,

- networkspecific		[3] IMPLICIT	IA5String,

- subscriber		[4] IMPLICIT	IA5String,

- abbreviated		[5] IMPLICIT	IA5String	}

- -- The public type of numbers  are derived from CCITT E.164

- PrivateTON ::= CHOICE

- {	unknown		[0] IMPLICIT	IA5String,

- level3RegionalNumber	[1] IMPLICIT	IA5String,

- level2RegionalNumber	[2] IMPLICIT	IA5String,

- level1RegionalNumber	[3] IMPLICIT	IA5String,

- pTNSpecificNumber	[4] IMPLICIT	IA5String,

- localNumber		[5] IMPLICIT	IA5String,

- abbreviated		[6] IMPLICIT	IA5String	}

- -- The private type of numbers are derived from ECMA-155

- OtherPlan ::= OCTET STRING	-- Allows future expansion to cover other numbering

- -- plans (such as X.121 etc.)

- END  -- of CSTA-device-identifiers

## 18.3	Call and connection identifiers

- CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- ConnectionID;

- IMPORTS

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- ConnectionID ::= [APPLICATION 11] IMPLICIT SEQUENCE

- {call		[2] IMPLICIT	OCTET STRING	OPTIONAL,

- device		CHOICE

- {staticID	DeviceID,

- dynamicID	[3] IMPLICIT	OCTET STRING}	OPTIONAL}

- END  -- of CSTA-call-connection-identifiers

## 18.4	Connection states

- CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- ConnectionList, LocalConnectionState;

- IMPORTS

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- DeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- ConnectionList ::= SEQUENCE OF

- CHOICE

- {	newConnectionOnly		[0] ConnectionID,

- deviceIDOnly			[1] DeviceID,

- oldConnectionOnly		[2] ConnectionID,

- newConnectionAndDeviceID	[3] SEQUENCE

- {newConnection	ConnectionID,

- deviceID		DeviceID	},

- newAndOldConnections		[4] SEQUENCE

- {newConnection	ConnectionID,

- oldConnection		ConnectionID	},

- deviceIDAndOldConnection	[5] SEQUENCE

- {deviceID		DeviceID,

- oldConnection		ConnectionID	},

- allInformation			[6] SEQUENCE

- {newConnection	ConnectionID,

- deviceID		DeviceID,

- oldConnection		ConnectionID	}

- }

- LocalConnectionState ::= [APPLICATION 14] IMPLICIT ENUMERATED

- {	null	(0),

- initiate	(1),

- alerting	(2),

- connect	(3),

- hold	(4),

- queued	(5),

- fail	(6)	}

- END  -- CSTA-connection-states

## 18.5	Status reporting

- CSTA-status-reporting

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) status-reporting( 126) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- MonitorObject, MonitorCrossRefID, MonitorFilter, MonitorType, SnapshotCallData,

- SnapshotDeviceData;

- IMPORTS

- CSTAObject FROM CSTA-switching-function-objects

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) switching-function-objects( 122) }

- DeviceID, SubjectDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 012)

- standard( 0) csta2( 218) device-identifiers( 123) }

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- LocalConnectionState FROM CSTA-connection-states

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) connection-states( 125) }

- DeviceClass, DeviceProfile FROM CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) };

- MonitorObject ::= CSTAObject

- MonitorCrossRefID ::= [APPLICATION 21] IMPLICIT OCTET STRING

- MonitorFilter ::= SEQUENCE	-- default is no filter (i.e. all events)

- {	call		[0] IMPLICIT CallFilter		DEFAULT {},

- feature		[1] IMPLICIT FeatureFilter	DEFAULT {},

- agent		[2] IMPLICIT AgentFilter		DEFAULT {},

- maintenance	[3] IMPLICIT MaintenanceFilter	DEFAULT {},

- voiceUnit	[5] IMPLICIT VoiceUnitFilter	DEFAULT {},

- private		[4] IMPLICIT BOOLEAN	DEFAULT FALSE	}

- -- setting the relevant bit requests the filter for the appropriate events

- CallFilter ::= BIT STRING

- {	callCleared		(0),

- conferenced		(1),

- connectionCleared	(2),

- delivered		(3),

- diverted			(4),

- established		(5),

- failed			(6),

- held			(7),

- networkReached	(8),

- originated		(9),

- queued			(10),

- retrieved		(11),

- serviceInitiated		(12),

- transferred		(13)	}

- FeatureFilter ::= BIT STRING

- {	callInformation		(0),

- doNotDisturb		(1),

- forwarding		(2),

- messageWaiting	(3),

- autoAnswer		(4),

- microphoneMute	(5),

- speakerMute		(6),

- speakerVolume		(7)	}

- AgentFilter ::= BIT STRING

- {	loggedOn		(0),

- loggedOff		(1),

- notReady		(2),

- ready			(3),

- workingAfterCall	(4),

- busy			(5)	}

- MaintenanceFilter ::= BIT STRING

- {	backInService		(0),

- outOfService		(1)	}

- VoiceUnitFilter ::= BIT STRING

- {	stop			(0),

- play			(1),

- suspendPlay		(2),

- record			(3),

- suspendRecord		(4),

- review			(5),

- voiceAttributesChange	(6)	}

- MonitorType ::= ENUMERATED

- {	call			(0),

- device			(1)	}

- SnapshotDeviceData ::= [APPLICATION 22] IMPLICIT SEQUENCE OF
SnapshotDeviceResponseInfo

- SnapshotDeviceResponseInfo ::= SEQUENCE

- {	callIdentifier		ConnectionID,

- localCallState		CallState,

- deviceProfile		DeviceProfile		OPTIONAL	}

- SnapshotCallData ::= [APPLICATION 23] IMPLICIT SEQUENCE OF SnapshotCallResponseInfo

- SnapshotCallResponseInfo ::= SEQUENCE

- {	deviceOnCall		SubjectDeviceID,

- callIdentifier		ConnectionID,

- callClass		DeviceProfile		OPTIONAL,

- localConnectionState	LocalConnectionState	OPTIONAL	}

- CallState ::= CHOICE

- {	compound	[0] IMPLICIT CompoundCallState,

- simple		[1] IMPLICIT SimpleCallState,

- unknown	[2] IMPLICIT NULL			}

- -- unknown is returned by server if no other CallState can be supplied

- CompoundCallState ::= SEQUENCE OF LocalConnectionState

- SimpleCallState ::= ENUMERATED

- {	callNull			(0),	-- ‘00’H	- null-null

- callPending		(1),	-- ‘01’H	- null-initiate

- callOriginated		(3),	-- ‘03’H	- null-connect

- callDelivered		(35),	-- ‘23’H	- alerting-connect

- callDeliveredHeld	(36),	-- ‘24’H	- alerting-held

- callReceived		(50),	-- ‘32’H	- connect-alerting

- callEstablished		(51),	-- ‘33’H	- connect-connect

- callEstablishedHeld	(52),	-- ‘34’H	- connected-held

- callReceivedOnHold	(66),	-- ‘42’H	- held-alerting

- callEstablishedOnHold	(67),	-- ‘43’H	- held-connect

- callQueued		(83),	-- ‘53’H	- queued-connect

- callQueuedHeld		(84),	-- ‘54’H	- queued-held

- callFailed		(99),	-- ‘63’H	- failed-connect

- callFailedHeld		(100)}	-- ‘64’H	- failed-held

- -- This represents the main call states in a simplified encoding.  The semantics

- -- are identical to the sequence of connection states but they are represented by

- -- an item from an enumerated list.

- END  -- of CSTA-status-reporting

## 18.6	Device and feature types and other parameters

- CSTA-device-feature-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-feature-types( 127) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- AccountInfo, AgentID, AgentGroup, AgentPassword, AgentParameter, AgentState, AllocationState, AuthCode, ConnectionDetails, CorrelatorData, DeviceClass, DeviceInfo, DeviceProfile , DeviceType, DivertInfo, FeatureInfo, ListForwardParameters, LoggedOnInfo, LoggedOffInfo, ForwardParameter, ForwardingType, MessageID, MessageIDList, NoOfCallsInQueue, NoOfCallsInFront, ParticipationType, QueryDeviceFeature, QueryDeviceInformation, ReserveConnection, SetDeviceFeature, SystemStatus, SelectValue, PriorityValue, SetupValues, RetryValue, RouteUsedFlag, DomainValue, RoutingCrossRefID, IOCrossRefID, IOCrossRefIDList, IOData, Speed, DataPathDirection, DataPathType, Period, TerminatingConditions, AttributeInfo, ControlData;

- IMPORTS

- ConnectionID FROM CSTA-call-connection-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) call-connection-identifiers( 124) }

- DeviceID, NumberDigits, CalledDeviceID FROM CSTA-device-identifiers

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) device-identifiers( 123) };

- AccountInfo ::= OCTET STRING	-- used in CallInformation event

- AgentID ::= OCTET STRING

- AgentGroup ::= DeviceID

- AgentPassword ::= OCTET STRING

- AgentParameter ::= CHOICE

- {	agentLoggedIn		[0] IMPLICIT LoggedOnInfo,

- agentLoggedOut	[1] IMPLICIT LoggedOffInfo,

- agentNotReady		[2] IMPLICIT NULL,

- agentReady		[3] IMPLICIT NULL,

- agentBusy		[4] IMPLICIT NULL,

- agentWorkingAfterCall	[5] IMPLICIT NULL		}

- AgentState ::= ENUMERATED

- {	agentNotReady			(0),

- agentNull			(1),

- agentReady			(2),

- agentBusy			(3),

- agentWorkingAfterCall		(4)	}

- AllocationState	::= ENUMERATED

- {	callDelivered	(0),

- callEstablished	(1)	}

- -- used in MakePredictiveCall to indicate when call should be allocated to the device

- AuthCode ::= OCTET STRING	-- used in Information Events

- ConnectionDetails ::= CHOICE

- {	heldCall			[0] IMPLICIT ConnectionID,

- activeCall			[1] IMPLICIT ConnectionID,

- bothCalls			[2] IMPLICIT SEQUENCE

- {heldCall	ConnectionID,

- activeCall	ConnectionID}	}

- CorrelatorData ::= OCTET STRING

- DeviceClass ::= BIT STRING

- {	voice	(0),

- data	(1),

- image	(2),

- audio	(4),

- other	(3)	}

- DeviceInfo ::= SEQUENCE

- {	deviceID	DeviceID	OPTIONAL,

- deviceType	DeviceType,

- deviceClass	DeviceClass			}

- DeviceProfile ::= SEQUENCE

- {	deviceClass	DeviceClass,

- iSDNSetup	SetupValues	OPTIONAL	}

- SetupValues ::= OCTET STRING

- --	When SetupValues is required, it may include:

- --		Bearer Capability

- --		Calling Party Subaddress (to enable callers specific TE to be selected)

- --		Called Party Subaddress

- --		High Layer Compatibility

- --		Low Layer Compatibility

- --	Where all the above information elements are defined in ISO/IEC 11572: 1993

- DeviceType ::= ENUMERATED

- {	station			(0),

- line			(1),

- button			(2),

- aCD			(3),

- trunk			(4),

- operator		(5),

- other			(6),

- conferenceBridge	(7),

- station-group		(16),

- line-group		(17),

- button-group		(18),

- aCD-group		(19),

- trunk-group		(20),

- operator-group		(21),

- parkingDevice		(22),

- other-group		(255)	}

- DivertInfo ::= CHOICE	-- used by Divert Call service

- {	deflect			[0] IMPLICIT SEQUENCE

- {callToBeDiverted	ConnectionID,

- newDestination		CalledDeviceID	},

- pickup			[1] IMPLICIT SEQUENCE

- {callToBePickedUp	ConnectionID,

- requestingDevice	DeviceID	},

- groupRequestingDevice	[2] DeviceID	}

- FeatureInfo ::= CHOICE	-- used by Call Completion service

- {	campon			[0] IMPLICIT	ConnectionID,

- callback		[1] IMPLICIT	ConnectionID,

- intrude			[2] IMPLICIT	ConnectionID,

- callbackMessage	[3] IMPLICIT	ConnectionID	}

- ListForwardParameters  ::= SEQUENCE OF SEQUENCE

- {	forwardingType		ForwardingType,

- forwardDN		NumberDigits	}

- LoggedOnInfo ::= SEQUENCE

- {	agentID		[10] IMPLICIT	AgentID	OPTIONAL,

- password	[11] IMPLICIT	AgentPassword	OPTIONAL,

- group		AgentGroup			OPTIONAL	}

- LoggedOffInfo	::= SEQUENCE

- {	agentID	 	[10] IMPLICIT	AgentID	OPTIONAL,

- password	[11] IMPLICIT	AgentPassword	OPTIONAL,

- group		AgentGroup			OPTIONAL	}

- ForwardParameter ::= SEQUENCE

- {	forwardingType		ForwardingType,

- forwardDN		NumberDigits		OPTIONAL	}

- ForwardingType	::= ENUMERATED

- {	forwardImmediateOn	(0),

- forwardImmediateOff	(1),

- forwardBusyOn		(2),

- forwardBusyOff		(3),

- forwardNoAnsOn	(4),

- forwardNoAnsOff	(5),

- forwardBusyIntOn	(6),

- forwardBusyIntOff	(7),

- forwardBusyExtOn	(8),

- forwardBusyExtOff	(9),

- forwardNoAnsIntOn	(10),

- forwardNoAnsIntOff	(11),

- forwardNoAnsExtOn	(12),

- forwardNoAnsExtOff	(13),

- forwardImmIntOn	(14),

- forwardImmIntOff	(15),

- forwardImmExtOn	(16),

- forwardImmExtOff	(17)	}

- IOCrossRefID ::= CHOICE

- {	switchProvided		[0] OCTET STRING,

- computerProvided	[1] OCTET STRING	}

- IOCrossRefIDList ::= SEQUENCE OF IOCrossRefID

- IOData ::= OCTET STRING

- MessageID ::= OCTET STRING

- MessageIDList ::= SEQUENCE OF MessageID

- NoOfCallsInQueue	::= INTEGER		-- used in Call Queued Event

- NoOfCallsInFront	::= INTEGER		-- used in Call Queued Event

- ParticipationType ::= ENUMERATED

- {	silent			(0),

- active			(1)	}

- QueryDeviceFeature ::= ENUMERATED	-- used by Query Feature service request

- {	msgWaitingOn		(0),

- doNotDisturbOn 	(1),

- forward			(2),

- deviceInfo		(4),

- agentState		(5),

- routingEnabled		(6),

- autoAnswer		(7),

- microphoneMuteOn	(8),

- speakerMuteOn		(9),

- speakerVolume		(10)	}

- QueryDeviceInformation ::= CHOICE

- {	msgWaitingOn		[0] IMPLICIT BOOLEAN,

- doNotDisturbOn		[1] IMPLICIT BOOLEAN,

- forward			[2] IMPLICIT ListForwardParameters,

- deviceInfo		[4] IMPLICIT DeviceInfo,

- agentState		[5] IMPLICIT AgentState,

- routingEnabled		[6] IMPLICIT BOOLEAN,

- autoAnswerOn		[7] IMPLICIT BOOLEAN,

- microphoneMuteOn	[8] IMPLICIT BOOLEAN,

- speakerMuteOn		[9] IMPLICIT BOOLEAN,

- speakerVolume		[10] IMPLICIT INTEGER (0 .. 100)	}

- ReserveConnection ::= BOOLEAN	-- used with Hold service to reserve ISDN connection

- SetDeviceFeature ::= CHOICE	-- used by SetFeature service request

- {	msgWaitingOn		[0] IMPLICIT BOOLEAN,

- doNotDisturbOn		[1] IMPLICIT BOOLEAN,

- forward			[2] IMPLICIT ForwardParameter,

- requestedAgentState	[3] AgentParameter,

- enableRouting		[4] IMPLICIT BOOLEAN,

- autoAnswerOn		[5] IMPLICIT BOOLEAN,

- microphoneMuteOn	[6] IMPLICIT BOOLEAN,

- speakerMuteOn		[7] IMPLICIT BOOLEAN,

- speakerVolume		[8] IMPLICIT INTEGER (0 .. 100)		}

- SystemStatus ::= ENUMERATED

- {	initializing		(0),

- enabled			(1),

- normal			(2),

- messagesLost		(3),

- disabled		(4),

- overloadImminent	(5),

- overloadReached	(6),

- overloadRelieved	(7)	}

- SelectValue ::= ENUMERATED

- {	normal		(0),

- leastCost	(1),

- emergency	(2),

- aCD		(3),

- userDefined	(4)	}

- PriorityValue ::= BOOLEAN		-- TRUE means priority call

- RetryValue ::= CHOICE	-- used in RouteSelect Request service

- {	noListAvailable		[0] IMPLICIT BOOLEAN,

- noCountAvailable	[1] IMPLICIT BOOLEAN,

- retryCount		[2] IMPLICIT INTEGER		}

- RouteUsedFlag ::= BOOLEAN	-- TRUE means RouteUsed Request service

- -- requested

- DomainValue ::= BOOLEAN	-- TRUE means CSTA subdomain destination

- RoutingCrossRefID ::= [APPLICATION 24] IMPLICIT OCTET STRING

- Speed ::= INTEGER (1 .. 100)

- DataPathDirection ::= ENUMERATED

- {	fromRequestor		(0),

- toRequestor		(1),

- biDirectional		(2)	}

- DataPathType ::= ENUMERATED

- {	text			(0),

- digitalVoice		(1)	}

- Period ::= CHOICE

- {	relativePosition [0] ENUMERATED

- {	beginingOfMessage		(0),

- endOfMessage			(1)	},

- position [1] INTEGER	}

- TerminatingConditions ::= ENUMERATED

- {	durationExceeded	(0),

- dTMFDetected		(1),

- endOfDataDetected	(2),

- speechDetected	(3)	}

- AttributeInfo ::= CHOICE

- {	encodingAlgorithm	[0] ENUMERATED

- {aDPCM6K		(0),

- aDPCM8K		(1),

- muLawPCM6K		(2),

- aLawPCM6K		(3)	},

- samplingRate		REAL,

- durations		[1] INTEGER,

- fileName		OCTET STRING,

- position			[2] INTEGER,

- speed			[3] Speed,

- volume			[4] INTEGER (1 .. 100),

- level			[5] INTEGER (1 .. 100),

- state			[6] ENUMERATED

- {stop			(0),

- play			(1),

- record			(2),

- suspendPlay		(3),

- suspendRecord	(4),

- review			(5)	}

- }

- ControlData ::= SEQUENCE

- {	sex		ENUMERATED

- {male		(0),

- female		(1)	},

- language	OCTET STRING

- }

- END  -- of CSTA-device-feature-types

## 18.7	Security service

- CSTA-security

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) security( 128) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- CSTASecurityData;

- IMPORTS

- PrivilegeAttributeCertificate, UTCTime FROM Security-Information

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) desd( 138) securityData( 1) };

- Seal ::= SEQUENCE

- {	algorithmIdentifier	OBJECT IDENTIFIER,

- keyIdentifier		OCTET STRING,

- seal			OCTET STRING	}

- CSTASecurityData ::= SEQUENCE

- {	messageSequenceNumber	INTEGER			OPTIONAL,

- timeStamp			UTCTime			OPTIONAL,

- privilegeAttributeCertificate	PrivilegeAttributeCertificate	OPTIONAL,

- seal				Seal				OPTIONAL	}

- END  -- of CSTA-security

## 18.8	Common extensions

- CSTA-extension-types

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) extension-types( 129) }

- DEFINITIONS ::=

- BEGIN

- EXPORTS

- CSTACommonArguments, CSTAPrivateData;

- IMPORTS

- CSTASecurityData FROM CSTA-security

- { iso( 1) identified-organization( 3) icd-ecma( 12)

- standard( 0) csta2( 218) security( 128) };

- CSTACommonArguments ::= [APPLICATION 30] IMPLICIT SEQUENCE

- {	security		[0] IMPLICIT	CSTASecurityData			OPTIONAL,

- privateData	[1] IMPLICIT	SEQUENCE OF CSTAPrivateData	OPTIONAL }

- CSTAPrivateData ::= NULL	      --  The actual encoding of the private event is added here,

- --  replacing NULL with another valid ASN.1 type.

- END  -- of CSTA-extension-types

# 19	CSTA parameter size constraints

- Maximum length

- Parameter	(in bytes)

- call (part of ConnectionID)	8

- MonitorCrossRefID	4

- ApplicationCorrelator	32

- PublicTON	32

- PrivateTON	32

- NumberDigits	32

- DeviceNumber	4

- dynamicID (part of ConnectionID)	32

- AgentID	16

- AgentPassword	16

- AccountInfo	32

- AuthCode	16

- SetupValues	260

- RoutingCrossRefID	4

- Review Variable	4

- Period	4

- Speed	4

- ROSE Invoke ID	4

- Number Ahead In Queue	2

- Number In Queue	2

- IOCrossRefID	4

- IOData	implementation-specific

- MessageID	256

- Annex A

- (normative)

- Protocol Implementation Conformance Statement (PICS) Proforma

- Contents

- A.1 Introduction

- A.2 Definitions and abbreviations

- A.3 Conformance

- A.4 Instructions for completing the PICS proforma

- A.5 Implementation identification

- A.6 Switching function services

- A.6.1 Alternate call

- A.6.2 Answer call

- A.6.3 Associate data

- A.6.4 Call completion

- A.6.5 Clear call

- A.6.6 Clear connection

- A.6.7 Conference call

- A.6.8 Consultation call

- A.6.9 Divert call

- A.6.10 Hold call

- A.6.11 Make call

- A.6.12 Make predictive call

- A.6.13 Park call

- A.6.14 Query device

- A.6.15 Reconnect call

- A.6.16 Retrieve call

- A.6.17 Send DTMF tones

- A.6.18 Set feature

- A.6.19 Single step conference

- A.6.20 Single step transfer

- A.6.21 Transfer call

- A.7 Switching function events

- A.7.1 Event report service

- A.7.2 Call events

- A.7.2.1 Call cleared

- A.7.2.2 Conferenced

- A.7.2.3 Connection cleared

- A.7.2.4 Delivered

- A.7.2.5 Diverted

- A.7.2.6 Established

- A.7.2.7 Failed

- A.7.2.8 Held

- A.7.2.9 Network reached

- A.7.2.10 Originated

- A.7.2.11 Queued

- A.7.2.12 Retrieved

- A.7.2.13 Service initiated

- A.7.2.14 Transferred

- A.7.3 Agent events

- A.7.3.1 Agent logged on

- A.7.3.2 Agent logged off

- A.7.3.3 Agent ready

- A.7.3.4 Agent not ready

- A.7.3.5 Agent working after call

- A.7.3.6 Agent busy

- A.7.4 Other feature events

- A.7.4.1 Call information

- A.7.4.2 Do not disturb

- A.7.4.3 Forwarding

- A.7.4.4 Message waiting

- A.7.4.5 Auto answer

- A.7.4.6 Microphone mute

- A.7.4.7 Speaker mute

- A.7.4.8 Speaker volume

- A.7.5 Maintenance events

- A.7.5.1 Back in service

- A.7.5.2 Out of service

- A.7.6 Private events

- A.7.6.1 Private

- A.7.7 Voice unit events

- A.7.7.1 Play

- A.7.7.2 Record

- A.7.7.3 Review

- A.7.7.4 Stop

- A.7.7.5 Suspend play	174

- A.7.7.6 Suspend record

- A.7.7.7 Voice attribute change

- A.8 Computing function services

- A.8.1 Route request

- A.8.2 Reroute request

- A.8.3 Route select request

- A.8.4 Route used request

- A.8.5 Route end request

- A.9 Bidirectional services

- A.9.1 Escape

- A.9.2 System status

- A.10 Status reporting services

- A.10.1 Change monitor filter

- A.10.2 Monitor start

- A.10.3 Monitor stop

- A.10.4 Snapshot call

- A.10.5 Snapshot device

- A.11 Input/output services

- A.11.1 Data path resumed

- A.11.2 Data path suspended

- A.11.3 Fast data

- A.11.4 Resume data path

- A.11.5 Send broadcast data

- A.11.6 Send data

- A.11.7 Send multicast data

- A.11.8 Start data path

- A.11.9 Stop data path

- A.11.10 Suspend data path

- A.12 Voice unit services

- A.12.1 Concatenate message

- A.12.2 Delete message

- A.12.3 Play message

- A.12.4 Query voice attribute

- A.12.5 Record message

- A.12.6 Reposition

- A.12.7 Resume

- A.12.8 Review

- A.12.9 Set voice attribute

- A.12.10 Stop

- A.12.11 Suspend

- A.12.12 Synthesize message

- .

# A.1	Introduction

- The Protocol Implementation Conformance Statement (PICS) is a statement of which capabilities and options of the protocol have been implemented. The PICS can have a number of uses, including use:

- by the protocol implementor, as a check-list to reduce the risk of failure to conform to the standard through oversight;

- by the supplier and acquirer (or potential acquirer) of the implementation, as a detailed indication of the capabilities of the implementation, stated relative to the common basis for understanding provided by the standard PICS proforma;

- by the user (or potential user) of the implementation, as a basis for initially checking the possibility of interworking with another implementation (note that, while interworking cannot be guaranteed, failure to interwork can often be predicted from incompatible PICS);

- by a protocol tester, as the basis for selecting appropriate tests against which to assess the claim for conformance of the implementation.

# A.2	Definitions and abbreviations

- This Standard uses the following terms defined in ISO 9646-1:

- Protocol Implementation Conformance Statement (PICS);

- PICS Proforma.

In the "Reference" columns of the body of the PICS proforma, the letter S refers to the CSTA Phase II Services Standard ECMA-217 and the letter P refers to the CSTA Phase II Protocol Standard ECMA-218.

The following terms are used in the "Status" columns of the body of the PICS proforma:

m	mandatory; the capability is required for conformance to the protocol.

o	optional; the capability is not required for conformance to the protocol, or is required only within constraints described in dependencies ("if" statements). If the capability is implemented, it is required to conform to the protocol specifications.

o.<n>	optional, but support of at least one of the group of options labelled by the same numeral <n> is required.

C.<cid>	conditional; the requirement is conditional according to the condition identified by <cid>.

<item>	simple-predicate condition, dependent on the support marked for <item>.

# A.3	Conformance

The supplier of a protocol implementation which is claimed to conform to ECMA-218 shall complete a copy of the Protocol Implementation Conformance Statement (PICS) proforma in clauses A.5 to A.13.

# A.4	Instructions for completing the PICS proforma

The first part of the PICS proforma, the Implementation Identification (clause A.5), is to be completed as indicated with the information necessary to identify fully both the supplier and the implementation.

The main part of the PICS proforma (clauses A.6 to A.13) is a fixed format questionnaire divided into subclauses each containing a group of individual items. Answers to the questionnaire items are to be provided in the rightmost column, either by marking an answer to indicate a restricted choice (usually Yes or No), or by checking off all supported values (for parameters with a default).

Each item is identified by an item reference in the first column; the second column title indicates the nature of the table items which follow. The third column contains the references to material that specifies the item in the main body of ECMA-217 and ECMA-218. The remaining columns record the status of the item - whether support is mandatory, optional, or not applicable - and provide space for the answers.

Where a service is not supported, any parameters or dependent service components are not applicable. These dependencies are indicated in the status column using the item identifier as a key.  It is not necessary to complete items in any subsidiary sections if a "No" response is given to the primary service component.

For supported services, a negative response to a mandatory subsidiary item indicates that the service does not conform to ECMA-218, and conformance cannot be claimed for that service.

Figure A.1 below is offered as an informative clarifying example of the first of the PICS proforma tables, that given in A.6.1 for the Alternate call service.

Figure A.1 - Clarifying diagram (informative)

# A.5	Implementation identification

| Supplier |  |
| --- | --- |
| Protocol Version | First Edition |
| Date of Statement |  |
| Contact point for queries about the PICS |  |
| Implementation Name(s) and Version(s) |  |
| Other information necessary for full identification - e.g. Name(s) and Version(s) for machines and/or operating systems; system name(s) |  |

The first five items are required for all implementations; other information may be completed as appropriate in meeting the requirement for full identification.

The terms Name and Version should be interpreted appropriately to correspond with a supplier's terminology (e.g. Type, Series, Model).

# A.6	Switching function services

## A.6.1	Alternate call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A1 | Alternate call service | S10.1 P9.1 | o |  | Yes [ ] No [ ] |
| A1a | Active call | P18.6 | A1:o1 |  | Yes [ ] No [ ] |
| A1b | Held call | P18.6 | A1:o1 |  | Yes [ ] No [ ] |
| A1c | Private data in request | P18.8 | A1:o |  | Yes [ ] No [ ] |
| A1d | Security data in request | P18.7 | A1:o |  | Yes [ ] No [ ] |
| A1e | Service result positive | P9.1 | A1:m | [ ] | Yes [ ] |
| A1f | Service result negative | P16 | A1:m | [ ] | Yes [ ] |
| A1g | Private data in result | P18.8 | A1e:o |  | Yes [ ] No [ ] |
| A1h | Security data in result | P18.7 | A1e:o |  | Yes [ ] No [ ] |

## A.6.2	Answer call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A2 | Answer call service | S10.2 P9.2 | o |  | Yes [ ] No [ ] |
| A2a | Call to be answered | P9.2 | A2:m | [ ] | Yes [ ] |
| A2b | Private data in request | P18.8 | A2:o |  | Yes [ ] No [ ] |
| A2c | Security data in request | P18.7 | A2:o |  | Yes [ ] No [ ] |
| A2d | Service result positive | P9.2 | A2:m | [ ] | Yes [ ] |
| A2e | Service result negative | P16 | A2:m | [ ] | Yes [ ] |
| A2f | Private data in result | P18.8 | A2d:o |  | Yes [ ] No [ ] |
| A2g | Security data in result | P18.7 | A2d:o |  | Yes [ ] No [ ] |

## A.6.3	Associate data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A3 | Associate data service | S10.3 P9.3 | o |  | Yes [ ] No [ ] |
| A3a | Existing call | P9.3 | A3:m | [ ] | Yes [ ] |
| A3b | Account code | P9.3 | A3:o1 |  | Yes [ ] No [ ] |
| A3c | Authorization code | P9.3 | A3:o1 |  | Yes [ ] No [ ] |
| A3d | Correlator data | P9.3 | A3:o1 |  | Yes [ ] No [ ] |
| A3e | Private data in request | P18.8 | A3:o |  | Yes [ ] No [ ] |
| A3f | Security data in request | P18.7 | A3:o |  | Yes [ ] No [ ] |
| A3g | Service result positive | P9.3 | A3:m | [ ] | Yes [ ] |
| A3h | Service result negative | P16 | A3:m | [ ] | Yes [ ] |
| A3i | Private data in result | P18.8 | A3g:o |  | Yes [ ] No [ ] |
| A3j | Security data in result | P18.7 | A3g:o |  | Yes [ ] No [ ] |

## A.6.4	Call completion

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A4 | Call completion service | S10.4 P9.4 | o |  | Yes [ ] No [ ] |
| A4a | Camp on caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4b | Call back caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4c | Intrude caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4d | Call back message caller | P18.6 | A4:o1 |  | Yes [ ] No [ ] |
| A4e | Private data in request | P18.8 | A4:o |  | Yes [ ] No [ ] |
| A4f | Security data in request | P18.7 | A4:o |  | Yes [ ] No [ ] |
| A4g | Service result positive | P9.4 | A4:m | [ ] | Yes [ ] |
| A4h | Service result negative | P16 | A4:m | [ ] | Yes [ ] |
| A4i | Private data in result | P18.8 | A4g:o |  | Yes [ ] No [ ] |
| A4j | Security data in result | P18.7 | A4g:o |  | Yes [ ] No [ ] |

## A.6.5	Clear call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A5 | Clear call service | S10.5 P9.5 | o |  | Yes [ ] No [ ] |
| A5a | Call to be cleared | P9.5 | A5:m | [ ] | Yes [ ] |
| A5b | Private data in request | P18.8 | A5:o |  | Yes [ ] No [ ] |
| A5c | Security data in request | P18.7 | A5:o |  | Yes [ ] No [ ] |
| A5d | Service result positive | P9.5 | A5:m | [ ] | Yes [ ] |
| A5e | Service result negative | P16 | A5:m | [ ] | Yes [ ] |
| A5f | Private data in result | P18.8 | A5d:o |  | Yes [ ] No [ ] |
| A5g | Security data in result | P18.7 | A5d:o |  | Yes [ ] No [ ] |

## A.6.6	Clear connection

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A6 | Clear connection service | S10.6 P9.6 | o |  | Yes [ ] No [ ] |
| A6a | Connection to be cleared | P9.6 | A6:m | [ ] | Yes [ ] |
| A6b | Private data in request | P18.8 | A6:o |  | Yes [ ] No [ ] |
| A6c | Security data in request | P18.7 | A6:o |  | Yes [ ] No [ ] |
| A6d | Service result positive | P9.6 | A6:m | [ ] | Yes [ ] |
| A6e | Service result negative | P16 | A6:m | [ ] | Yes [ ] |
| A6f | Private data in result | P18.8 | A6d:o |  | Yes [ ] No [ ] |
| A6g | Security data in result | P18.7 | A6d:o |  | Yes [ ] No [ ] |

## A.6.7	Conference call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A7 | Conference call service | S10.7 P9.7 | o |  | Yes [ ] No [ ] |
| A7a | Held call | P18.6 | A7:o1 |  | Yes [ ] No [ ] |
| A7b | Active call | P18.6 | A7:o1 |  | Yes [ ] No [ ] |
| A7c | Private data in request | P18.8 | A7:o |  | Yes [ ] No [ ] |
| A7d | Security data in request | P18.7 | A7:o |  | Yes [ ] No [ ] |
| A7e | Service result positive | P9.7 | A7:m | [ ] | Yes [ ] |
| A7f | Service result negative | P16 | A7:m | [ ] | Yes [ ] |
| A7g | Conference call | P9.7 | A7e:m | [ ] | Yes [ ] |
| A7h | Connections | P18.4 | A7e:o |  | Yes [ ] No [ ] |
| A7i | New connection | P18.4 | A7h:o2 |  | Yes [ ] No [ ] |
| A7j | Device ID | P18.4 | A7h:o2 |  | Yes [ ] No [ ] |
| A7k | Old connection | P18.4 | A7h:o2 |  | Yes [ ] No [ ] |
| A7l | Private data in result | P18.8 | A7e:o |  | Yes [ ] No [ ] |
| A7m | Security data in result | P18.7 | A7e:o |  | Yes [ ] No [ ] |

## A.6.8	Consultation call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A8 | Consultation call service | S10.8 P9.8 | o |  | Yes [ ] No [ ] |
| A8a | Existing call | P9.8 | A8:m | [ ] | Yes [ ] |
| A8b | Consulted device | P9.8 | A8:m | [ ] | Yes [ ] |
| A8c | Consulted device profile | P9.8 | A8:o |  | Yes [ ] No [ ] |
| A8d | Device class | P18.6 | A8c:m | [ ] | Yes [ ] |
| A8e | ISDN set up | P18.6 | A8c:o |  | Yes [ ] No [ ] |
| A8f | Account code | P9.8 | A8:o |  | Yes [ ] No [ ] |
| A8g | Authorization code | P9.8 | A8:o |  | Yes [ ] No [ ] |
| A8h | Correlator data | P9.8 | A8:o |  | Yes [ ] No [ ] |
| A8i | Private data in request | P18.8 | A8:o |  | Yes [ ] No [ ] |
| A8j | Security data in request | P18.7 | A8:o |  | Yes [ ] No [ ] |
| A8k | Service result positive | P9.8 | A8:m | [ ] | Yes [ ] |
| A8l | Service result negative | P16 | A8:m | [ ] | Yes [ ] |
| A8m | Initiated call | P9.8 | A8k:m | [ ] | Yes [ ] |
| A8n | Private data in result | P18.8 | A8k:o |  | Yes [ ] No [ ] |
| A8o | Security data in result | P18.7 | A8k:o |  | Yes [ ] No [ ] |

## A.6.9	Divert call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A9 | Divert call service | S10.9 P9.9 | o |  | Yes [ ] No [ ] |
| A9a | Deflect | P18.6 | A9:o1 |  | Yes [ ] No [ ] |
| A9b | Pickup | P18.6 | A9:o1 |  | Yes [ ] No [ ] |
| A9c | Group pickup | P18.6 | A9:o1 |  | Yes [ ] No [ ] |
| A9d | Correlator data | P9.9 | A9:o |  | Yes [ ] No [ ] |
| A9e | Private data in request | P18.8 | A9:o |  | Yes [ ] No [ ] |
| A9f | Security data in request | P18.7 | A9:o |  | Yes [ ] No [ ] |
| A9g | Service result positive | P9.9 | A9:m | [ ] | Yes [ ] |
| A9h | Service result negative | P16 | A9:m | [ ] | Yes [ ] |
| A9i | Private data in result | P18.8 | A9g:o |  | Yes [ ] No [ ] |
| A9j | Security data in result | P18.7 | A9g:o |  | Yes [ ] No [ ] |

## A.6.10	Hold call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A10 | Hold call service | S10.10 P9.10 | o |  | Yes [ ] No [ ] |
| A10a | Call to be held | P9.10 | A10:m | [ ] | Yes [ ] |
| A10b | Connection reservation | P9.10 | A10:o |  | Yes [ ] No [ ] |
| A10c | Private data in request | P18.8 | A10:o |  | Yes [ ] No [ ] |
| A10d | Security data in request | P18.7 | A10:o |  | Yes [ ] No [ ] |
| A10e | Service result positive | P9.10 | A10:m | [ ] | Yes [ ] |
| A10f | Service result negative | P16 | A10:m | [ ] | Yes [ ] |
| A10g | Private data in result | P18.8 | A10e:o |  | Yes [ ] No [ ] |
| A10h | Security data in result | P18.7 | A10e:o |  | Yes [ ] No [ ] |

## A.6.11	Make call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A11 | Make call service | S10.11 P9.11 | o |  | Yes [ ] No [ ] |
| A11a | Calling device | P9.11 | A11:m | [ ] | Yes [ ] |
| A11b | Called device | P9.11 | A11:m | [ ] | Yes [ ] |
| A11c | Device profile | P9.11 | A11:o |  | Yes [ ] No [ ] |
| A11d | Device class | P18.6 | A11c:m | [ ] | Yes [ ] |
| A11e | ISDN set up | P18.6 | A11c:o |  | Yes [ ] No [ ] |
| A11f | Account code | P9.11 | A11:o |  | Yes [ ] No [ ] |
| A11g | Authorization code | P9.11 | A11:o |  | Yes [ ] No [ ] |
| A11h | Correlator data | P9.11 | A11:o |  | Yes [ ] No [ ] |
| A11i | Private data in request | P18.8 | A11:o |  | Yes [ ] No [ ] |
| A11j | Security data in request | P18.7 | A11:o |  | Yes [ ] No [ ] |
| A11k | Service result positive | P9.11 | A11:m | [ ] | Yes [ ] |
| A11l | Service result negative | P16 | A11:m | [ ] | Yes [ ] |
| A11m | Initiated call | P9.11 | A11k:m | [ ] | Yes [ ] |
| A11n | Private data in result | P18.8 | A11k:o |  | Yes [ ] No [ ] |
| A11o | Security data in result | P18.7 | A11k:o |  | Yes [ ] No [ ] |

## A.6.12	Make predictive call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A12 | Make predictive call service | S10.12 P9.12 | o |  | Yes [ ] No [ ] |
| A12a | Calling device | P9.12 | A12:m | [ ] | Yes [ ] |
| A12b | Called device | P9.12 | A12:m | [ ] | Yes [ ] |
| A12c | Allocation | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12d | Device profile | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12e | Device class | P18.6 | A12d:m | [ ] | Yes [ ] |
| A12f | ISDN set up | P18.6 | A12d:o |  | Yes [ ] No [ ] |
| A12g | Account code | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12h | Authorization code | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12i | Correlator data | P9.12 | A12:o |  | Yes [ ] No [ ] |
| A12j | Private data in request | P18.8 | A12:o |  | Yes [ ] No [ ] |
| A12k | Security data in request | P18.7 | A12:o |  | Yes [ ] No [ ] |
| A12l | Service result positive | P9.12 | A12:m | [ ] | Yes [ ] |
| A12m | Service result negative | P16 | A12:m | [ ] | Yes [ ] |
| A12n | Initiated call | P9.12 | A12l:m | [ ] | Yes [ ] |
| A12o | Private data in result | P18.8 | A12l:o |  | Yes [ ] No [ ] |
| A12p | Security data in result | P18.7 | A12l:o |  | Yes [ ] No [ ] |

## A.6.13	Park call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A13 | Park call service | S10.13 P9.13 | o |  | Yes [ ] No [ ] |
| A13a | Call to park | P9.13 | A13:m | [ ] | Yes [ ] |
| A13b | Park to | P9.13 | A13:m | [ ] | Yes [ ] |
| A13c | Park device profile | P9.13 | A13:o |  | Yes [ ] No [ ] |
| A13d | Device class | P18.6 | A13c:m | [ ] | Yes [ ] |
| A13e | ISDN set up | P18.6 | A13c:o |  | Yes [ ] No [ ] |
| A13f | Correlator data | P9.13 | A13:o |  | Yes [ ] No [ ] |
| A13g | Private data in request | P18.8 | A13:o |  | Yes [ ] No [ ] |
| A13h | Security data in request | P18.7 | A13:o |  | Yes [ ] No [ ] |
| A13i | Service result positive | P9.13 | A13:m | [ ] | Yes [ ] |
| A13j | Service result negative | P16 | A13:m | [ ] | Yes [ ] |
| A13k | Private data in result | P18.8 | A13i:o |  | Yes [ ] No [ ] |
| A13l | Security data in result | P18.7 | A13i:o |  | Yes [ ] No [ ] |

## A.6.14	Query device

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A14 | Query device service | S10.14 P9.14 | o |  | Yes [ ] No [ ] |
| A14a | Device | P9.14 | A14:m | [ ] | Yes [ ] |
| A14b | Feature | P9.14 | A14:m | [ ] | Yes [ ] |
| A14c | Private data in request | P18.8 | A14:o |  | Yes [ ] No [ ] |
| A14d | Security data in request | P18.7 | A14:o |  | Yes [ ] No [ ] |
| A14e | Service result positive | P9.14 | A14:m | [ ] | Yes [ ] |
| A14f | Service result negative | P16 | A14:m | [ ] | Yes [ ] |
| A14g | Device information in service result | P9.14 | A14e:m | [ ] | Yes [ ] |
| A14h | Message waiting on | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14i | Do not disturb on | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14j | Forward | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14k | Forwarding type | P18.6 | A14j:m | [ ] | Yes [ ] |
| A14l | Forward to device | P18.6 | A14j:m | [ ] | Yes [ ] |
| A14m | Device information | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14n | Device ID | P18.6 | A14m:o |  | Yes [ ] No [ ] |
| A14o | Device type | P18.6 | A14m:m | [ ] | Yes [ ] |
| A14p | Device class | P18.6 | A14m:m | [ ] | Yes [ ] |
| A14q | Agent state | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14r | Routing enabled | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14s | Auto answer on | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14t | Microphone mute on | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14u | Speaker mute on | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14v | Speaker volume | P18.6 | A14g:o1 |  | Yes [ ] No [ ] |
| A14w | Private data in result | P18.8 | A14e:o |  | Yes [ ] No [ ] |
| A14x | Security data in result | P18.7 | A14e:o |  | Yes [ ] No [ ] |

## A.6.15	Reconnect call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A15 | Reconnect call service | S10.15 P9.15 | o |  | Yes [ ] No [ ] |
| A15a | Held call | P18.6 | A15:o1 |  | Yes [ ] No [ ] |
| A15b | Active call | P18.6 | A15:o1 |  | Yes [ ] No [ ] |
| A15c | Private data in request | P18.8 | A15:o |  | Yes [ ] No [ ] |
| A15d | Security data in request | P18.7 | A15:o |  | Yes [ ] No [ ] |
| A15e | Service result positive | P9.15 | A15:m | [ ] | Yes [ ] |
| A15f | Service result negative | P16 | A15:m | [ ] | Yes [ ] |
| A15g | Private data in result | P18.8 | A15e:o |  | Yes [ ] No [ ] |
| A15h | Security data in result | P18.7 | A15e:o |  | Yes [ ] No [ ] |

## A.6.16	Retrieve call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A16 | Retrieve call service | S10.16 P9.16 | o |  | Yes [ ] No [ ] |
| A16a | Call to be retrieved | P9.16 | A16:m | [ ] | Yes [ ] |
| A16b | Private data in request | P18.8 | A16:o |  | Yes [ ] No [ ] |
| A16c | Security data in request | P18.7 | A16:o |  | Yes [ ] No [ ] |
| A16d | Service result positive | P9.16 | A16:m | [ ] | Yes [ ] |
| A16e | Service result negative | P16 | A16:m | [ ] | Yes [ ] |
| A16f | Private data in result | P18.8 | A16d:o |  | Yes [ ] No [ ] |
| A16g | Security data in result | P18.7 | A16d:o |  | Yes [ ] No [ ] |

## A.6.17	Send DTMF tones

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A17 | Send DTMF tones service | S10.17 P9.17 | o |  | Yes [ ] No [ ] |
| A17a | Connection to send | P9.17 | A17:m | [ ] | Yes [ ] |
| A17b | Characters to send | P9.17 | A17:m | [ ] | Yes [ ] |
| A17c | Tone duration | P9.17 | A17:o |  | Yes [ ] No [ ] |
| A17d | Pause duration | P9.17 | A17:o |  | Yes [ ] No [ ] |
| A17e | Private data in request | P18.8 | A17:o |  | Yes [ ] No [ ] |
| A17f | Security data in request | P18.7 | A17:o |  | Yes [ ] No [ ] |
| A17g | Service result positive | P9.17 | A17:m | [ ] | Yes [ ] |
| A17h | Service result negative | P16 | A17:m | [ ] | Yes [ ] |
| A17i | Private data in result | P18.8 | A17g:o |  | Yes [ ] No [ ] |
| A17j | Security data in result | P18.7 | A17g:o |  | Yes [ ] No [ ] |

## A.6.18	Set feature

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A18 | Set feature service | S10.18 P9.18 | o |  | Yes [ ] No [ ] |
| A18a | Device | P9.18 | A18:m | [ ] | Yes [ ] |
| A18b | Feature | P9.18 | A18:m | [ ] | Yes [ ] |
| A18c | Message waiting on | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18d | Do not disturb on | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18e | Forward | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18f | Forward immediate | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18g | Forward busy | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18h | Forward no answer | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18i | Forward busy internal | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18j | Forward busy external | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18k | Forward no answer internal | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18l | Forward no answer external | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18m | Forward immediate internal | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18n | Forward immediate external | P18.6 | A18e:o2 | [ ] | Yes [ ] No [ ] |
| A18o | Forward to device | P18.6 | A18e:m | [ ] | Yes [ ] |
| A18p | Agent state | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18q | Agent working after call | P18.6 | A18p:o3 | [ ] | Yes [ ] No [ ] |
| A18r | Agent ready | P18.6 | A18p:o3 | [ ] | Yes [ ] No [ ] |
| A18s | Agent logged on info | P18.6 | A18p:o3 | [ ] | Yes [ ] No [ ] |
| A18t | Agent logged off info | P18.6 | A18p:o3 | [ ] | Yes [ ] No [ ] |
| A18u | Agent not ready | P18.6 | A18p:o3 | [ ] | Yes [ ] No [ ] |
| A18v | Enable routing | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18w | Auto answer on | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18x | Microphone mute on | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18y | Speaker mute on | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18z | Speaker volume | P18.6 | A18b:o1 |  | Yes [ ] No [ ] |
| A18aa | Device profile | P9.18 | A18:o |  | Yes [ ] No [ ] |
| A18ab | Device class | P18.6 | A18aa:m | [ ] | Yes [ ] |
| A18ac | ISDN set up | P18.6 | A18aa:o |  | Yes [ ] No [ ] |
| A18ad | Private data in request | P18.8 | A18:o |  | Yes [ ] No [ ] |
| A18ae | Security data in request | P18.7 | A18:o |  | Yes [ ] No [ ] |
| A18af | Service result positive | P9.18 | A18:m | [ ] | Yes [ ] |
| A18ag | Service result negative | P16 | A18:m | [ ] | Yes [ ] |
| A18ah | Private data in result | P18.8 | A18af:o |  | Yes [ ] No [ ] |
| A18ai | Security data in result | P18.7 | A18af:o |  | Yes [ ] No [ ] |

## A.6.19	Single step conference

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A19 | Single step conference service | S10.19 P9.19 | o |  | Yes [ ] No [ ] |
| A19a | Active call | P9.19 | A19:m | [ ] | Yes [ ] |
| A19b | Device to join | P9.19 | A19:m | [ ] | Yes [ ] |
| A19c | Participation type | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19d | Device profile | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19e | Device class | P18.6 | A19d:m | [ ] | Yes [ ] |
| A19f | ISDN set up | P18.6 | A19d:o |  | Yes [ ] No [ ] |
| A19g | Account information | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19h | Authorization code | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19i | Correlator data | P9.19 | A19:o |  | Yes [ ] No [ ] |
| A19j | Private data in request | P18.8 | A19:o |  | Yes [ ] No [ ] |
| A19k | Security data in request | P18.7 | A19:o |  | Yes [ ] No [ ] |
| A19l | Service result positive | P9.19 | A19:m | [ ] | Yes [ ] |
| A19m | Service result negative | P16 | A19:m | [ ] | Yes [ ] |
| A19n | Conferenced call | P9.19 | A19l:m | [ ] | Yes [ ] |
| A19o | Private data in result | P18.8 | A19l:o |  | Yes [ ] No [ ] |
| A19p | Security data in result | P18.7 | A19l:o |  | Yes [ ] No [ ] |

## A.6.20	Single step transfer

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A20 | Single step transfer service | S10.20 P9.20 | o |  | Yes [ ] No [ ] |
| A20a | Active call | P9.20 | A20:m | [ ] | Yes [ ] |
| A20b | Device to transfer | P9.20 | A20:m | [ ] | Yes [ ] |
| A20c | Device profile | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20d | Device class | P18.6 | A20c:m | [ ] | Yes [ ] |
| A20e | ISDN set up | P18.6 | A20c:o |  | Yes [ ] No [ ] |
| A20f | Account information | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20g | Authorization code | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20h | Correlator data | P9.20 | A20:o |  | Yes [ ] No [ ] |
| A20i | Private data in request | P18.8 | A20:o |  | Yes [ ] No [ ] |
| A20j | Security data in request | P18.7 | A20:o |  | Yes [ ] No [ ] |
| A20k | Service result positive | P9.20 | A20:m | [ ] | Yes [ ] |
| A20l | Service result negative | P16 | A20:m | [ ] | Yes [ ] |
| A20m | Transferred call | P9.20 | A20k:o |  | Yes [ ] No [ ] |
| A20n | Connections | P18.4 | A20k:o |  | Yes [ ] No [ ] |
| A20o | New connection | P18.4 | A20n:o1 |  | Yes [ ] No [ ] |
| A20p | Device ID | P18.4 | A20n:o1 |  | Yes [ ] No [ ] |
| A20q | Old connection | P18.4 | A20n:o1 |  | Yes [ ] No [ ] |
| A20r | Private data in result | P18.8 | A20k:o |  | Yes [ ] No [ ] |
| A20s | Security data in result | P18.7 | A20k:o |  | Yes [ ] No [ ] |

## A.6.21	Transfer call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| A21 | Transfer call service | S10.21 P9.21 | o |  | Yes [ ] No [ ] |
| A21a | Transfer information | P9.21 | A21:m | [ ] | Yes [ ] |
| A21b | Held call | P18.6 | A21a:o1 |  | Yes [ ] No [ ] |
| A21c | Active call | P18.6 | A21a:o1 |  | Yes [ ] No [ ] |
| A21d | Private data in request | P18.8 | A21:o |  | Yes [ ] No [ ] |
| A21e | Security data in request | P18.7 | A21:o |  | Yes [ ] No [ ] |
| A21f | Service result positive | P9.21 | A21:m | [ ] | Yes [ ] |
| A21g | Service result negative | P16 | A21:m | [ ] | Yes [ ] |
| A21h | Transferred call | P9.21 | A21f:o |  | Yes [ ] No [ ] |
| A21i | Connections | P18.4 | A21f:o |  | Yes [ ] No [ ] |
| A21j | New connection | P18.4 | A21i:o2 |  | Yes [ ] No [ ] |
| A21k | Device ID | P18.4 | A21i:o2 |  | Yes [ ] No [ ] |
| A21l | Old connection | P18.4 | A21i:o2 |  | Yes [ ] No [ ] |
| A21m | Private data in result | P18.8 | A21f:o |  | Yes [ ] No [ ] |
| A21n | Security data in result | P18.7 | A21f:o |  | Yes [ ] No [ ] |

# A.7	Switching function events

## A.7.1	Event report service

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B1 | Event report service | S11.2 P10 | o |  | Yes [ ] No [ ] |
| B1a | Monitor cross reference | P5.4 P10 | B1:m | [ ] | Yes [ ] |
| B1b | Event specific information | P10 | B1:m | [ ] | Yes [ ] |
| B1c | Private data | P18.8 | B1:o |  | Yes [ ] No [ ] |
| B1d | Security data | P18.7 | B1:o |  | Yes [ ] No [ ] |

## A.7.2	Call events

### A.7.2.1	Call cleared

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B2 | Call cleared event | S11.2.3.1 P10.1.1 | B1b:o |  | Yes [ ] No [ ] |
| B2a | Cleared call | P10.1.1 | B2:m | [ ] | Yes [ ] |
| B2b | Local connection state | P10.1.1 | B2:o |  | Yes [ ] No [ ] |
| B2c | Correlator data | P10.1.1 | B2:o |  | Yes [ ] No [ ] |
| B2d | Cause | P10.1.1 | B2:o |  | Yes [ ] No [ ] |

### A.7.2.2	Conferenced

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B3 | Conferenced event | S11.2.3.2 P10.1.2 | B1b:o |  | Yes [ ] No [ ] |
| B3a | Primary old call | P10.1.2 | B3:m | [ ] | Yes [ ] |
| B3b | Conference controller | P10.1.2 | B3:m | [ ] | Yes [ ] |
| B3c | Added party | P10.1.2 | B3:m | [ ] | Yes [ ] |
| B3d | Secondary old call | P10.1.2 | B3:C | [ ] | Yes [ ] No [ ] |
| B3e | Connections | P10.1.2 | B3:o |  | Yes [ ] No [ ] |
| B3f | New connection | P18.4 | B3e:o1 |  | Yes [ ] No [ ] |
| B3g | Device ID | P18.4 | B3e:o1 |  | Yes [ ] No [ ] |
| B3h | Old connection | P18.4 | B3e:o1 |  | Yes [ ] No [ ] |
| B3i | Local connection information | P10.1.2 | B3:o |  | Yes [ ] No [ ] |
| B3j | Correlator data | P10.1.2 | B3:o |  | Yes [ ] No [ ] |
| B3k | Cause | P10.1.2 | B3:o |  | Yes [ ] No [ ] |

C: If provided in previous events then mandatory

else optional

### A.7.2.3	Connection cleared

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B4 | Connection cleared event | S11.2.3.3 P10.1.3 | B1b:o |  | Yes [ ] No [ ] |
| B4a | Dropped connection | P10.1.3 | B4:m | [ ] | Yes [ ] |
| B4b | Releasing device | P10.1.3 | B4:m | [ ] | Yes [ ] |
| B4c | Local connection information | P10.1.3 | B4:o |  | Yes [ ] No [ ] |
| B4d | Correlator data | P10.1.3 | B4:o |  | Yes [ ] No [ ] |
| B4e | Cause | P10.1.3 | B4:o |  | Yes [ ] No [ ] |

### A.7.2.4	Delivered

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B5 | Delivered event | S11.2.3.4 P10.1.4 | B1b:o |  | Yes [ ] No [ ] |
| B5a | Connection | P10.1.4 | B5:m | [ ] | Yes [ ] |
| B5b | Alerting device | P10.1.4 | B5:m | [ ] | Yes [ ] |
| B5c | Calling device | P10.1.4 | B5:m | [ ] | Yes [ ] |
| B5d | Called device | P10.1.4 | B5:m | [ ] | Yes [ ] |
| B5e | Last redirection device | P10.1.4 | B5:m | [ ] | Yes [ ] |
| B5f | Originating connection | P10.1.4 | B5:o |  | Yes [ ] No [ ] |
| B5g | Local connection information | P10.1.4 | B5:o |  | Yes [ ] No [ ] |
| B5h | Correlator data | P10.1.4 | B5:o |  | Yes [ ] No [ ] |
| B5i | Cause | P10.1.4 | B5:o |  | Yes [ ] No [ ] |

### A.7.2.5	Diverted

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B6 | Diverted event | S11.2.3.5 P10.1.5 | B1b:o |  | Yes [ ] No [ ] |
| B6a | Diverted connection | P10.1.5 | B6:C | [ ] | Yes [ ] No [ ] |
| B6b | Diverting device | P10.1.5 | B6:m | [ ] | Yes [ ] |
| B6c | New destination | P10.1.5 | B6:m | [ ] | Yes [ ] |
| B6d | Local connection information | P10.1.5 | B6:o |  | Yes [ ] No [ ] |
| B6e | Correlator data | P10.1.5 | B6:o |  | Yes [ ] No [ ] |
| B6f | Cause | P10.1.5 | B6:o |  | Yes [ ] No [ ] |

C: If the call alerted the device then mandatory

else optional

### A.7.2.6	Established

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B7 | Established event | S11.2.3.6 P10.1.6 | B1b:o |  | Yes [ ] No [ ] |
| B7a | Established connection | P10.1.6 | B7:m | [ ] | Yes [ ] |
| B7b | Answering device | P10.1.6 | B7:m | [ ] | Yes [ ] |
| B7c | Calling device | P10.1.6 | B7:m | [ ] | Yes [ ] |
| B7d | Called device | P10.1.6 | B7:m | [ ] | Yes [ ] |
| B7e | Last redirection device | P10.1.6 | B7:m | [ ] | Yes [ ] |
| B7f | Originating connection | P10.1.6 | B7:o |  | Yes [ ] No [ ] |
| B7g | Local connection information | P10.1.6 | B7:o |  | Yes [ ] No [ ] |
| B7h | Correlator data | P10.1.6 | B7:o |  | Yes [ ] No [ ] |
| B7i | Cause | P10.1.6 | B7:o |  | Yes [ ] No [ ] |

### A.7.2.7	Failed

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B8 | Failed event | S11.2.3.7 P10.1.7 | B1b:o |  | Yes [ ] No [ ] |
| B8a | Failed connection | P10.1.7 | B8:m | [ ] | Yes [ ] |
| B8b | Failed device | P10.1.7 | B8:m | [ ] | Yes [ ] |
| B8c | Called device | P10.1.7 | B8:m | [ ] | Yes [ ] |
| B8d | Local connection information | P10.1.7 | B8:o |  | Yes [ ] No [ ] |
| B8e | Correlator data | P10.1.7 | B8:o |  | Yes [ ] No [ ] |
| B8f | Cause | P10.1.7 | B8:o |  | Yes [ ] No [ ] |

### A.7.2.8	Held

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B9 | Held event | S11.2.3.8 P10.1.8 | B1b:o |  | Yes [ ] No [ ] |
| B9a | Held connection | P10.1.8 | B9:m | [ ] | Yes [ ] |
| B9b | Holding device | P10.1.8 | B9:m | [ ] | Yes [ ] |
| B9c | Local connection information | P10.1.8 | B9:o |  | Yes [ ] No [ ] |
| B9d | Correlator data | P10.1.8 | B9:o |  | Yes [ ] No [ ] |
| B9e | Cause | P10.1.8 | B9:o |  | Yes [ ] No [ ] |

### A.7.2.9	Network reached

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B10 | Network reached event | S11.2.3.9 P10.1.9 | B1b:o |  | Yes [ ] No [ ] |
| B10a | Connection | P10.1.9 | B10:m | [ ] | Yes [ ] |
| B10b | Trunk used | P10.1.9 | B10:m | [ ] | Yes [ ] |
| B10c | Called device | P10.1.9 | B10:m | [ ] | Yes [ ] |
| B10d | Local connection information | P10.1.9 | B10:o |  | Yes [ ] No [ ] |
| B10e | Correlator data | P10.1.9 | B10:o |  | Yes [ ] No [ ] |
| B10f | Cause | P10.1.9 | B10:o |  | Yes [ ] No [ ] |

### A.7.2.10	Originated

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B11 | Originated event | S11.2.3.10 P10.1.10 | B1b:o |  | Yes [ ] No [ ] |
| B11a | Originated connection | P10.1.10 | B11:m | [ ] | Yes [ ] |
| B11b | Calling device | P10.1.10 | B11:m | [ ] | Yes [ ] |
| B11c | Called device | P10.1.10 | B11:m | [ ] | Yes [ ] |
| B11d | Originating device | P10.1.10 | B11:o |  | Yes [ ] No [ ] |
| B11e | Local connection information | P10.1.10 | B11:o |  | Yes [ ] No [ ] |
| B11f | Correlator data | P10.1.10 | B11:o |  | Yes [ ] No [ ] |
| B11g | Cause | P10.1.10 | B11:o |  | Yes [ ] No [ ] |

### A.7.2.11	Queued

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B12 | Queued event | S11.2.3.11 P10.1.11 | B1b:o |  | Yes [ ] No [ ] |
| B12a | Queued connection | P10.1.11 | B12:m | [ ] | Yes [ ] |
| B12b | Queue | P10.1.11 | B12:m | [ ] | Yes [ ] |
| B12c | Calling device | P10.1.11 | B12:m | [ ] | Yes [ ] |
| B12d | Called device | P10.1.11 | B12:m | [ ] | Yes [ ] |
| B12e | Last redirection device | P10.1.11 | B12:m | [ ] | Yes [ ] |
| B12f | Number of calls in queue | P10.1.11 | B12:o |  | Yes [ ] No [ ] |
| B12g | Calls in front | P10.1.11 | B12:o |  | Yes [ ] No [ ] |
| B12h | Local connection information | P10.1.11 | B12:o |  | Yes [ ] No [ ] |
| B12i | Correlator data | P10.1.11 | B12:o |  | Yes [ ] No [ ] |
| B12j | Cause | P10.1.11 | B12:o |  | Yes [ ] No [ ] |

### A.7.2.12	Retrieved

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B13 | Retrieved event | S11.2.3.12 P10.1.12 | B1b:o |  | Yes [ ] No [ ] |
| B13a | Retrieved connection | P10.1.12 | B13:m | [ ] | Yes [ ] |
| B13b | Retrieving device | P10.1.12 | B13:m | [ ] | Yes [ ] |
| B13c | Local connection information | P10.1.12 | B13:o |  | Yes [ ] No [ ] |
| B13d | Correlator data | P10.1.12 | B13:o |  | Yes [ ] No [ ] |
| B13e | Cause | P10.1.12 | B13:o |  | Yes [ ] No [ ] |

### A.7.2.13	Service initiated

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B14 | Service initiated event | S11.2.3.13 P10.1.13 | B1b:o |  | Yes [ ] No [ ] |
| B14a | Initiated connection | P10.1.13 | B14:m | [ ] | Yes [ ] |
| B14b | Local connection information | P10.1.13 | B14:o |  | Yes [ ] No [ ] |
| B14c | Cause | P10.1.13 | B14:o |  | Yes [ ] No [ ] |

### A.7.2.14	Transferred

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B15 | Transferred event | S11.2.3.14 P10.1.14 | B1b:o |  | Yes [ ] No [ ] |
| B15a | Primary old call | P10.1.14 | B15:m | [ ] | Yes [ ] |
| B15b | Secondary old call | P10.1.14 | B15:C | [ ] | Yes [ ] No [ ] |
| B15c | Transferring device | P10.1.14 | B15:m | [ ] | Yes [ ] |
| B15d | Transferred-to device | P10.1.14 | B15:m | [ ] | Yes [ ] |
| B15e | Connections | P10.1.14 | B15:o |  | Yes [ ] No [ ] |
| B15f | New connection | P18.4 | B15e:o1 |  | Yes [ ] No [ ] |
| B15g | Device ID | P18.4 | B15e:o1 |  | Yes [ ] No [ ] |
| B15h | Old connection | P18.4 | B15e:o1 |  | Yes [ ] No [ ] |
| B15i | Local connection information | P10.1.14 | B15:o |  | Yes [ ] No [ ] |
| B15j | Correlator data | P10.1.14 | B15:o |  | Yes [ ] No [ ] |
| B15k | Cause | P10.1.14 | B15:o |  | Yes [ ] No [ ] |

C: If parameter in previous events then mandatory

else optional

## A.7.3	Agent events

### A.7.3.1	Agent logged on

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B16 | Agent logged on event | S11.2.2.2 P10.3.2 | B1b:o |  | Yes [ ] No [ ] |
| B16a | Agent device | P10.3.2 | B16:m | [ ] | Yes [ ] |
| B16b | Agent ID | P10.3.2 | B16:o |  | Yes [ ] No [ ] |
| B16c | Agent group | P10.3.2 | B16:o |  | Yes [ ] No [ ] |
| B16d | Password | P10.3.2 | B16:o |  | Yes [ ] No [ ] |
| B16e | Cause | P10.3.2 | B16:o |  | Yes [ ] No [ ] |

### A.7.3.2	Agent logged off

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B17 | Agent logged off event | S11.2.2.1 P10.3.3 | B1b:o |  | Yes [ ] No [ ] |
| B17a | Agent device | P10.3.3 | B17:m | [ ] | Yes [ ] |
| B17b | Agent ID | P10.3.3 | B17:o |  | Yes [ ] No [ ] |
| B17c | Password | P10.3.3 | B17:o |  | Yes [ ] No [ ] |
| B17d | Agent group | P10.3.3 | B17:o |  | Yes [ ] No [ ] |
| B17e | Cause | P10.3.3 | B17:o |  | Yes [ ] No [ ] |

### A.7.3.3	Agent ready

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B18 | Agent ready event | S11.2.2.4 P10.3.5 | B1b:o |  | Yes [ ] No [ ] |
| B18a | Agent device | P10.3.5 | B18:m | [ ] | Yes [ ] |
| B18b | Agent ID | P10.3.5 | B18:o |  | Yes [ ] No [ ] |
| B18c | Cause | P10.3.5 | B18:o |  | Yes [ ] No [ ] |

### A.7.3.4	Agent not ready

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B19 | Agent not ready event | S11.2.2.3 P10.3.4 | B1b:o |  | Yes [ ] No [ ] |
| B19a | Agent device | P10.3.4 | B19:m | [ ] | Yes [ ] |
| B19b | Agent ID | P10.3.4 | B19:o |  | Yes [ ] No [ ] |
| B19c | Cause | P10.3.4 | B19:o |  | Yes [ ] No [ ] |

### A.7.3.5	Agent working after call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B20 | Agent working after call event | S11.2.2.5 P10.3.6 | B1b:o |  | Yes [ ] No [ ] |
| B20a | Agent device | P10.3.6 | B20:m | [ ] | Yes [ ] |
| B20b | Agent ID | P10.3.6 | B20:o |  | Yes [ ] No [ ] |
| B20c | Agent group | P10.3.6 | B20:o |  | Yes [ ] No [ ] |
| B20d | Cause | P10.3.6 | B20:o |  | Yes [ ] No [ ] |

### A.7.3.6	Agent busy

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B21 | Agent busy event | S11.2.2.6 P10.3.1 | B1b:o |  | Yes [ ] No [ ] |
| B21a | Agent device | P10.3.1 | B21:m | [ ] | Yes [ ] |
| B21b | Agent ID | P10.3.1 | B21:o |  | Yes [ ] No [ ] |
| B21c | Agent group | P10.3.1 | B21:o |  | Yes [ ] No [ ] |
| B21d | Cause | P10.3.1 | B21:o |  | Yes [ ] No [ ] |

## A.7.4	Other feature events

### A.7.4.1	Call information

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B22 | Call information event | S11.2.4.1 P10.2.2 | B1b:o |  | Yes [ ] No [ ] |
| B22a | Connection ID | P10.2.2 | B22:m | [ ] | Yes [ ] |
| B22b | Device | P10.2.2 | B22:m | [ ] | Yes [ ] |
| B22c | Account information | P10.2.2 | B22:o |  | Yes [ ] No [ ] |
| B22d | Authorization code | P10.2.2 | B22:o |  | Yes [ ] No [ ] |
| B22e | Correlator data | P10.2.2 | B22:o |  | Yes [ ] No [ ] |

### A.7.4.2	Do not disturb

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B23 | Do not disturb event | S11.2.4.2 P10.2.3 | B1b:o |  | Yes [ ] No [ ] |
| B23a | Device | P10.2.3 | B23:m | [ ] | Yes [ ] |
| B23b | Do not disturb on | P10.2.3 | B23:m | [ ] | Yes [ ] |

### A.7.4.3	Forwarding

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B24 | Forwarding event | S11.2.4.3 P10.2.4 | B1b:o |  | Yes [ ] No [ ] |
| B24a | Device | P10.2.4 | B24:m | [ ] | Yes [ ] |
| B24b | Forwarding type | P18.6 | B24:m | [ ] | Yes [ ] |
| B24c | Forward DN | P18.6 | B24:o |  | Yes [ ] No [ ] |

### A.7.4.4	Message waiting

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B25 | Message waiting event | S11.2.4.4 P10.2.5 | B1b:o |  | Yes [ ] No [ ] |
| B25a | Device for message | P10.2.5 | B25:m | [ ] | Yes [ ] |
| B25b | Invoking device | P10.2.5 | B25:m | [ ] | Yes [ ] |
| B25c | Message waiting on | P10.2.5 | B25:m | [ ] | Yes [ ] |

### A.7.4.5	Auto answer

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B26 | Auto answer event | S11.2.4.5 P10.2.1 | B1b:o |  | Yes [ ] No [ ] |
| B26a | Device | P10.2.1 | B26:m | [ ] | Yes [ ] |
| B26b | Auto answer on | P10.2.1 | B26:m | [ ] | Yes [ ] |

### A.7.4.6	Microphone mute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B27 | Microphone mute event | S11.2.4.6 P10.2.6 | B1b:o |  | Yes [ ] No [ ] |
| B27a | Device | P10.2.6 | B27:m | [ ] | Yes [ ] |
| B27b | Microphone mute on | P10.2.6 | B27:m | [ ] | Yes [ ] |

### A.7.4.7	Speaker mute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B28 | Speaker mute event | S11.2.4.7 P10.2.7 | B1b:o |  | Yes [ ] No [ ] |
| B28a | Device | P10.2.7 | B28:m | [ ] | Yes [ ] |
| B28b | Speaker mute on | P10.2.7 | B28:m | [ ] | Yes [ ] |

### A.7.4.8	Speaker volume

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B29 | Speaker volume event | S11.2.4.8 P10.2.8 | B1b:o |  | Yes [ ] No [ ] |
| B29a | Device | P10.2.8 | B29:m | [ ] | Yes [ ] |
| B29b | Speaker volume | P10.2.8 | B29:m | [ ] | Yes [ ] |

## A.7.5	Maintenance events

### A.7.5.1	Back in service

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B30 | Back in service event | S11.2.5.1 P10.4.1 | B1b:o |  | Yes [ ] No [ ] |
| B30a | Device | P10.4.1 | B30:m | [ ] | Yes [ ] |
| B30b | Cause | P10.4.1 | B30:o |  | Yes [ ] No [ ] |

### A.7.5.2	Out of service

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B31 | Out of service event | S10.2.5.2 P10.4.2 | B1b:o |  | Yes [ ] No [ ] |
| B31a | Device | P10.4.2 | B31:m | [ ] | Yes [ ] |
| B31b | Cause | P10.4.2 | B31:o |  | Yes [ ] No [ ] |

## A.7.6	Private events

### A.7.6.1	Private

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B32 | Is/are private event(s) | S11.2.6 P10.5 | B1b:o |  | Yes [ ] No [ ] |

## A.7.7	Voice unit events

### A.7.7.1	Play

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B33 | Play event | S11.2.7.1 P10.6.1 | B1b:o |  | Yes [ ] No [ ] |
| B33a | Connection | P10.6.1 | B33:m | [ ] | Yes [ ] |
| B33b | Message | P10.6.1 | B33:m | [ ] | Yes [ ] |
| B33c | Message length | P10.6.1 | B33:o |  | Yes [ ] No [ ] |
| B33d | Current position in message | P10.6.1 | B33:o |  | Yes [ ] No [ ] |
| B33e | Speed | P10.6.1 | B33:o |  | Yes [ ] No [ ] |
| B33f | Cause | P10.6.1 | B33:o |  | Yes [ ] No [ ] |

### A.7.7.2	Record

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B34 | Record event | S11.2.7.2 P10.6.2 | B1b:o |  | Yes [ ] No [ ] |
| B34a | Connection | P10.6.2 | B34:m | [ ] | Yes [ ] |
| B34b | Message | P10.6.2 | B34:m | [ ] | Yes [ ] |
| B34c | Message length | P10.6.2 | B34:o |  | Yes [ ] No [ ] |
| B34d | Current position in message | P10.6.2 | B34:o |  | Yes [ ] No [ ] |
| B34e | Cause | P10.6.2 | B34:o |  | Yes [ ] No [ ] |

### A.7.7.3	Review

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B35 | Review event | S11.2.7.3 P10.6.3 | B1b:o |  | Yes [ ] No [ ] |
| B35a | Connection | P10.6.3 | B35:m | [ ] | Yes [ ] |
| B35b | Message | P10.6.3 | B35:m | [ ] | Yes [ ] |
| B35c | Message length | P10.6.3 | B35:o |  | Yes [ ] No [ ] |
| B35d | Current position in message | P10.6.3 | B35:o |  | Yes [ ] No [ ] |
| B35e | Cause | P10.6.3 | B35:o |  | Yes [ ] No [ ] |

### A.7.7.4	Stop

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B36 | Stop event | S11.2.7.4 P10.6.4 | B1b:o |  | Yes [ ] No [ ] |
| B36a | Connection | P10.6.4 | B36:m | [ ] | Yes [ ] |
| B36b | Message | P10.6.4 | B36:m | [ ] | Yes [ ] |
| B36c | Message length | P10.6.4 | B36:o |  | Yes [ ] No [ ] |
| B36d | Current position in message | P10.6.4 | B36:o |  | Yes [ ] No [ ] |
| B36e | Cause | P10.6.4 | B36:o |  | Yes [ ] No [ ] |

### A.7.7.5	Suspend play

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B37 | Suspend play event | S11.2.7.5 P10.6.5 | B1b:o |  | Yes [ ] No [ ] |
| B37a | Connection | P10.6.5 | B37:m | [ ] | Yes [ ] |
| B37b | Message | P10.6.5 | B37:m | [ ] | Yes [ ] |
| B37c | Message length | P10.6.5 | B37:o |  | Yes [ ] No [ ] |
| B37d | Current position in message | P10.6.5 | B37:o |  | Yes [ ] No [ ] |
| B37e | Cause | P10.6.5 | B37:o |  | Yes [ ] No [ ] |

### A.7.7.6	Suspend record

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B38 | Suspend record event | S11.2.7.6 P10.6.6 | B1b:o |  | Yes [ ] No [ ] |
| B38a | Connection | P10.6.6 | B38:m | [ ] | Yes [ ] |
| B38b | Message | P10.6.6 | B38:m | [ ] | Yes [ ] |
| B38c | Message length | P10.6.6 | B38:o |  | Yes [ ] No [ ] |
| B38d | Current position in message | P10.6.6 | B38:o |  | Yes [ ] No [ ] |
| B38e | Cause | P10.6.6 | B38:o |  | Yes [ ] No [ ] |

### A.7.7.7	Voice attribute change

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| B39 | Voice attribute change event | S11.2.7.7 P10.6.7 | B1b:o |  | Yes [ ] No [ ] |
| B39a | Connection | P10.6.7 | B39:m | [ ] | Yes [ ] |
| B39b | Message | P10.6.7 | B39:m | [ ] | Yes [ ] |
| B39c | Speaker volume | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39d | Recording level | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39e | Speed | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39f | Current position in message | P10.6.7 | B39:o |  | Yes [ ] No [ ] |
| B39g | Cause | P10.6.7 | B39:o |  | Yes [ ] No [ ] |

# A.8	Computing function services

## A.8.1	Route request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C1 | Route request service | S12.3 P11.1 | o |  | Yes [ ] No [ ] |
| C1a | Cross reference | P11.1 | C1:m | [ ] | Yes [ ] |
| C1b | Current route | P11.1 | C1:m | [ ] | Yes [ ] |
| C1c | Calling device | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1d | Routing device | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1e | Routed call | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1f | Route select algorithm | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1g | Priority | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1h | Device profile | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1i | Device class | P18.6 | C1h:m | [ ] | Yes [ ] |
| C1j | ISDN set up | P18.6 | C1h:o |  | Yes [ ] No [ ] |
| C1k | Correlator data | P11.1 | C1:o |  | Yes [ ] No [ ] |
| C1l | Private data in request | P18.8 | C1:o |  | Yes [ ] No [ ] |
| C1m | Security data in request | P18.7 | C1:o |  | Yes [ ] No [ ] |
| C1n | Service result negative | P16 | C1:m | [ ] | Yes [ ] |

## A.8.2	Reroute request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C2 | Reroute request service | S12.1 P11.2 | o |  | Yes [ ] No [ ] |
| C2a | Cross reference | P11.2 | C2:m | [ ] | Yes [ ] |
| C2b | Private data in request | P18.8 | C2:o |  | Yes [ ] No [ ] |
| C2c | Security data in request | P18.7 | C2:o |  | Yes [ ] No [ ] |
| C2d | Service result negative | P16 | C2:m | [ ] | Yes [ ] |

## A.8.3	Route select request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C3 | Route select request service | S12.4 P11.3 | o |  | Yes [ ] No [ ] |
| C3a | Cross reference | P11.3 | C3:m | [ ] | Yes [ ] |
| C3b | Route selected | P11.3 | C3:m | [ ] | Yes [ ] |
| C3c | Remaining retries | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3d | Device profile | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3e | Device class | P18.6 | C3d:m | [ ] | Yes [ ] |
| C3f | ISDN set up | P18.6 | C3d:o |  | Yes [ ] No [ ] |
| C3g | Route used request positive | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3h | Correlator data | P11.3 | C3:o |  | Yes [ ] No [ ] |
| C3i | Private data in request | P18.8 | C3:o |  | Yes [ ] No [ ] |
| C3j | Security data in request | P18.7 | C3:o |  | Yes [ ] No [ ] |
| C3k | Service result negative | P16 | C3:m | [ ] | Yes [ ] |

## A.8.4	Route used request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C4 | Route used request service | S12.5 P11.4 | o |  | Yes [ ] No [ ] |
| C4a | Cross reference | P11.4 | C4:m | [ ] | Yes [ ] |
| C4b | Route used | P11.4 | C4:m | [ ] | Yes [ ] |
| C4c | Calling device | P11.4 | C4:o |  | Yes [ ] No [ ] |
| C4d | Domain | P11.4 | C4:o |  | Yes [ ] No [ ] |
| C4e | Correlator data | P11.4 | C4:o |  | Yes [ ] No [ ] |
| C4f | Private data in request | P18.8 | C4:o |  | Yes [ ] No [ ] |
| C4g | Security data in request | P18.7 | C4:o |  | Yes [ ] No [ ] |
| C4h | Service result negative | P16 | C4:m | [ ] | Yes [ ] |

## A.8.5	Route end request

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| C5 | Route end request service | S12.2 P11.5 | o |  | Yes [ ] No [ ] |
| C5a | Cross reference | P11.5 | C5:m | [ ] | Yes [ ] |
| C5b | Error value | P11.5 | C5:o |  | Yes [ ] No [ ] |
| C5c | Private data in request | P18.8 | C5:o |  | Yes [ ] No [ ] |
| C5d | Security data in request | P18.7 | C5:o |  | Yes [ ] No [ ] |
| C5e | Service result negative | P16 | C5:o |  | Yes [ ] No [ ] |

# A.9	Bidirectional services

## A.9.1	Escape

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| D1 | Escape service | S13.1 P12.1 | o |  | Yes [ ] No [ ] |
| D1a | Security data in request | P18.7 | D1:o |  | Yes [ ] No [ ] |
| D1b | Private data in request | P18.8 | D1:m | [ ] | Yes [ ] |
| D1c | Service result positive | P12.1 | D1:m | [ ] | Yes [ ] |
| D1d | Service result negative | P16 | D1:m | [ ] | Yes [ ] |
| D1e | Security data in result | P18.7 | D1c:o |  | Yes [ ] No [ ] |
| D1f | Private data in result | P18.8 | D1c:o |  | Yes [ ] No [ ] |

## A.9.2	System status

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| D2 | System status service | S13.2 P12.2 | o |  | Yes [ ] No [ ] |
| D2a | System status cause | P12.2 | D2:m | [ ] | Yes [ ] |
| D2b | Initializing | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2c | Enabled | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2d | Normal | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2e | Messages lost | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2f | Disabled | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2g | Overload imminent | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2h | Overload reached | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2i | Overload relieved | P18.6 | D2a:o |  | Yes [ ] No [ ] |
| D2j | Security data in request | P18.7 | D2:o |  | Yes [ ] No [ ] |
| D2k | Private data in request | P18.8 | D2:o |  | Yes [ ] No [ ] |
| D2l | Service result positive | P12.2 | D2:m | [ ] | Yes [ ] |
| D2m | Service result negative | P16 | D2:m | [ ] | Yes [ ] |
| D2n | Security data in result | P18.7 | D2l:o |  | Yes [ ] No [ ] |
| D2o | Private data in result | P18.8 | D2l:o |  | Yes [ ] No [ ] |

# A.10	Status reporting services

## A.10.1	Change monitor filter

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E1 | Change monitor filter service | S11.1 P13.2 | o |  | Yes [ ] No [ ] |
| E1a | Cross reference ID in request | P13.2 | E1:m | [ ] | Yes [ ] |
| E1b | Filter list in request | P13.2 | E1:m | [ ] | Yes [ ] |
| E1c | Security data in request | P18.7 | E1:o |  | Yes [ ] No [ ] |
| E1d | Private data in request | P18.8 | E1:o |  | Yes [ ] No [ ] |
| E1e | Service result positive | P13.2 | E1:m | [ ] | Yes [ ] |
| E1f | Service result negative | P16 | E1:m | [ ] | Yes [ ] |
| E1g | Filter list in result | P13.2 | E1e:o |  | Yes [ ] No [ ] |
| E1h | Security data in result | P18.7 | E1e:o |  | Yes [ ] No [ ] |
| E1i | Private data in result | P18.8 | E1e:o |  | Yes [ ] No [ ] |

## A.10.2	Monitor start

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E2 | Monitor start service | S11.3 P13.1 | o |  | Yes [ ] No [ ] |
| E2a | Monitor object | P13.1 | E2:m | [ ] | Yes [ ] |
| E2b | Monitor object device | P18.5 | E2a:o1 |  | Yes [ ] No [ ] |
| E2c | Monitor object call | P18.5 | E2a:o1 |  | Yes [ ] No [ ] |
| E2d | Monitor type device | P18.5 | E2a:o |  | Yes [ ] No [ ] |
| E2e | Monitor type call | P18.5 | E2a:o |  | Yes [ ] No [ ] |
| E2f | Monitor filter for call processing events | P18.5 | E2:o |  | Yes [ ] No [ ] |
| E2g | Monitor filter for feature events | P18.5 | E2:o |  | Yes [ ] No [ ] |
| E2h | Monitor filter for agent events | P18.5 | E2:o |  | Yes [ ] No [ ] |
| E2i | Monitor filter for maintenance events | P18.5 | E2:o |  | Yes [ ] No [ ] |
| E2j | Monitor filter for voice unit events | P18.5 | E2:o |  | Yes [ ] No [ ] |
| E2k | Monitor filter for private filter event | P18.5 | E2:o |  | Yes [ ] No [ ] |
| E2l | Service result positive | P13.1 | E2:m | [ ] | Yes [ ] |
| E2m | Service result negative | P16 | E2:m | [ ] | Yes [ ] |
| E2n | Cross reference ID in result | P13.1 | E2l:m | [ ] | Yes [ ] |
| E2o | Monitor filter in result | P13.1 | E2l:o |  | Yes [ ] No [ ] |
| E2p | Security data in request | P18.7 | E2:o |  | Yes [ ] No [ ] |
| E2q | Private data in request | P18.8 | E2:o |  | Yes [ ] No [ ] |
| E2r | Security data in result | P18.7 | E2l:o |  | Yes [ ] No [ ] |
| E2s | Private data in result | P18.8 | E2l:o |  | Yes [ ] No [ ] |

## A.10.3	Monitor stop

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E3 | Monitor stop service | S11.4 P13.3 | o |  | Yes [ ] No [ ] |
| E3a | Cross reference ID in request | P13.3 | E3:m | [ ] | Yes [ ] |
| E3b | Security data in request | P18.7 | E3:o |  | Yes [ ] No [ ] |
| E3c | Private data in request | P18.8 | E3:o |  | Yes [ ] No [ ] |
| E3d | Service result positive | P13.3 | E3:m | [ ] | Yes [ ] |
| E3e | Service result negative | P16 | E3:m | [ ] | Yes [ ] |
| E3f | Security data in result | P18.7 | E3d:o |  | Yes [ ] No [ ] |
| E3g | Private data in result | P18.8 | E3d:o |  | Yes [ ] No [ ] |

## A.10.4	Snapshot call

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E4 | Snapshot call service | S11.5 P13.5 | o |  | Yes [ ] No [ ] |
| E4a | Snapshot object in request | P13.5 | E4:m | [ ] | Yes [ ] |
| E4b | Security data in request | P18.7 | E4:o |  | Yes [ ] No [ ] |
| E4c | Private data in request | P18.8 | E4:o |  | Yes [ ] No [ ] |
| E4d | Service result positive | P13.5 | E4:m | [ ] | Yes [ ] |
| E4e | Service result negative | P16 | E4:m | [ ] | Yes [ ] |
| E4f | Device ID in result | P13.5 | E4d:m | [ ] | Yes [ ] |
| E4g | Connection ID in result | P13.5 | E4d:m | [ ] | Yes [ ] |
| E4h | Device profile | P13.5 | E4d:o |  | Yes [ ] No [ ] |
| E4i | Device class | P18.6 | E4h:m | [ ] | Yes [ ] |
| E4j | ISDN set up | P18.6 | E4h:o |  | Yes [ ] No [ ] |
| E4k | Local connection state in result | P13.5 | E4d:o |  | Yes [ ] No [ ] |
| E4l | Security data in result | P18.7 | E4d:o |  | Yes [ ] No [ ] |
| E4m | Private data in result | P18.8 | E4d:o |  | Yes [ ] No [ ] |

## A.10.5	Snapshot device

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| E5 | Snapshot device service | S11.6 P13.4 | o |  | Yes [ ] No [ ] |
| E5a | Device ID in request | P13.4 | E5:m | [ ] | Yes [ ] |
| E5b | Security data in request | P18.7 | E5:o |  | Yes [ ] No [ ] |
| E5c | Private data in request | P18.8 | E5:o |  | Yes [ ] No [ ] |
| E5d | Service result positive | P13.4 | E5:m | [ ] | Yes [ ] |
| E5e | Service result negative | P16 | E5:m | [ ] | Yes [ ] |
| E5f | Connection ID in result | P13.4 | E5d:m | [ ] | Yes [ ] |
| E5g | Call state in result | P13.4 | E5d:m | [ ] | Yes [ ] |
| E5h | Device profile | P13.4 | E5d:o |  | Yes [ ] No [ ] |
| E5i | Device class | P18.6 | E5h:m | [ ] | Yes [ ] |
| E5j | ISDN set up | P18.6 | E5h:o |  | Yes [ ] No [ ] |
| E5k | Security data in result | P18.7 | E5d:o |  | Yes [ ] No [ ] |
| E5l | Private data in result | P18.8 | E5d:o |  | Yes [ ] No [ ] |

# A.11	Input/output services

## A.11.1	Data path resumed

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F1 | Data path resumed service | S14.1 P14.9 | o |  | Yes [ ] No [ ] |
| F1a | Cross reference ID | P14.9 | F1:m | [ ] | Yes [ ] |
| F1b | Security data in request | P18.7 | F1:o |  | Yes [ ] No [ ] |
| F1c | Private data in request | P18.8 | F1:o |  | Yes [ ] No [ ] |
| F1d | Service result positive | P14.9 | F1:m | [ ] | Yes [ ] |
| F1e | Service result negative | P16 | F1:m | [ ] | Yes [ ] |
| F1f | Security data in result | P18.7 | F1d:o |  | Yes [ ] No [ ] |
| F1g | Private data in result | P18.8 | F1d:o |  | Yes [ ] No [ ] |

## A.11.2	Data path suspended

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F2 | Data path suspended service | S14.2 P14.7 | o |  | Yes [ ] No [ ] |
| F2a | Cross reference ID | P14.7 | F2:m | [ ] | Yes [ ] |
| F2b | Security data in request | P18.7 | F2:o |  | Yes [ ] No [ ] |
| F2c | Private data in request | P18.8 | F2:o |  | Yes [ ] No [ ] |
| F2d | Service result positive | P14.7 | F2:m | [ ] | Yes [ ] |
| F2e | Service result negative | P16 | F2:m | [ ] | Yes [ ] |
| F2f | Security data in result | P18.7 | F2d:o |  | Yes [ ] No [ ] |
| F2g | Private data in result | P18.8 | F2d:o |  | Yes [ ] No [ ] |

## A.11.3	Fast data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F3 | Fast data service | S14.3 P14.10 | o |  | Yes [ ] No [ ] |
| F3a | CSTA object | P14.10 | F3:m | [ ] | Yes [ ] |
| F3b | Object device | P14.10 | F3a:o1 |  | Yes [ ] No [ ] |
| F3c | Object call | P14.10 | F3a:o1 |  | Yes [ ] No [ ] |
| F3d | I/O data | P14.10 | F3:m | [ ] | Yes [ ] |
| F3e | Data path type text | P14.10 | F3:o |  | Yes [ ] No [ ] |
| F3f | Data path type voice | P14.10 | F3:o |  | Yes [ ] No [ ] |
| F3g | Security data in request | P18.7 | F3:o |  | Yes [ ] No [ ] |
| F3h | Private data in request | P18.8 | F3:o |  | Yes [ ] No [ ] |
| F3i | Service result positive | P14.10 | F3:m | [ ] | Yes [ ] |
| F3j | Service result negative | P16 | F3:m | [ ] | Yes [ ] |
| F3k | Security data in result | P18.7 | F3i:o |  | Yes [ ] No [ ] |
| F3l | Private data in result | P18.8 | F3i:o |  | Yes [ ] No [ ] |

## A.11.4	Resume data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F4 | Resume data path service | S14.4 P14.8 | o |  | Yes [ ] No [ ] |
| F4a | Cross reference ID | P14.8 | F4:m | [ ] | Yes [ ] |
| F4b | Security data in request | P18.7 | F4:o |  | Yes [ ] No [ ] |
| F4c | Private data in request | P18.8 | F4:o |  | Yes [ ] No [ ] |
| F4d | Service result positive | P14.8 | F4:m | [ ] | Yes [ ] |
| F4e | Service result negative | P16 | F4:m | [ ] | Yes [ ] |
| F4f | Security data in result | P18.7 | F4d:o |  | Yes [ ] No [ ] |
| F4g | Private data in result | P18.8 | F4d:o |  | Yes [ ] No [ ] |

## A.11.5	Send broadcast data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F5 | Send broadcast data service | S14.5 P14.5 | o |  | Yes [ ] No [ ] |
| F5a | I/O data | P14.5 | F5:m | [ ] | Yes [ ] |
| F5b | Data path type text | P14.5 | F5:o |  | Yes [ ] No [ ] |
| F5c | Data path type voice | P14.5 | F5:o |  | Yes [ ] No [ ] |
| F5d | Security data in request | P18.7 | F5:o |  | Yes [ ] No [ ] |
| F5e | Private data in request | P18.8 | F5:o |  | Yes [ ] No [ ] |
| F5f | Service result positive | P14.5 | F5:m | [ ] | Yes [ ] |
| F5g | Service result negative | P16 | F5:m | [ ] | Yes [ ] |
| F5h | Security data in result | P18.7 | F5f:o |  | Yes [ ] No [ ] |
| F5i | Private data in result | P18.8 | F5f:o |  | Yes [ ] No [ ] |

## A.11.6	Send data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F6 | Send data service | S14.6 P14.3 | o |  | Yes [ ] No [ ] |
| F6a | Cross reference ID | P14.3 | F6:m | [ ] | Yes [ ] |
| F6b | I/O data | P14.3 | F6:m | [ ] | Yes [ ] |
| F6c | Cause code | S10.4 P16 | F6:o |  | Yes [ ] No [ ] |
| F6d | Security data in request | P18.7 | F6:o |  | Yes [ ] No [ ] |
| F6e | Private data in request | P18.8 | F6:o |  | Yes [ ] No [ ] |
| F6f | Service result positive | P14.3 | F6:m | [ ] | Yes [ ] |
| F6g | Service result negative | P16 | F6:m | [ ] | Yes [ ] |
| F6h | Security data in result | P18.7 | F6f:o |  | Yes [ ] No [ ] |
| F6i | Private data in result | P18.8 | F6f:o |  | Yes [ ] No [ ] |

## A.11.7	Send multicast data

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F7 | Send multicast data service | S14.7 P14.4 | o |  | Yes [ ] No [ ] |
| F7a | I/O cross reference ID list | P14.4 | F7:m | [ ] | Yes [ ] |
| F7b | I/O data | P14.4 | F7:m | [ ] | Yes [ ] |
| F7c | Security data in request | P18.7 | F7:o |  | Yes [ ] No [ ] |
| F7d | Private data in request | P18.8 | F7:o |  | Yes [ ] No [ ] |
| F7e | Service result positive | P14.4 | F7:m | [ ] | Yes [ ] |
| F7f | Service result negative | P16 | F7:m | [ ] | Yes [ ] |
| F7g | Security data in result | P18.7 | F7e:o |  | Yes [ ] No [ ] |
| F7h | Private data in result | P18.8 | F7e:o |  | Yes [ ] No [ ] |

## A.11.8	Start data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F8 | Start data path service | S14.8 P14.1 | o |  | Yes [ ] No [ ] |
| F8a | CSTA object | P14.1 | F8:m | [ ] | Yes [ ] |
| F8b | Object device | P14.1 | F8:o1 |  | Yes [ ] No [ ] |
| F8c | Object call | P14.1 | F8:o1 |  | Yes [ ] No [ ] |
| F8d | Data path from the computing function | P18.6 | F8:o |  | Yes [ ] No [ ] |
| F8e | Data path to the computing function | P18.6 | F8:o |  | Yes [ ] No [ ] |
| F8f | Data path bidirectional | P18.6 | F8:o |  | Yes [ ] No [ ] |
| F8g | Data path type text | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8h | Data path type voice | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8i | Number of characters to collect | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8j | Termination character | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8k | Timeout | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8l | Cross reference ID in result | P14.1 | F8:m | [ ] | Yes [ ] |
| F8m | Number of characters to collect in result | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8n | Termination character in result | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8o | Timeout in result | P14.1 | F8:o |  | Yes [ ] No [ ] |
| F8p | Security data in request | P18.7 | F8:o |  | Yes [ ] No [ ] |
| F8q | Private data in request | P18.8 | F8:o |  | Yes [ ] No [ ] |
| F8r | Service result positive | P14.1 | F8:m | [ ] | Yes [ ] |
| F8s | Service result negative | P16 | F8:m | [ ] | Yes [ ] |
| F8t | Security data in result | P18.7 | F8r:o |  | Yes [ ] No [ ] |
| F8u | Private data in result | P18.8 | F8r:o |  | Yes [ ] No [ ] |

## A.11.9	Stop data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F9 | Stop data path service | S14.9 P14.2 | o |  | Yes [ ] No [ ] |
| F9a | CSTA cross reference ID | P14.2 | F9:m | [ ] | Yes [ ] |
| F9b | Security data in request | P18.7 | F9:o |  | Yes [ ] No [ ] |
| F9c | Private data in request | P18.8 | F9:o |  | Yes [ ] No [ ] |
| F9d | Service result positive | P14.2 | F9:m | [ ] | Yes [ ] |
| F9e | Service result negative | P16 | F9:m | [ ] | Yes [ ] |
| F9f | Security data in result | P18.7 | F9d:o |  | Yes [ ] No [ ] |
| F9g | Private data in result | P18.8 | F9d:o |  | Yes [ ] No [ ] |

## A.11.10	Suspend data path

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| F10 | Suspend data path service | S14.10 P14.6 | o |  | Yes [ ] No [ ] |
| F10a | CSTA cross reference ID | P14.6 | F10:m | [ ] | Yes [ ] |
| F10b | Security data in request | P18.7 | F10:o |  | Yes [ ] No [ ] |
| F10c | Private data in request | P18.8 | F10:o |  | Yes [ ] No [ ] |
| F10d | Service result positive | P14.6 | F10:m | [ ] | Yes [ ] |
| F10e | Service result negative | P16 | F10:m | [ ] | Yes [ ] |
| F10f | Security data in result | P18.7 | F10d:o |  | Yes [ ] No [ ] |
| F10g | Private data in result | P18.8 | F10d:o |  | Yes [ ] No [ ] |

# A.12	Voice unit services

## A.12.1	Concatenate message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G1 | Concatenate message service | S15.1 P15.1 | o |  | Yes [ ] No [ ] |
| G1a | Message ID list | P15.1 | G1:m | [ ] | Yes [ ] |
| G1b | Security data in request | P18.7 | G1:o |  | Yes [ ] No [ ] |
| G1c | Private data in request | P18.8 | G1:o |  | Yes [ ] No [ ] |
| G1d | Service result positive | P15.1 | G1:m | [ ] | Yes [ ] |
| G1e | Service result negative | P16 | G1:m | [ ] | Yes [ ] |
| G1f | Security data in result | P18.7 | G1d:o |  | Yes [ ] No [ ] |
| G1g | Private data in result | P18.8 | G1d:o |  | Yes [ ] No [ ] |

## A.12.2	Delete message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G2 | Delete message service | S15.2 P15.2 | o |  | Yes [ ] No [ ] |
| G2a | Message ID | P15.2 | G2:m | [ ] | Yes [ ] |
| G2b | Security data in request | P18.7 | G2:o |  | Yes [ ] No [ ] |
| G2c | Private data in request | P18.8 | G2:o |  | Yes [ ] No [ ] |
| G2d | Service result positive | P15.2 | G2:m | [ ] | Yes [ ] |
| G2e | Service result negative | P16 | G2:m | [ ] | Yes [ ] |
| G2f | Security data in result | P18.7 | G2d:o |  | Yes [ ] No [ ] |
| G2g | Private data in result | P18.8 | G2d:o |  | Yes [ ] No [ ] |

## A.12.3	Play message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G3 | Play message service | S15.3 P15.3 | o |  | Yes [ ] No [ ] |
| G3a | Message ID | P15.3 | G3:m | [ ] | Yes [ ] |
| G3b | Call associated with message | P15.3 | G3:m | [ ] | Yes [ ] |
| G3c | Duration | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3d | Duration exceeded | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3e | DTMF detected | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3f | End of data detected | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3g | Speech detected | P15.3 | G3:o |  | Yes [ ] No [ ] |
| G3h | Security data in request | P18.7 | G3:o |  | Yes [ ] No [ ] |
| G3i | Private data in request | P18.8 | G3:o |  | Yes [ ] No [ ] |
| G3j | Service result positive | P15.3 | G3:m | [ ] | Yes [ ] |
| G3k | Service result negative | P16 | G3:m | [ ] | Yes [ ] |
| G3l | Security data in result | P18.7 | G3j:o |  | Yes [ ] No [ ] |
| G3m | Private data in result | P18.8 | G3j:o |  | Yes [ ] No [ ] |

## A.12.4	Query voice attribute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G4 | Query voice attribute service | S15.4 P15.4 | o |  | Yes [ ] No [ ] |
| G4a | Message ID | P15.4 | G4:m | [ ] | Yes [ ] |
| G4b | Connection associated with message | P15.4 | G4:o |  | Yes [ ] No [ ] |
| G4c | Query attribute | P15.4 | G4:m | [ ] | Yes [ ] |
| G4d | Attribute encode algorithm | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4e | Attribute sampling rate | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4f | Attribute duration | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4g | Attribute file name | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4h | Attribute current position | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4i | Attribute current speed | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4j | Attribute current volume | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4k | Attribute current level | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4l | Attribute current state | P15.4 | G4:o1 |  | Yes [ ] No [ ] |
| G4m | Attribute in result | P15.4 | G4ag:m | [ ] | Yes [ ] |
| G4n | ADPCM-6k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4o | ADPCM-8k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4p | Mu-Law PCM 6k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4q | A-Law PCM 6k | P15.4 | G4d:o2 |  | Yes [ ] No [ ] |
| G4r | Sampling rate in result | P15.4 | G4e:m | [ ] | Yes [ ] |
| G4s | Duration in result | P15.4 | G4f:m | [ ] | Yes [ ] |
| G4t | File name in result | P15.4 | G4g:m | [ ] | Yes [ ] |
| G4u | Current position in result | P15.4 | G4h:m | [ ] | Yes [ ] |
| G4v | Current speed in result | P15.4 | G4i:m | [ ] | Yes [ ] |
| G4w | Current volume in result | P15.4 | G4j:m | [ ] | Yes [ ] |
| G4x | Current level in result | P15.4 | G4k:m | [ ] | Yes [ ] |
| G4y | Stop | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4z | Play | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4aa | Record | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ab | Suspend play | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ac | Suspend record | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ad | Review | P15.4 | G4l:o3 |  | Yes [ ] No [ ] |
| G4ae | Security data in request | P18.7 | G4:o |  | Yes [ ] No [ ] |
| G4af | Private data in request | P18.8 | G4:o |  | Yes [ ] No [ ] |
| G4ag | Service result positive | P15.4 | G4:m | [ ] | Yes [ ] |
| G4ah | Service result negative | P16 | G4:m | [ ] | Yes [ ] |
| G4ai | Security data in result | P18.7 | G4ag:o |  | Yes [ ] No [ ] |
| G4aj | Private data in result | P18.8 | G4ag:o |  | Yes [ ] No [ ] |

## A.12.5	Record message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G5 | Record message service | S15.5 P15.5 | o |  | Yes [ ] No [ ] |
| G5a | Connection associated with record | P15.5 | G5:m | [ ] | Yes [ ] |
| G5b | Sampling rate | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5c | ADPCM-6k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5d | ADPCM-8k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5e | Mu-Law PCM 6k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5f | A-Law PCM 6k | P15.5 | G5:o1 |  | Yes [ ] No [ ] |
| G5g | Maximum duration | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5h | DTMF detected | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5i | No speech detected | P15.5 | G5:o |  | Yes [ ] No [ ] |
| G5j | Security data in request | P18.7 | G5:o |  | Yes [ ] No [ ] |
| G5k | Private data in request | P18.8 | G5:o |  | Yes [ ] No [ ] |
| G5l | Service result positive | P15.5 | G5:m | [ ] | Yes [ ] |
| G5m | Service result negative | P16 | G5:m | [ ] | Yes [ ] |
| G5n | Security data in result | P18.7 | G5l:o |  | Yes [ ] No [ ] |
| G5o | Private data in result | P18.8 | G5l:o |  | Yes [ ] No [ ] |

## A.12.6	Reposition

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G6 | Reposition service | S15.6 P15.6 | o |  | Yes [ ] No [ ] |
| G6a | Connection associated with message | P15.6 | G6:m | [ ] | Yes [ ] |
| G6b | Period | P15.6 | G6:m | [ ] | Yes [ ] |
| G6c | Message ID | P15.6 | G6:o |  | Yes [ ] No [ ] |
| G6d | Security data in request | P18.7 | G6:o |  | Yes [ ] No [ ] |
| G6e | Private data in request | P18.8 | G6:o |  | Yes [ ] No [ ] |
| G6f | Service result positive | P15.6 | G6:m | [ ] | Yes [ ] |
| G6g | Service result negative | P16 | G6:m | [ ] | Yes [ ] |
| G6h | Security data in result | P18.7 | G6f:o |  | Yes [ ] No [ ] |
| G6i | Private data in result | P18.8 | G6f:o |  | Yes [ ] No [ ] |

## A.12.7	Resume

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G7 | Resume service | S15.7 P15.7 | o |  | Yes [ ] No [ ] |
| G7a | Connection associated with message | P15.7 | G7:m | [ ] | Yes [ ] |
| G7b | Message ID | P15.7 | G7:o |  | Yes [ ] No [ ] |
| G7c | Duration | P15.7 | G7:o |  | Yes [ ] No [ ] |
| G7d | Security data in request | P18.7 | G7:o |  | Yes [ ] No [ ] |
| G7e | Private data in request | P18.8 | G7:o |  | Yes [ ] No [ ] |
| G7f | Service result positive | P15.7 | G7:m | [ ] | Yes [ ] |
| G7g | Service result negative | P16 | G7:m | [ ] | Yes [ ] |
| G7h | Security data in result | P18.7 | G7f:o |  | Yes [ ] No [ ] |
| G7i | Private data in result | P18.8 | G7f:o |  | Yes [ ] No [ ] |

## A.12.8	Review

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G8 | Review service | S15.8 P15.8 | o |  | Yes [ ] No [ ] |
| G8a | Connection associated with message | P15.8 | G8:m | [ ] | Yes [ ] |
| G8b | Period | P15.8 | G8:m | [ ] | Yes [ ] |
| G8c | Message ID | P15.8 | G8:o |  | Yes [ ] No [ ] |
| G8d | Security data in request | P18.7 | G8:o |  | Yes [ ] No [ ] |
| G8e | Private data in request | P18.8 | G8:o |  | Yes [ ] No [ ] |
| G8f | Service result positive | P15.8 | G8:m | [ ] | Yes [ ] |
| G8g | Service result negative | P16 | G8:m | [ ] | Yes [ ] |
| G8h | Security data in result | P18.7 | G8f:o |  | Yes [ ] No [ ] |
| G8i | Private data in result | P18.8 | G8f:o |  | Yes [ ] No [ ] |

## A.12.9	Set voice attribute

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G9 | Set voice attribute service | S15.9 P15.9 | o |  | Yes [ ] No [ ] |
| G9a | Connection associated with message | P15.9 | G9:m | [ ] | Yes [ ] |
| G9b | Speed | P15.9 | G9:o1 |  | Yes [ ] No [ ] |
| G9c | Volume | P15.9 | G9:o1 |  | Yes [ ] No [ ] |
| G9d | Level | P15.9 | G9:o1 |  | Yes [ ] No [ ] |
| G9e | Message ID | P15.9 | G9:o |  | Yes [ ] No [ ] |
| G9f | Security data in request | P18.7 | G9:o |  | Yes [ ] No [ ] |
| G9g | Private data in request | P18.8 | G9:o |  | Yes [ ] No [ ] |
| G9h | Service result positive | P15.9 | G9:m | [ ] | Yes [ ] |
| G9i | Service result negative | P16 | G9:m | [ ] | Yes [ ] |
| G9j | Security data in result | P18.7 | G9h:o |  | Yes [ ] No [ ] |
| G9k | Private data in result | P18.8 | G9h:o |  | Yes [ ] No [ ] |

## A.12.10	Stop

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G10 | Stop service | S15.10 P15.10 | o |  | Yes [ ] No [ ] |
| G10a | Connection associated with message | P15.10 | G10:m | [ ] | Yes [ ] |
| G10b | Message ID | P15.10 | G10:o |  | Yes [ ] No [ ] |
| G10c | Security data in request | P18.7 | G10:o |  | Yes [ ] No [ ] |
| G10d | Private data in request | P18.8 | G10:o |  | Yes [ ] No [ ] |
| G10e | Service result positive | P15.10 | G10:m | [ ] | Yes [ ] |
| G10f | Service result negative | P16 | G10:m | [ ] | Yes [ ] |
| G10g | Security data in result | P18.7 | G10e:o |  | Yes [ ] No [ ] |
| G10h | Private data in result | P18.8 | G10e:o |  | Yes [ ] No [ ] |

## A.12.11	Suspend

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G11 | Suspend service | S15.11 P15.11 | o |  | Yes [ ] No [ ] |
| G11a | Connection associated with message | P15.11 | G11:m | [ ] | Yes [ ] |
| G11b | Message ID | P15.11 | G11:o |  | Yes [ ] No [ ] |
| G11c | Security data in request | P18.7 | G11:o |  | Yes [ ] No [ ] |
| G11d | Private data in request | P18.8 | G11:o |  | Yes [ ] No [ ] |
| G11e | Service result positive | P15.11 | G11:m | [ ] | Yes [ ] |
| G11f | Service result negative | P16 | G11:m | [ ] | Yes [ ] |
| G11g | Security data in result | P18.7 | G11e:o |  | Yes [ ] No [ ] |
| G11h | Private data in result | P18.8 | G11e:o |  | Yes [ ] No [ ] |

## A.12.12	Synthesize message

| Item | Service / Feature | Reference | Status | N/A | Supported? |
| --- | --- | --- | --- | --- | --- |
| G12 | Synthesize message service | S15.12 P15.12 | o |  | Yes [ ] No [ ] |
| G12a | Text | P15.12 | G12:m | [ ] | Yes [ ] |
| G12b | Male | P15.12 | G12:o1 |  | Yes [ ] No [ ] |
| G12c | Female | P15.12 | G12:o1 |  | Yes [ ] No [ ] |
| G12d | Language | P15.12 | G12:o |  | Yes [ ] No [ ] |
| G12e | Message ID in result | P15.12 | G12:m | [ ] | Yes [ ] |
| G12f | Security data in request | P18.7 | G12:o |  | Yes [ ] No [ ] |
| G12g | Private data in request | P18.8 | G12:o |  | Yes [ ] No [ ] |
| G12h | Service result positive | P15.12 | G12:m | [ ] | Yes [ ] |
| G12i | Service result negative | P16 | G12:m | [ ] | Yes [ ] |
| G12j | Security data in result | P18.7 | G12h:o |  | Yes [ ] No [ ] |
| G12k | Private data in result | P18.8 | G12h:o |  | Yes [ ] No [ ] |

.

.

.

Printed copies can be ordered from:

ECMA
114 Rue du Rhône
CH-1204 Geneva
Switzerland

Fax:	+41 22  849.60.01

Internet:	documents@ecma.ch

Files can be downloaded from our FTP site, ftp.ecma.ch, logging in as anonymous and giving your E-mail address as password. This Standard is available from library ECMA-ST as a compacted, self-expanding file in MSWord 6.0 format (file E218-DOC.EXE) and as an Acrobat PDF file (file E218-PDF.PDF). File E218-EXP.TXT gives a short presentation of the Standard.

Our web site, http://www.ecma.ch, gives full information on ECMA, ECMA activities, ECMA Standards and Technical Reports.

ECMA

114 Rue du Rhône
CH-1204 Geneva
Switzerland

This Standard ECMA-218 is available free of charge in printed form and as a file.

See inside cover page for instructions